SA WG2 Meeting #149e	S2-2201781

# February 14th – 25th, 2022; Elbonia               	(revision of S2-220105R07)

Source: 	Nokia, Nokia Shanghai Bell (rapporteur), Deutsche Telekom

Title: 	FS_5WWC_Ph2 TR KI related with WT2

Document for: 	Approval

Agenda Item: 	9.13

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution: FS_5WWC_Ph2 TR KI related with WT2

# 1	Discussion

The FS_5WWC_Ph2 SID justification reads

SA2 received different LS from BBF asking for 5GC to support features not supported by R16 5WWC e.g.-

-	How to support differentiated services for different Non-3GPP devices behind a 5G-RG .

As part of TEI17_N3SLICE it has not been possible to work on How to select a TNGF/N3IWF that supports the S-NSSAI(s) requested by the UE during registration via trusted non-3GPP access network because N3SLICE had the constraint not to impact the UE.

The FS_5WWC_Ph2 SID objectives reads

Work task #2: Whether and how to improve the support of devices connecting behind 5G-RG including :

- void

- providing differentiated service (e.g.  QoS and charging) for UE and Non-3GPP devices connected behind a 5G RG. This may imply s studying ways for 5GC to identify Non-3GPP devices connected behind a 5G RG and/or their traffic. Conclusions may differ for UE and for Non-3GPP devices .

# 2 Proposal

It is proposed to update TR 23.700-17 as follows

First CHANGE ALL text is new

## 5.X	Key Issue #X: Providing differentiated service for UE and Non-3GPP devices connected behind a 5G RG.

### 5.X.1	Description

This key issue will study :

- 	how to support differentiated services (e.g. QoS and charging) for the same and different Non-3GPP devices, and UEs connected behind a 5G-RG.

NOTE 1:	A device capable of access to a N3IWF or to a TNGF is a UE; Non-3GPP devices can correspond to,  either AUN3 devices and NAUN3 devices defined in clause 3.1.

NOTE 2: The definitions in the clause 3.1 are considered as indication of kinds of Non-3GPP devices to be considered, but each solution may propose alternative kinds of devices with relative definition, if needed.

This may imply studying ways for 5GC to identify Non-3GPP devices connected behind a 5G RG and/or their traffic.

Conclusions to this key issue may differ for UE and for Non-3GPP devices. Different solutions may address different kinds of Non-3GPP devices, but as conclusion of this KI the kinds of Non-3GPP devices with related definitions considered applicable for the normative work will be selected.

This Key Issue will focus on 5G-RG as a first priority; No solution specific only to FN RG can be proposed, but common solutions for 5G RG and FN RG can be proposed.

NEXT CHANGE (2)

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], in 23.501 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] or in 23.501 [2].

Authenticable Non-3GPP (AUN3) devices: A Non-3GPP device that the 5GC can authenticate.

Non Authenticable Non-3GPP (NAUN3) devices: A Non-3GPP device that the 5GC cannot authenticate.

Non-3GPP devices: A device that use Non-3GPP access technology to connect to the RG and does not support NAS over non 3GPP access.