SA WG2 Meeting #149e	S2-2201824

# February 14th – 25th, 2022; Elbonia               	was S2-2201173r11

Source: 	Nokia, Nokia Shanghai Bell, China Mobile

Title: 	FS_UPEAS new KI: study UPF event exposure service registration and discovery

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: FS_UPEAS new KI: study UPF event exposure service registration and discovery.

# 1	Discussion

FS-UPEAS objectives contain:

WT#1: Study UPF event exposure service(s) registration/deregistration, and discovery via the NRF.

WT#2: Study UPF event exposure service(s) that would support, e.g.

-	Consumption of UPF exposure services by the PCF, NWDAF, CHF, NEF, Trusted AF and other NFs (if needed).

-	(To support the UPF exposure service, if needed) Use of SMF services, PCF services, NWDAF services, CHF services, NEF services, Trusted AF services by the UPF.

NOTE 1: This will not define solutions where UPF exposes information that it is not originator of, i.e. not re-expose information owned and exposed by other NFs.

-	Relevant Event IDs.

WT#3: Evaluate usage of UPF event exposure service(s) as defined in WT#2 also considering the architectural impacts.

# 2 Proposal

It is proposed to update 23.700-62 as follows.

CHANGE (1) ALL text is new

# 5	Key Issues

## 5.X	Key Issue #X: Study UPF event exposure service registration and discovery

### 5.X.1	Description

This KI maps to the WT1 of the SID.

The KI will study the registration and discovery of UPF event exposure service(s). The following aspects should be studied:

- How to support UPF event exposure service(s) registration/deregistration on NRF, and what parameters to be registered in the NF profile of UPF.

- How to support UPF service discovery via the NRF, and what parameters that can be used for discovery.

This key issue is not meant to define for which UPF event / exposed information a direct consumer subscription to the UPF would take place (if any).

NOTE: the information to be registered to the NRF is related to the UPF event exposure service requirement.