3GPP TSG-SA2 Meeting #150-e	S2-2202087

April 06 – April 12, 2022, Elbonia

Source:	Ericsson

Title:	New Key Issue on Study Whether and How Interactions between NWDAF can Leverage MDAS/MDAF Functionality for Analytics

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Spec:	TR 23.700-81

1. Introduction

The agreed eNA_ph3 SID includes the following Work Task:

WT #3.4: Study whether and how interactions between NWDAF can leverage MDAS/MDAF functionality for data collection and analytics

NOTE: The study should be led by and coordinated with SA5 and conclusions will be reached jointly

Interactions between NWDAF and MDAS have been considered in the existed specifications, for example in TS 28.533 clause 5.4 for management interactions with NWDAF, in TS 28.104 Clause 5 for management function interacting with NWDAF, and in TS 23.288 clause 6.2.3 for NWDAF retrieving management data from OAM. However, how NWDAF can leverage MDAS functionality for analytics are still void in these existing specifications. Studies on interactions between NWDAF and MDAS to leverage MDAS functionality for analytics are in needed.

NOTE: Whether and how NWDAF leverage MDAS functionality for support the analytics and services given in TS 23.288 (V17.4.0) are in the scope of SA2 and will be studied in SA2. Whether and how MDAS provide the requested analytics and/or data/information to NWDAF are in the scope of SA5 and need to be studied in SA5.

2. Reason for Change

The studies of how NWDAF can leverage MDAS functionality for analytics, within the context of WT#3.4, are in needed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-81 v0.1.0.

* * * Start of change  all new text* * * *

## 5.X	Key Issue #X: Study Whether and How Interactions between NWDAF can Leverage MDAS/MDAF Functionality for Analytics

### 5.x.1	General Description

NWDAF provides network analytics service (as described in TS 23.288 [x]), and MDAF provides the management data analytics service (MDAS) (as introduced in TS 28.533 [y]). As specified in TS 28.809 [z] clauses 4 and 5.2, NWDAF and MDAS can be producer and consumer to each other for data collection and analytics generation. In TS 28.104 [xx] clauses 4 and 5, NWDAF can consume the services exposed by the MDA, and management function may use the analytics results produced by NWDAF as input. For interactions between NWDAF and MDAS, there are already some considerations in the existed specifications, e.g., in TS 28.533 [y] clause 5.4 for management interactions with NWDAF, and in TS 23.288 [x] clause 6.2.3 for NWDAF retrieving management data from OAM. However, how NWDAF can leverage MDAS functionality for analytics are still void in the existing specifications. Hence, this Key Issue studies whether and how interactions between NWDAF can leverage MDAS/MDAF functionality for analytics.

This Key Issue only consider the Analytics IDs given in TS 23.288 [x] Rel-17 for NWDAF and the use cases given in TS 28.809 [z] (clause 6) for MDAS. In this Key Issue, the NWDAF and MDAS belong to the same PLMN.

This Key Issue suggests studying whether and how interactions between NWDAF can leverage MDAS/MDAF functionality for its analytics. This key issue includes the following aspects:

- Whether any existing MDAS report may support NWDAF producing Analytics ID given in TS 23.288 [x] Rel-17.

- Whether any changes to MDAS reports are needed to serve as input to NWDAF.

Editor’s Note: This Key Issue is limited to how NWDAF can leverage MDAS functionality for analytics. Solution impacts to MDAS/MDAF need SA5 to give inputs before conclusions are reach.

* * * End of change * * * *