3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2202118

# Elbonia, April 6 – 12, 2022	(revision of S2- 220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0516 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the data and analytics storage in ADRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As defined in clause 10.2.9, the “Service Operation, Analytics Specification or Data Specification and Time Window” parameters are included in the Nadrf_DataManagement_Delete service when a consumer NF would like to delete the stored data in the ADRF. 10.2.9 Nadrf_DataManagement_Delete Service operation name: Nadrf_DataManagement_Delete Description: This service operation instructs the ADRF to delete stored data. Inputs, Required: One of the following: - Storage Transaction Identifier; or - Service Operation, Analytics Specification or Data Specification and Time Window. Inputs, Optional: None. Outputs, Required: Operation execution result indication. Outputs, Optional: None. However, as defined in clause 10.2.2, these parameters are not included in the Nadrf_DataManagement_StorageRequest service when a consumer NF stores the data or analytics. 10.2.2 Nadrf_DataManagement_StorageRequest service operation Service operation name: Nadrf_DataManagement_StorageRequest Description: The consumer NF uses this service operation to request the ADRF to store data or analytics. Data or analytics are provided to the ADRF in the request message. Inputs, Required: Data or Analytics to be stored. Inputs, Optional: None. Outputs Required: Result Indication. Outputs, Optional: Storage Transaction Identifier. So it is unclear how ADRF locally retrieve the data or analytics and remove them when it receives the Nadrf_DataManagement_StorageRequest service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It proposes to add the “Service Operation, Analytics Specification or Data Specification and Time Window” parameters into the Nadrf_DataManagement_StorageRequest service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how ADRF locally retrieve the data or analytics and remove them when it receives the Nadrf_DataManagement_StorageRequest service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2, 10.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2B.2	Historical Data and Analytics storage

The procedure depicted in figure 6.2B.2-1 is used by consumers (e.g. NWDAF, DCCF or MFAF) to store historical data and/or analytics, i.e. data and/or analytics related to past time period that has been obtained by the consumer. After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an ADRF. Whether the consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.

Figure 6.2B.2-1: Historical Data and Analytics storage

1.	The consumer sends data and/or analytics to the ADRF by invoking the Nadrf_DataManagement_StorageRequest (collected data, analytics, Service Operation, Analytics Specification or Data Specification and Time Window) service operation.

2.	The ADRF stores the data and/or analytics sent by the consumer. The ADRF may, based on implementation, determines whether the same data and/or analytics is already stored or being stored based on the information sent in step 1 by the consumer NF and, if the data and/or analytics is already stored or being stored in the ADRF, the ADRF decides to not store again the data and/or analytics sent by the consumer.

3.	The ADRF sends Nadrf_DataManagement_StorageRequest Response message to the consumer indicating that data and/or analytics is stored, including when the ADRF may have determined at step 2 that data or analytics is already stored.

# * * * * Next change * * * *

10.2.2	Nadrf_DataManagement_StorageRequest service operation

Service operation name: Nadrf_DataManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store data or analytics. Data or analytics are provided to the ADRF in the request message.

Inputs, Required: Data or Analytics to be stored, Service Operation, Analytics Specification or Data Specification and Time Window.

Inputs, Optional: None.

Outputs Required: Result Indication.

Outputs, Optional: Storage Transaction Identifier.

# * * * * End of changes * * * *