3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202119

Elbonia, April 6 – 12, 2022	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on the parameter “list of analytics subsets that are requested”

Document for:	Discussion

Agenda Item:	8.1

Work Item / Release:	eNA_Ph2 / Rel-17

Abstract: This contribution proposes to discussion on the parameter “list of analytics subsets that are requested”.

1. Introduction/Discussion

As defined in clause 6.1.3 of TS 23.288, during the analytics request or subscription, the parameter “list of analytics subsets that are requested” can be included in the Analyics Filter to indicate the NWDAF which type of analytics information is requested:

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information is defined in the analytics related clauses.

However, there are some problems for the usage of this parameters in different Analytics IDs.

Observation 1: Which level this parameters should be applied?

-	For slice load level (clause 6.3.3A): the parameter can be used for the lowest level analytics types (e.g. number of UE registrations, number of PDU sessions establishment, etc.)

-	For service experience (clause 6.4.1/3): the parameter can be used for the middle level analytics types (Network Slice instance service experience, Application service Experience).

As most of the Analytics IDs apply this parameter for the lowest level analytics types, so it proposes the Service Experience should also do the same.

Observation 2: Should this parameter be set for any analytics type of a specific Analytics ID?

-	For NF load (clause 6.5.3): this parameter cannot applied to “NF load (per area of interest)”.

-	For DN Performance (clause 6.14.3): the NOTE 3 should be moved to the first column.

It proposes to indicate which analytics type should be applied by this parameter for all the Analytics IDs.

2. Proposal

Proposal 1: It proposes the “list of analytics subsets that are requested” should be applied on the lowest level analytics type for all the Analytics IDs.

Proposal 2: Indicate which analytics type per Analytics ID should be applied by “list of analytics subsets that are requested”.