3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2202120

# Elbonia, April 6 – 12, 2022	(revision of S2- 220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0517 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the Analytics subset per different Analytics ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | See the discussion in S2-2202119. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It proposes the “list of analytics subsets that are requested” should be applied on the lowest level analytics type for all the Analytics IDs. - Indicate which analytics type per Analytics ID should be applied by “list of analytics subsets that are requested”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are some problems for the usage of this parameters in different Analytics IDs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.3, 6.5.3, 6.14.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI (NOTE 1) | Identifies the Network Slice |
| > NSI ID (NOTE 1, NOTE 2) | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience (NOTE 1) | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 1,NOTE 3) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| Application service experiences (0..max) | List of observed service experience information for each Application. |
| > S-NSSAI (NOTE 1) | Identifies the Network Slice used to access the Application. |
| > Application ID (NOTE 1) | Identification of the Application. |
| > Service Experience Type (NOTE 1) | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location (NOTE 1, NOTE 5) | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered. |
| > UPF Info (NOTE 1, NOTE 4) | Indicating UPF serving the UE. |
| > DNAI (NOTE 1) | Indicating which DNAI the UE service uses/camps on. |
| > DNN (NOTE 1,NOTE 4) | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address (NOTE 1) | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience (NOTE 1) | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 1, NOTE 3) | List of SUPI(s) with the same application service experience. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > RAT Type (NOTE 1, NOTE 7) | Indicating the list of RAT type(s) for which the application service experience analytics applies. |
| > Frequency (NOTE 1, NOTE 7) | Indicating the list of carrier frequency value(s) of UE's serving cell(s) where the application service experience analytics applies. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested. NOTE 2: The NSI ID is an optional parameter. If not provided the Slice instance service experience indicates the service experience for the S-NSSAI. NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 4: If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. UE location shall only be included if the Consumer analytics request is for single UE or a list of UEs. Inclusion of UE location requires user consent. NOTE 6: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. NOTE 7: When "any" value has been provided in the request (e.g. "any" RAT type, "any" frequency, or "any" for all the RAT type and frequency indication), the NWDAF provides an instance of the Application service experience per combination of RAT Type(s) and/or Frequency value(s) having the same Service Experience. |  |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI (NOTE 1) | Identifies the Network Slice |
| > NSI ID (NOTE 1, NOTE 2) | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience (NOTE 1) | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 1, NOTE 3) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| Application service experiences (0..max) | List of predicted service experience information for each Application. |
| > S-NSSAI (NOTE 1) | Identifies the Network Slice used to access the Application. |
| > Application ID (NOTE 1) | Identification of the Application. |
| > Service Experience Type (NOTE 1) | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location (NOTE 1, NOTE 5) | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered. |
| > UPF Info (NOTE 1, NOTE 4) | Indicating UPF serving the UE. |
| > DNAI (NOTE 1) | Indicating which DNAI the UE service uses/camps on. |
| > DNN (NOTE 1, NOTE 4) | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address (NOTE 1) | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience (NOTE 1) | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 1, NOTE 3) | List of SUPI(s) with the same application service experience. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| > RAT Type (NOTE 1, NOTE 7) | Indicating the list of RAT type(s) for which the application service experience analytics applies. |
| > Frequency (NOTE 1, NOTE 7) | Indicating the list of carrier frequency value(s) of UE's serving cell(s) where the application service experience analytics applies. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 2: The NSI ID is an optional parameter. If not provided the Slice instance service experience indicates the service experience for the S-NSSAI. NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 4: If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. UE location shall only be included if the Consumer analytics request is for single UE or a list of UEs. Inclusion of UE location requires user consent. NOTE 6: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. NOTE 7: When "any" value has been provided in the request (e.g. "any" RAT type, "any" frequency, or "any" for all the RAT type and frequency indication), the NWDAF provides an instance of the Application service experience per combination of RAT Type(s) and/or Frequency value(s) having the same Service Experience. |  |

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided as part of Analytics Reporting Information by the NWDAF Service Consumer.

# * * * * Next change * * * *

6.5.3	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics. NF load statistics information are defined in Table 6.5.3-1. NF load predictions information are defined in Table 6.5.3-2.

Table 6.5.3-1: NF load statistics

| Information | Description |
| --- | --- |
| List of resource status (1..max) | List of observed load information for each NF instance along with the corresponding NF id / NF Set ID (as applicable). |
| > NF type | Type of the NF instance. |
| > NF instance ID | Identification of the NF instance. |
| > NF status (NOTE 1) | The availability status of the NF on the Analytics target period, expressed as a percentage of time per status value (registered, suspended, undiscoverable). |
| > NF resource usage (NOTE 1) | The average usage of assigned resources (CPU, memory, disk). |
| > NF load (NOTE 1) | The average load of the NF instance over the Analytics target period. |
| > NF peak load (NOTE 1) | The maximum load of the NF instance over the Analytics target period. |
| > NF load (per area of interest) (NOTE1, NOTE 2) | The average load of the NF instances over the area of interest. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 2: Applicable only to AMF load based on Input data in clause 6.5.2, Table 6.5.2-3 and Table 6.5.2-5. |  |

Table 6.5.3-2: NF load predictions

| Information | Description |
| --- | --- |
| List of resource status (1..max) | List of predicted load information for each NF instance along with the corresponding NF id / NF Set ID (as applicable) |
| > NF type | Type of the NF instance |
| > NF instance ID | Identification of the NF instance |
| > NF status (NOTE 1) | The availability status of the NF on the Analytics target period, expressed as a percentage of time per status value (registered, suspended, undiscoverable) |
| > NF resource usage (NOTE 1) | The average usage of assigned resources (CPU, memory, disk) |
| > NF load (NOTE 1) | The average load of the NF instance over the Analytics target period |
| > NF peak load (NOTE 1) | The maximum load of the NF instance over the Analytics target period |
| > Confidence | Confidence of this prediction |
| > NF load (per area of interest) (NOTE1, NOTE 2) | The predicted average load of the NF instances over the area of interest. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 2: Applicable only to AMF load based on Input data in clause 6.5.2, Table 6.5.2-3 and Table 6.5.2-5. |  |

NOTE:	The variations on per-instance NF load and resource usage could be influenced by the number of running NF instances in addition to the load itself.

The predictions are provided with a Validity Period, as defined in clause 6.1.3.

The number of resource status is limited by the maximum number of objects provided as part of Analytics Reporting Information.

# * * * * Next change * * * *

6.14.3	Output Analytics

The DN performance analytics is shown in table 6.14.3-1 and table 6.14.3-2.

Table 6.14.3-1: DN service performance statistics

| Information | Description |
| --- | --- |
| Application ID | Identifies the application for which analytics information is provided. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. See note 1. |
| DNN | Identifies the data network name (e.g. "internet") for which analytics information is provided. See NOTE 1. |
| DN performance (0-x) | List of DN performances for the application. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| > Serving anchor UPF | The involved anchor UPF. See NOTE 2. |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > Performance | Performance indicators. |
| >> Average Traffic rate (NOTE 3) | Average traffic rate observed for UEs communicating with the application. See NOTE 3. |
| >> Maximum Traffic rate (NOTE 3) | Maximum traffic rate observed for UEs communicating with the application. See NOTE 3. |
| >> Average Packet Delay (NOTE 3) | Average packet delay observed for UEs communicating with the application. See NOTE 3. |
| >> Maximum Packet Delay (NOTE 3) | Maximum packet delay for observed for UEs communicating with the application. See NOTE 3. |
| >> Average Packet Loss Rate (NOTE 3) | Average packet loss observed for UEs communicating with the application. See NOTE 3. |
| > Spatial Validity Condition | Area where the DN performance analytics applies. |
| > Temporal Validity Condition | Validity period for the DN performance analytics. |
| NOTE 1: The item "DNN" and "S-NSSAI" shall not be included if the consumer NF is an untrusted AF. NOTE 2: The item "Serving anchor UPF" shall not be included if the consumer is an AF. NOTE 3: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". |  |

Table 6.14.3-2: DN service performance predictions

| Information | Description |
| --- | --- |
| Application ID | Identifies the application for which analytics information is provided. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. See NOTE 1. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. See NOTE 1. |
| DN performance (0-x) | List of DN performance for the application. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| > Serving anchor UPF | The involved anchor UPF. See NOTE 2. |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > Performance | Performance indicators |
| >> Average Traffic rate (NOTE 3) | Average traffic rate predicted for UEs communicating with the application. See NOTE 3. |
| >> Maximum Traffic rate (NOTE 3) | Maximum traffic rate predicted for UEs communicating with the application. See NOTE 3. |
| >> Average Packet Delay (NOTE 3) | Average packet delay predicted for UEs communicating with the application. See NOTE 3. |
| >> Maximum Packet Delay (NOTE 3) | Maximum packet delay for predicted for UEs communicating with the application. See NOTE 3. |
| >> Average Packet Loss Rate (NOTE 3) | Average packet loss predicted for UEs communicating with the application. See NOTE 3. |
| > Spatial Validity Condition | Area where the DN performance analytics applies. |
| > Temporal Validity Condition | Validity period for the DN performance analytics. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: The item "DNN" and "S-NSSAI" shall not be included if the consumer is an untrusted AF. NOTE 2: The item "Serving anchor UPF" shall not be included if the consumer is an AF. NOTE 3: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". |  |

# * * * * End of changes * * * *