3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202136

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New solution on support of onboarding over trusted non-3GPP access in SNPN

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract: This contribution would like to propose a new solution on support of onboarding over trusted non-3GPP access in SNPN

1. Introduction/Discussion

Clause 5.30.2.8 and Annex D, clause D.3 of TS 23.501 [3] specify how the UE can access SNPN services via a PLMN. Solution #3 defines how the UE can access SNPN services via trusted non-3GPP access network using the credentials owned by the SNPN.

This solution defines how the UE can access SNPN via trusted non-3GPP access network for Onboarding.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-08.

# * * * * First change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| X |  | X |  |  |  |  |

# * * * * Second change * * * *

6.X	Solution #X: Support of onboarding over trusted non-3GPP access in SNPN

6.X.1	Introduction

Editor's Note:	This clause lists the key issue(s) addressed by this solution, and briefly the main principles of the solution.

This solution aims at addressing key Issue #2 about support of Non-3GPP access for SNPN. In particular, this solution mainly focuses on how to support functionalities defined in R17 eNPN such as onboarding over trusted non-3GPP access. For UE accessing SNPN using credentials owned by the SNPN over trusted non-3GPP access, Solution #3 specified in clause 6.3 can be applied.

6.X.2	Functional Description

Editor's Note:	This clause further details the solution principles and any assumptions made.

When UE registers to a SNPN over trusted non-3GPP access for Onboarding, it shall select a TNAN in the SNPN which supports Onboarding. Additionally, the SNPN shall support the Default Credentials Server (DCS) belonging to a group identified by GINs. Therefore, clause 6.3 can be applied with following clarifications and additions:

-	The UE initiates the access network selection procedure and is configured with a list of available GINs indicating a group of DCS. This list contains the GINs advertised by all discovered trusted WLAN access networks. A trusted WLAN access network may advertise (e.g. with ANQP), not only the SNPNs with which 5G connectivity is supported, but also the GINs and/or Onboarding indication indicating that the TNAN supports Onboarding.

-	The UE selects an TNAN that supports the GIN which is included in the list of available GINs.

After UE selects a TNAN that supports Onboarding, it shall include an Onboarding indication in the AN parameters included in the EAP-Res/5G-NAS message which are sent to TNGF during registration procedure as specified in clause 4.12a.2 in TS 23.502 [4]. The TNGF shall select an AMF that supports Onboarding based on the Onboarding indication included in the AN parameters.

6.X.3	Procedures

Editor's Note: This clause describes procedures and information flows for the solution.

6.X.4	Impacts on services, entities, and interfaces

Editor's Note: This clause lists impacts to services, entities, and interfaces.

UE impact:

-	Ability to read GINs and/or Onboarding indication in the list of available networks with which 5G connectivity is supported, as advertised by the trusted non-3GPP access network.

-	Ability to select an SNPN that supports the GINs which is included in the list of available GINs.

-	Ability to select an TNAN in an SNPN that supports Onboarding.

Trusted non-3GPP access network impact:

-	Ability to advertise (e.g., via ANQP) the GINs and/or Onboarding indication.

TNGF impact:

-	Ability to select an AMF that supports Onboarding.

# * * * * End of changes * * * *