3GPP TSG-SA2 Meeting # 150E (e-meeting)	S2-2202143

# Elbonia, April 6 - 12, 2022																	 (revision of )

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0102 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Broadcast MBS Session Release Require Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In SA2#149E, RAN3 asked in LS R3-221468 to introduce a RAN initiated Broadcast Session Release Require procedure as below: Furthermore, majority companies in RAN3 think that it is beneficial to introduce a RAN initiated Broadcast Session Release Require procedure, to trigger the CN to initiate Broadcast Session Release procedure. To be used in case RAN is not able to provide the service over radio, due to e.g. lack of radio resources. In the response LS S2-2201315, SA2 agreed with RAN3 about this Broadcast Session Release Require procedure: SA2 shares the view of RAN3 that it is beneficial for NG-RAN to notify CN if a broadcast session cannot be provided by a NG-RAN node. SA2 respectfully asks RAN3 to proceed with the work. SA2 intends to agree necessary related changes at a future meeting. However, SA2 did not manage to add the corresponding procedure in SA2#149E |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the Broadcast Session Release Require procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MBS function incomplete, and misalignment with RAN3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

### 7.3.X	Broadcast MBS Session Release Require

When NG-RAN is not able continue to provide the Broadcast MBS Session, e.g., due to lack of radiou resources, NG-RAN may intitiate Broadcast Session Release Require procedure.

Figure 7.3.X-1: Broadcast MBS Session Release Require

- If NG-RAN cannot continue to provide the Broadcast MBS session, e.g., due to lack of radio resources, the NG-RAN sends Broadcast Session Release Require (MBS Session ID) to the AMF.

- The AMF intitiates Broadcast Session Release towards the NG-RAN as defined in step 4 - 7 in clause 7.3.2.

- If unicast transport applies in N3mb, the AMF receives the DL tunnel Info for the Broadcast MBS Session from the NG-RAN in step 2. The AMF notifies the MB-SMF about the DL tunnel release via Namf_MBSBroadcast_ContextStatusNotify.

- The MB-SMF performs N4mb Session Update to let MB-UPF stop the broadcast data forwarding towards the indicated DL tunnel and release the DL tunnel.

****************** END CHANGES ***************