3GPP TSG-SA WG2 Meeting #150E	S2-2202228

# E-meeting, April 06 - 12 2022

Title:	[DRAFT] LS on enabling support for Slice Groups and Slice priorities for RAN Slicing

Response to:	-

Release:	Rel-17

Work Item:	NR_Slice-Core

Source:	SA2

To:	RAN2, RAN3, CT1, CT4

Cc:	SA, RAN, CT, SA5

Contact person:	Peter Hedman

Peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  S2-220xxx, …

# 1	Overall description

SA2 would like to inform RAN2, RAN3, CT1, CT4 about SA2 progress of supporting Slice Groups and Network Slice priorities required for enabling RAN Slicing as per Work Item NR_Slice-Core.

SA2 has agreed the attached CRs…

SA2 has technically endorsed the attached CRs…

Considering that SA2 could not reach a consensus on how the logic of a per TA granularity is meant to work, e.g. see questions below, SA2 technically endorsed CRs, in S2-2202226/S2-2202227, with a per PLMN granularity; based on OAM to NG-RAN and OAM to AMF/NSSF a configuration is achieved of a per Network Slice to Slice Group(s) association. Note, SA2 assumes that it is in deployments possible to re-use the Slice Group values implying another Network Slice association as long as the re-use is at a geographical far distance and the values to be re-used have not been used within this distance. This can be achieved with appropriate OAM configuration.

SA2 understands that RAN2 assumes that the Network Slice to Slice Group association is defined per TA, but to understand the assumed functionality for a per TA granularity SA2 has the following questions to RAN2 (these identified issues are not any issues for a PLMN granularity as per endorsed CRs):

Q1)	For per TA granularity of Slice Group, how is the UE meant to understand the definition of the Slice Groups used in the neighbour cell information (especially when they are in another TA) e.g. is the UE meant to read SIB1 of neighbour cells for understanding neighbour cell info in current cell, or is the definition of the values of the sliceGroupIDs of current cell required to be the same as used in neighbour cells?

Q2)	Slice group in RRC Release, if valid per TA, how does the UE interpret the content and especially the encoding of the sliceGroupID?

For Network Slice prioritization, SA2 assumes it is important to maintain a network control of the cell reselection logic. The attached CR, in S2-2202224, maintains network control while explicitly allowing the UE to only change the Network Slice priority when attempting to register new Network Slice(s) i.e. the UE is not allowed to change the priority for other reasons. Additional reasons can be added, based on further work, while also such additional reasons need to be possible to being explicitly allowed by the network.

For progressing network control of the Slice aware cell reselection logic with regards to Slice priorities, SA2 has the following question to RAN2:

Q3)	Is there a benefit in triggering a cell reselelection to a cell/TA within the RA due to a change of slice priorities e.g. when UE establish PDU Session or activate UP with service request?

# 2	Actions

To RAN3, CT1, CT4:

ACTION: SA2 would like to ask RAN3, CT1 and CT4 to take the above into consideration.

To RAN2:

ACTION: SA2 would like to ask RAN2 to take the above into consideration and answer the above questions.

# 3	Dates of next TSG CT WG1 meetings

TSG SA WG2 Meeting 151E	16 – 20 May. 2022, E-meeting