3GPP SA WG2 Meeting #150E	S2-2202300

e-meeting, April 06-12, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#2 Solution proposal Time synchronization service enhancements – Solution proposal

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper proposes a solution for the key issue #2 “Time synchronization service enhancements” to the TR 23.700-25.

# Discussion

The FS_5TRS_URLLC SID has a KI#2 for time synchronization service enhancements agreed in S2-2201762 with the following objectives:

-	How to enable AFs to request time synchronization service in a specific coverage area and whether and how to enforce the coverage area.

To address them, the following solution is proposed for TR 23.700-25.

# Proposal

It is proposed to add the following solution to TR 23.700-25.

*** Start of changes (all new text) ***

## 6.x	Solution #x: Geographical area filters for time synchronization service

### 6.x.1	Introduction

This solution is proposed to solve Key Issue #2: Time synchronization service enhancements. In this Key Issue, the coverage area is taken into account for the configuration of the time synchronization service. Therefore, this solution addresses the time synchronization service scenarios already supported in 5G Release-17 based on access stratum or (g)PTP time distribution methods.

### 6.x.2	Functional Description

The following principles are proposed to enable the use of coverage area for time synchronization service operation:

-	An AF may optionally include geographical area filter(s) within the timing synchronization service request.

-	The AF may provide as geographical area filter(s) spatial validity information (e.g., a civic address or shapes) or area(s) of interest (AoI). If the AF provides spatial validity, the NEF/TSCTSF maps the spatial validity to validity area(s) (based on pre-configuration). Later, the TSCTSF or PCF determines the area(s) of interest based on validity area(s). The TSCTSF or AMF subscriptions use area of interest that may be the same as spatial validity condition or may be a subset of the spatial validity condition (e.g., a list of TAs) based on the latest known UE location.

-	TSCTSF uses Location or UE presence in Area of Interest services at the AMF Event Exposure service  (Namf_EventExposure operations) to identify the UE(s) to which the AF request with geographical area filter(s) applies. The TSCTSF subscribes to this information at the AMF and stores it.

-	Geographical area filter(s) will be used at the TSCTSF for time synchronization service activation/deactivation.

-	For a UE that is inside of the allowed geographical area(s), the activation of time synchronization service follows the operation described in TS 23.501 [2] and procedures of TS 23.502 [3].

-	When a UE that previously was inside the allowed geographical area(s) leaves the allowed area, the TSCTSF will initiate UE time synchronization service deactivation. For access stratum distribution deactivation, the TSCTSF will disable ReferenceTimeInformation delivery to the UE at the serving NG-RAN node. For (g)PTP distribution deactivation, the TSCTSF will modify the PTP instance configuration by means of sending a PMIC to the impacted UE/DS-TT and UMIC to the UPF/NW-TT, as described in clause K.2.2 of TS 23.501 [2].

### 6.x.3	Procedures

#### 6.x.3.1	Procedure for AF requested (g)PTP timing synchronization with geographical area filters

An overall procedure for AF providing coverage area filter(s) for (g)PTP based time synchronization service is illustrated in Figure 6.x.3.1-1.

Figure 6.x.3.1-1: (g)PTP time distribution configuration with geographical area filters

1.	The AF creates a time synchronization service configuration for a PTP instance by invoking Nnef_TimeSynchronization_ConfigCreate service operation. The request includes the parameters as described in table 4.15.9.3.1 in TS 23.502 [3] and optionally geographical area filter(s) in the format of spatial validity or area(s) of interest.

2.	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_TimeSynchronization_ConfigCreate service operation with the corresponding TSCTSF, with the parameters as received from the AF.

The AF that is part of operator's trust domain may invoke the services directly with TSCTSF.

3.	If the AF provided spatial validity, the TSCTSF maps the information to areas of validity based on pre-configuration, then the TSCTSF determines area(s) of interest based on validity area(s).

4.	The TSCTSF responds with the Ntsctsf_TimeSynchronization_ConfigCreate response.

5.	The NEF responds with the Nnef_TimeSynchronization_ConfigCreate response.

6.	The spatial validity condition (or the area(s) of interest condition) is resolved at the TSCTSF. In order to do that, the TSCTSF subscribes to the AMF to receive notifications about the UE presence in an area of interest (or UE location) using Namf_EventExposure operation. The subscribed area of interest may be the same as spatial validity condition or may be a subset of the spatial validity condition (e.g., a list of TAs) based on the latest known UE location.

7. 	The TSCTSF determines the target UEs for the PTP instance configuration which match the geographical area filter(s). The TSCTSF uses the procedures described in clause K.2.2 of TS 23.501 [2] to configure and initialize the PTP instance in the DS-TT(s) and NW-TT.

8.	The TSCTSF uses the procedure in clause 4.15.9.4 of TS 23.502 [3] to manage the 5G access stratum time distribution for the UEs that are part of the impacted PTP instance.

9. 	UE moves out of the configured area of interest.

10. NG-RAN determines the UE presence in the area of interest has changed and notifies the AMF.

11.	The AMF detects the subscription change related event occurs and it sends the event report by means of Namf_EventExposure_Notify message to the TSCTSF.

12. The TSCTSF initiates the deactivation of time synchronization service for the UE. The TSCTSF modifies the PTP instance configuration by means of sending a PMIC to the impacted UE/DS-TT and UMIC to the UPF/NW-TT, as described in clause K.2.2 of TS 23.501 [2]. For access stratum distribution deactivation, the TSCTSF disables ReferenceTimeInformation delivery to the UE at the serving NG-RAN node, as described in clause 4.15.9.4 of TS 23.502 [3].

#### 6.x.3.2	Procedure for AF requested access stratum timing synchronization with geographical area filters

An overall procedure for AF providing coverage area filter(s) for access stratum based time synchronization service (refer TS 23.502) is illustrated in Figure 6.x.3.2-1 .

Figure 6.x.3.2-1: 5G access stratum time distribution configuration with geographical area filters

1.	The AF request to influence the 5G access stratum time distribution providing access stratum time distribution parameters to the NEF (together with the AF identifier and potentially further inputs as specified in table 4.15.9.4-1 in TS 23.502 [3]) and optionally geographical area filter(s) in the format of spatial validity or area(s) of interest.

2.	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_ ASTI_Create service operation with the corresponding TSCTSF.

3.	If the AF provided spatial validity, the TSCTSF maps the information to areas of validity based on pre-configuration, then the TSCTSF determines area(s) of interest based on validity area(s).

4.	The spatial validity condition (or the area(s) of interest condition) is resolved at the TSCTSF. In order to do that, the TSCTSF subscribes to the AMF to receive notifications about change of UE location in an area of interest using Namf_EventExposure operation. The subscribed area of interest may be the same as spatial validity condition or may be a subset of the spatial validity condition (e.g., a list of TAs) based on the latest known UE location.

5.	The TSCTSF determines the target UEs for the access stratum time distribution configuration which match the geographical area filter(s). The TSCTSF uses the procedures described in clause 4.15.9.4 of TS 23.502 [3] to configure ReferenceTimeInformation delivery to the UE(s) at the serving NG-RAN nodes.

6.	The TSCTSF responds the AF with the Ntsctsf_ ASTI_Create service operation response.

7.	The NEF informs the AF about the result of the Nnef_ ASTI_Create service operation performed in step 1.

8. 	UE moves out of the configured area of interest.

9. 	NG-RAN determines the change of the UE presence in the area of interest has changed and notifies the AMF.

10.	The AMF detects the subscription change related event occurs and it sends the event report by means of Namf_EventExposure_Notify message to the TSCTSF.

11. The TSCTSF initiates the deactivation of time synchronization service for the UE. The TSCTSF disables ReferenceTimeInformation delivery to the UE at the serving NG-RAN node, as described in clause 4.15.9.4 of TS 23.502 [3].

### 6.x.4	Impacts on services, entities and interfaces

-	TSCTSF:

-	Subscribe for receiving UE’s presence in Area of Interest from AMF.

-	Mapping from spatial validity to validity area(s) (based on pre-configuration).

-	Determination of area of interest based on the validity area(s).

-	Support for time synchronization service configuration conditioned to geographical area filter(s) received from AF.

*** End of changes ***