3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202358

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Solution update for RRC inactive MBS data reception

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: This solution provides an update for RRC inactive MBS data reception and remove the corresponding ENs.

1. Introduction/Discussion

5GC provides assistance information to NG-RAN node so that NG-RAN can make the decision to move UE in multicast MBS session to RRC Inactive state. This paper proposes that the assistance information includes the MBS session priority and the UE priority within each MBS session. Since the MBS session priority is per session granularity, so it can be delivered from MB-SMF to the NG-RAN during the shared tunnel establishment procedure. And the UE priority within each MBS session is per UE granularity, so it can be delivered from SMF to the NG-RAN during the UE join procedure.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

## 6.1	Solution #1: Procedures for RRC Inactive MBS data reception

### 6.1.1	Introduction

This solution addresses Key Issue #1.

### 6.1.2	Functional description

Editor's note:	This clause outlines solution principles and documents any assumptions made.

It is assumed to reuse the current architecture defined in Rel-17 MBS work (see TS 23.247 [4]). In other words, MB-SMF is used to handle MBS session-level management while SMF performs per-UE MBS session management, e.g. authorization, multicast session information provisioning, managing 5GC Individual MBS traffic delivery.

Procedures in the following clauses focus on the following functionalities:

-	5GC provisioning necessary parameters to NG-RAN node(s).

-	Switching between RRC Connected and RRC Inactive modes.

There are two levels of priority as a part of 5GC-provided parameters, namely:

-	MBS session priority: the MBS session priority denotes the priority level of an MBS session, and the priority level defines the relative importance of an MBS session. This allows the NG-RAN nodes deciding (the members of) which MBS session can be switched to RRC Inactive state, to free up resources of NG-RAN node upon e.g., congestion. NG-RAN node may also use it to decide (the members of) which MBS session can be switched to RRC Connected state, once the resources are regarded as sufficient.

The details of how the 5GC provides MBS session priority to NG-RAN node are further described in clause 6.1.3.2.

-	UE session priority: the UE session priority denotes the priority level of a certain UE within a certain MBS session, and the priority level defines the relative importance of a UE for an MBS session. This allows the NG-RAN nodes deciding if the UE of an MBS session can be switched to RRC Inactive state, to free up resources of NG-RAN node upon e.g., congestion. NG-RAN node may also use it to decide if the UE of an MBS session can be switched to RRC Connected state, once the resources are regarded as sufficient.

The details of how the 5GC provides UE session priority to NG-RAN node are further described in clause 6.1.3.2.

NOTE 1:	The NG-RAN node can combine MBS session priority and UE session priority to determine the RRC status switching for a certain UE. The RAN WGs will determine the details.

NOTE 2:	The NG-RAN node can use the most demanding UE session priorities to determine the RRC state transition if the UE is with multiple multicast MBS sessions.

### 6.1.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

#### 6.1.3.1	General

NOTE:	The message names in the procedures below are descriptive. It is assumed that the names are updated with corresponding SBI based names where applicable during the normative phase.

#### 6.1.3.2	MBS session creation, multicast session join and session establishment procedure

Figure 6.1.3.2-1: Enhancement to current MBS procedures

0.	The AF starts the MBS Session towards 5GC by invoking Nmbsmf_MBSSession_Create Request services. AF provides the MBS session priority to MB-SMF, optionally via NEF or MBSF.

1a-7. UE sends join request to the and SMF/MB-SMF provide necessary information to the NG-RAN node, details see step 1a-8 of clause 7.2.1.3, TS 23.247 [4]. The enhancements are as follows:

-	Step 4 and 5: SMF provides the UE session priority to NG-RAN. In step 4, SMF includes UE session priority as a part of N2 SM information in Nsmf_PDUSession_UpdateSMContext response to AMF. In step 5, AMF sends the N2 SM information received from SMF to NG-RAN node.

-	Step 6: during the shared tunnel establishment procedure, MB-SMF provides the MBS session priority to NG-RAN node. In step 6, since MB-SMF receives the MBS session priority in step 0, MB-SMF includes MBS session priority in the N2 SM information of Nmbsmf_MBSSession_ContextUpdate response message. And AMF sends N2 MBS Session response message to NG-RAN node.

Editor's note:	How SMF gets the UE session priority within the MBS session is FFS.

#### 6.1.3.3	Moving a UE to RRC Inactive state

Figure 6.1.3.3-1: NG-RAN node moves a UE to CM-CONNECTED with RRC Inactive state

0.	5GC provides assistance information of RRC Inactive multicast MBS data reception to NG-RAN node, details see clause 6.1.3.2. UE receives multicast MBS data in CM-CONNECTED mode.

Editor's Note:	Whether/what assistance information is needed is to be coordinated with RAN WGs.  It is ffs if some UEs in the same cell can receive MBS data in RRC Inactive reception mode and other UEs can receive MBS data in Rel-17 reception mode.

1.	RAN determines to move UE in multicast MBS session to RRC Inactive state based on MBS session priority and/or UE session priority included in the assistance information.

NOTE 1:	The NG-RAN node can combine MBS session priority and UE session priority to determine the RRC status switching for a certain UE. The RAN WGs will determine the details.

NOTE 2:	The NG-RAN node can use the most demanding UE session priorities to determine the RRC state transition if the UE is with multiple multicast MBS sessions.

Editor's note:	Determination of switching to RRC Inactive will be confirmed by the RAN WGs.

2.	NG-RAN node releases the RRC connection and moves the UE to CM-CONNECTED with RRC Inactive state.

Editor's note:	How to release the UEs belongs to multicast MBS session will be determined by RAN WGs.

3.	UE receives multicast MBS data in CM-CONNECTED with RRC Inactive mode.

Editor's note:	RAN WGs will determine the configuration of UE receiving multicast MBS data in RRC Inactive.

#### 6.1.3.4	Moving a UE to RRC-CONNECTED from RRC Inactive state

Figure 6.1.3.4-1: NG-RAN node moves a UE to RRC-CONNECTED state

0.	5GC provides assistance information of RRC Inactive multicast MBS data reception to NG-RAN node, details see clause 6.1.3.2. UE receives multicast MBS data in CM-CONNECTED with RRC Inactive mode.

Editor's note:	Whether/what assistance information is needed is to be coordinated with RAN WGs.

1.	RAN determines to move UE in multicast MBS session to RRC-CONNECTED state based on MBS session priority and/or UE session priority included in the assistance information.

Editor's note:	Determination of switching to RRC Inactive will be confirmed by the RAN WGs.

2.	NG-RAN node informs related UEs to RRC-CONNECTED state.

Editor's note:	How to inform the related UEs belonging to multicast MBS session will be determined by RAN WGs.

3.	UE receives multicast MBS data in RRC-CONNECTED mode.

Editor's note:	RAN WGs will determine the configuration of UE receiving multicast MBS data in RRC-CONNECTED mode.

### 6.1.4	Impacts on services, entities and interfaces.

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

Functional entities defined in clause 5.3.2 of TS 23.247 [4] is reused.

AF:

-	The AF includes the MBS session priority in the message sent to MB-SMF.

MB-SMF:

-	The MB-SMF stores MBS session priority as a part of multicast context, and provides MBS session priority to the NG-RAN node.

SMF:

-	SMF fetches the UE MBS priority and provide to the NG-RAN node during PDU Session modification/establishment procedure.

NG-RAN:

-	Determine which UE can be switched RRC Inactive/Connected mode based on the UE MBS priority and MBS session priority from SMF/MB-SMF, respectively.

Editor's note:	Additional impacts are FFS.

# * * * * End of changes * * * *