3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2202366

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Title:	[DRAFT] Reply LS on EPS fallback enhancements

Response to:	LS (R2-2204236) on LS on EPS fallback enhancements from RAN2

Release:	Rel-17

Work Item:	TEI17

Source:	[Huawei to be] SA2

To:	RAN2

Cc:	CT1, SA3

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

1. Overall Description:

SA2 thanks RAN2 for the LS on EPS fallback enhancements. SA2 would like to provide the following feedback:

Regarding the question provided in the LS:

In RAN2 discussion there is no consensus whether SA2/CT1 existing procedures and specification can support these candidate solutions without any change, therefore RAN2 would like to request SA2 and CT1 to evaluate and inform RAN2 whether there is any impact foreseen. If there is, indicating the detailed impact would be helpful to RAN2.

SA2 answer:

SA2 discuss this issue and would like to response to RAN2 that there is no foreseen crucial impact to current messages of SA2 procedures. The followings processing logics need to be enhanced on the other hand:

For the MT case:

- Reporting the capability of Paging Cause indication for Voice service: UE needs to report the NAS capability of Paging Cause indication for voice services (PIV) in the NAS message once it supports the EPS fallback enhancements. Currently only if UE with the capabilities of Multi-USIM features, operating two or more USIMs and intends to use one or more Multi-USIM features shall indicate in the UE 5GMM Core Network Capability and include PIV indication if needed.

- Paging for CM-IDLE UE:

  - For Alt#1, NG-RAN node shall know whether the Uu Paging shall include EPS fallback indication after receiving N2 Paging with voice indication.

  - For Alt#2, NG-RAN node shall broadcast the support of EPS fallback in the SIB message in order to redirect UE to EPS.

SA2 notices the above-mentioned enhancements w.r.t paging mentioned in RAN2 contributions (R2-2202818/ R2-2202505/ R2-2202791) and SA2 understands the related feasibility analyses of Alt#1 and Alt#2 are under the remit of RAN2.

- Connecting to EPC: CM-IDLE or RRC Inactive UE needs to send NAS TAU request to EPC upon receiving EPS fallback indication in the Paging (Alt#1) or in the SIB (Alt#2), instead of connecting to 5GC.

- Paging failure processing in 5GC: Paging (e.g., CN or RAN paging) at 5G side may experience failure since UE may connect to E-UTRAN+EPC. SA2 spec needs to clarify how to handle the paging procedure without paging response in 5GC.

For the MO case:

- Connecting to EPC: UE needs to send NAS TAU request to EPC upon receiving the SIB (Alt#2), instead of connecting to 5GC.

2. Actions:

To RAN2 group.

ACTION: 	SA2 asks RAN2 to take above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #151E   	May 16 – 20, 2022 				Elbonia