3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202373

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Solution for Selecting the same EAS/DNAI for collection of UEs KI#4

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract: Solution for Selecting the same EAS/DNAI for collection of UEs

1. Introduction/Discussion

In KI#4, it is to investigate the following issues:

- whether and how to define a collection of UEs forming a dynamic ad-hoc group that should use the same EAS and/or same local part of DN and/or same DNAI and how the collection is identified;

- whether and what improvements are required for EAS discovery and re-discovery for UEs belonging to a collection of UEs.

It assumes the member of UE collection is dynamic, and UEs join/leave dynamically.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-48.

# * * * * First change * * * *

6.0	Solution-Key issue matrix

The solutions in clause 6 can apply to one or more key issues described in clause 5 of this report. Table 6.0-1 describes the relationship between solutions and key issues.

Editor's note:	The table below will be updated with actual content when generating the TR with approved contributions. Page number is automatically updated to ease reference (ctrl-left click to reach the solution).

Table 6.0-1: Solution-Key issue matrix

| Solution |  | Key issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | (page) | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 | KI#7 |
| 01: <solution title> | 10 | X |  |  |  |  |  |  |
| Selecting the same EAS/DNAI for collection of UEs |  |  |  |  | x |  |  |  |

# * * * * Second change * * * *

6.X	Solution X: Selecting the same EAS/DNAI for collection of UEs

6.X.1 Description

This solution issues corresponds to KI#4:

-	whether and how to define a collection of UEs forming a dynamic ad-hoc group that should use the same EAS and/or same local part of DN and/or same DNAI and how the collection is identified;

-	whether and what improvements are required for EAS discovery and re-discovery for UEs belonging to a collection of UEs.

The solution assumes the member of UE collection is dynamic, UEs join/leave dynamically, and here are two scenarios:

Scenario#1: the application layer just indicates 5GC to select the same EAS or the same DNAI for collection of UEs accessing the application service and under certain criteria (e.g. UEs within a specific location area), and it is 5GC to decide UE list belongs to the UE collection.

Scenario#2: the application indicates 5GC to select same EAS or the same DNAI for the UE collection without providing UE list belongs to the UE collection explicitly.

6.X.2 Procedures

The following is the procedure for selecting the same EAS for collection of UEs accessing the same application. The procedures defined in Figure 4.3.6.2-1 TS23.502[x] and Figure 6.2.3.2.2-1 TS23.548[03] are reused.

There could be only one SMF for serving the collection of UEs that connecting to the same EAS/DNAI for accessing the same application, or there could be multiple SMF for different UEs in the collection. And for the latter case, UDR is used for coordination between SMFs to make sure selecting the same EAS/DNAI for UEs, i.e. SMF send the selected EAS IP/DNAI to UDR or SMF gets the updated EAS IP/DNAI from UDR.

Figure X: Discovery procedure for selecting the same EAS/DNAI for collection of UEs

1.

The AF request in Step 1 of Figure 4.3.6.2-1 TS23.502 is used to request selecting the same EAS or same DNAI for UEs accessing the application as identified in the AF Request. An eas_correlation indication or dnai_correlation indication is provided for indicating selecting the same EAS or the same DNAI (i.e. selecting EAS corresponding to the same DNAI) for collection of UEs accessing the same application (e.g. FQDN), Spatial Validity Condition could be provided for limiting the location of the UEs, and also “any UE” or an UE list will be provided for defining UE collection accessing the same EAS or the same DNAI (the UE list could be determined by AF based on application layer mechanism). The DNAI could be determined and provided by AF to PCF and then to SMF.

In Step 5 of Figure 4.3.6.2-1 TS23.502, PCF determines the UEs influenced by the AF Request, and based on AF request, PCF creates PCC rule with FQDN, and eas_correlation indication or dnai_correlation indication to SMF.

2. The same as step 1~9 in Figure 6.2.3.2.2-1 TS23.548 [03].

3. If FQDN in Neasdf_DNSContext_Notify Request is for the application (e.g. FQDN) indicated in AF request , and if eas_correlation indication is set, SMF determines the UE belongs to collection of UEs accessing the application and selects the same EAS as UEs in the UE collection; or if dnai_correlation indication is set, SMF determine the UE belongs to collection of UEs accessing the application and selects EAS corresponding to the same DNAI as UEs in the UE collection.

4. In case of multiple SMFs, SMF synchronizes with UDR and receives EAS IP or DNAI for the UE collection. UDR maintains EAS IP or DNAI for the UE collection.

5. Based on step 10~19 in Figure6.2.3.2.2-1 TS23.548[03]:

For selecting the same EAS case: if the same EAS for the UE collection has not been determined yet, step 10~15 is used for discovering an EAS, and between Step 14 and Step 16, SMF could send the selected EAS IP to UDR; When the same EAS for the UE collection has been determined, in step 17, SMF sends DNS message handling rule with IP address for the EAS instructing EASDF to return the IP address for the EAS to UE in Step 19, step 10~15 are skipped;

For selecting EAS corresponding to the same DNAI case: if no DNAI for the UE collection has been determined, in Step 10 SMF determines DNAI for the UE collection and selects information to build ECS option or Local DNS server based on the DNAI; When DNAI for the UE collection has been determined, in Step 10 SMF determines information to build ECS option or local DNS server related to the DNAI, and sends DNS message handling rule with the information to build ECS option or local DNS server.

6. If UDR is used for coordination between SMFs for selecting the same EAS or the same DNAI for collection of UEs, SMF synchronizes EAS IP/DNAI with UDR.

6.X.3 Impacts on services, entities and interfaces

AF is to be updated with eas_correlation indication/ dnai_correlation indication for indicating selecting the same EAS/DNAI for collection of UEs accessing the application.

SMF is to be updated for selecting the same EAS/DNAI for collection of UEs.

UDR is to be updated for maintaining mapping between the collection of UEs and EAS IP/DNAI for the application.

EASDF is to be updated for create and send DNS response to UE.

Npcf_SMPolicyControl_UpdateNotify service is to be updated to transmit PCC rule with eas_correlation indication/ dnai_correlation indication.

# * * * * End of changes * * * *