3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202399

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update on Solution#1 to support multiple satellite backhaul

Document for:	Approval

Agenda Item:	9.12

Work Item / Release:	FS_5GSATB / Rel-18

Abstract: Update on solution#1 to support the scenario that gNB may connect multiple satellite backhaul on NG interface.

# 1. Introduction

In last meeting, solution#1 for KI#1 was approved. There is an EN as following:

Editor's note:	It is FFS whether and how the detection of satellite backhaul is based on local configuration if a gNB simultaneously has multiple backhaul connections over different satellites on NG-U interface.

The scenario that gNB simultaneously connects multiple satellite backhaul can be shown as below:

Figure: gNB has multiple satellite backhaul simultaneously

As shown in the figure, the gNB connects the 5GC on N2 and N3 interface over different satellite backhaul simultaneously, e.g. GEO satellite backhaul, LEO satellite backhaul, LEO satellite backhaul with ISL.

In R17, it assumes that there is only one satellite backhaul between gNB and 5GC. AMF can determine the Satellite backhaul category (i.e. GEO, MEO, LEO or OTHERSAT) based on local configuration, e.g. based on Global RAN Node IDs associated with satellite backhaul.

While for the scenario shown as the figure, the AMF cannot determine the Satellite backhaul category based on Global RAN Node ID.

To solve the above issue, this contribution proposes a solution with the assumption that N2 and N3 use the same satellite backhaul. In this solution, AMF determines Satellite backhaul category based on CP Transport Layer Information used on N2 interface. For example, gNB uses one Endpoint IP Address for GEO satellite backhaul and uses another different one for LEO satellite backhaul. The AMF can determine the Satellite backhaul category based on the Endpoint IP Address used on N2 interface.

This solution has no impacts on N2 and N3 interfaces.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-27.

# * * * * First change * * * *

## 6.1	Dynamic delay solution

### 6.1.1	Description

In Rel-17 reference architecture specified in TS 23.501 [2] and TS 23.502 [3], the AMF is configured with the awareness of satellite backhaul categories as specified in clause 5.8.2.15 of TS 23.501 [2]. The AMF informs the SMF of satellite backhaul category that is being used when PDU session is established and after change or handover causing different backhaul connection to be used. In roaming case, the V-SMF relays the satellite backhaul category to H-SMF. The PCF is informed of satellite backhaul category as the SMF includes it SM Policy Association establishment. Based on this knowledge, the PCF can set the QoS policies accordingly.

NOTE 1:	For a gNB only has single type of satellite backhaul, the AMF detection of satellite backhaul is based on local configuration, e.g. based on AMF awareness of Global RAN Node ID.

A gNB may have multiple satellite backhaul on N2 and N3 interfaces, e.g. GEO satellite backhaul, LEO satellite backhaul, LEO satellite backhaul with ISL. The following figure gives an example scenario:

Figure 6.1.1-1: example scenario that gNB has multiple satellite backhaul

For such scenario, the gNB uses different CP Transport Layer Information on N2 interface. The AMF can determine the Satellite backhaul category (i.e. GEO, MEO, LEO or OTHERSAT) based on local configuration and CP Transport Layer Information used on N2 interface.

NOTE 2: It’s assumed that N2 and N3 interfaces use the same satellite backhaul.

Awareness of QoS changes is needed to adapt this Rel-17 solution to the dynamic delay requirement in Rel-18. That awareness of variable delay can be obtained by re-using the capability for the SMF to request QoS monitoring from the UPF that is specified in clause 5.33.3.3 of TS 23.501 [2].

In addition to the already specified use cases where the SMF informs the PCF of the satellite category, the SMF needs to be enhanced to measure the N3 using the echo procedure and to inform the PCF of not only satellite category but also the observed delay.

# * * * * End of changes * * * *