3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202409

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, KDDI

Title:	KI#5: Solution for differentiated QoS handling for different PDU Sets

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: A new solution for KI#5 is proposed to support differentiated QoS handling for different PDU Sets.

# 1. Introduction

This paper introduces a solution for KI#5 “Differentiated PDU Set Handling”.

In media layer, depending on media codec, profile and configuration, a video frame or video slice is considered as base unit for decoding/handling. The importance/dependency information of PDU Set that carrying the frame or video slice should be identified and handled differentially by 5GS to reduce the resource wasting or improve the user service experience.

This contribution proposes a solution to support differentiated QoS handling for different PDU Sets.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[6]	3GPP TR 22.847: "Study on supporting tactile and multi-modality communication services; Stage 1".

[x]	IETF RFC 6184: "RTP Payload Format for H.264 Video".

[y]	IETF draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[z]	IETF RFC 6190: "RTP Payload Format for Scalable Video Coding".

# * * * * Second change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> | <Key Issue #5> | <Key Issue #6> | <Key Issue #7> | <Key Issue #8> | <Key Issue #9> |
| #1 |  |  |  |  |  |  |  |  |  |
| #2 |  |  |  |  |  |  |  |  |  |
| #x |  |  |  |  | X |  |  |  |  |

# * * * * Third change (All new texts) * * * *

## 6.X	Solution #X: Support differentiated QoS Handling for different PDU Sets

### 6.X.1	Key Issue mapping

This solution corresponds to KI#5 on Differentiated PDU Set Handling.

### 6.X.2	Description

For interactive media services, e.g. Cloud XR, cloud gaming, real-time video based remote control, PDUs of the media services carry different frame/video slice. For example, in GOP, the first I-frame is considered as the most important frame, other P-frames are encoded based on the I-frame. To ensure the limited radio resources be used to guarantee the successful delivery of the most important packet, 5GS can bring better user experience with same resource. 5GS should be aware of the different importance/dependencies of PDU Sets to achieve such target.

The design of typical media codec and transporting protocol, e.g. RTP, have already considered such transporting requirements and provided importance information via RTP header or NAL (Network Abstract Layer) header (See details in clause 6.X.3.2), so that network node can protect more important NAL units better than they do less important NAL units as described in clause 5.3 of RFC 6184[x].

In this solution, the following aspects are included:

1.	AF initiates AF session with required QoS as described in clause 4.15.6.6 of TS 23.502 [3].

2.	PCF determines updated or new policy information based on AF requirements. The policy information includes detection template for PDU Set importance/dependency and different QoS parameters for different PDU Sets, and optional dependency related instructions. The PCF initiates PDU Session Modification procedure and sends the policy information to SMF.

3.	Based on the policy information, the SMF assigns different QFIs with corresponding QoS parameters for different PDU Sets. The SMF sends N4 rules including PDR and FAR to UPF and N2 SM information including QoS Profiles to RAN.

4.	When the UPF receives DL packets from the application server, the UPF identifies the importance/dependency information of PDU Set of the packet and maps the packet into corresponding QoS Flow. Details on how to identify the importance/dependency information of PDU Set is described in clause 6.X.3.2.

5.	RAN receives DL packets from the UPF. The RAN handles the packets based on N2 SM information from SMF.

The dependency related instructions include:

-	allow for extended transmission for reference PDU sets, i.e. reference PDU set will not be dropped even its delay exceeds PDB for a limited time duration

-	drop the PDU sets if transmission of its reference PDU set is considered as failed.

### 6.X.3	Procedures

#### 6.X.3.1 Processing of PDU Sets based on different importance/dependency information

Figure 6.x.3.1-1 shows the procedure for supporting differentiated QoS handling for different PDU Sets.

Figure 6.x.3.1-1: Differentiated QoS Handling for different PDU Sets

0.	The UE establishes a PDU Session as defined in clause 4.3.2.2.1 of TS 23.502 [3].

1.	AF sets up an AF session with required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3].

The AF request may include flow description for XR/media services, the dependency information of PDU Sets (See option 3 of clause 6.X.3.2.1), and traffic pattern (See option 4 of clause 6.X.3.2.1).

2.	The PCF derives detection template for PDU Set importance/dependency and different QoS parameters for different PDU Sets and makes policy decision. Optionally, the PCF may derive the dependency related instructions in the PCC rule. The PCF initiates the PDU Session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3], steps 1b, 3b, 4-8b. The PCF sends policy information to SMF.

3.	The SMF assigns different QoS Flow IDs and QoS parameters for the PDU Sets of the service data flow based on their importance/dependencies. For example, the SMF assigns a QoS Flow ID and corresponding QoS parameters for important PDU Set (e.g. those I-frame or base layer which are references of other frames and layers hence better not be dropped), and another QoS Flow ID and corresponding QoS parameters for other PDU Set (e.g. other frames or additional layers that have dependence on the I-frame or base layer). Different QoS handling on PDU Sets may be performed.

-	For important PDU Set, prioritized admission control/resource scheduling can be performed. These PDU Set should be delivered to the UE no matter whether PDB is exceeded or not.

-	For other droppable PDU Set, perform correlated packets dropping, resource pre-emption.

The SMF sends N4 rules to the UPF to indicate the UPF to identify importance/dependency information of PDU Sets for received packets and map the packets to corresponding QoS Flows. Details on how the UPF identify importance/dependency information of PDU Sets are described in clause 6.X.3.2.

The SMF also sends N2 SM information including QoS profiles to RAN.

4.	The UPF identifies the importance/dependency information of PDU Sets for the DL packets and maps the packets into corresponding QoS Flow based on N4 rules received in step 2(See details in clause 6.X.3.2). UPF sends the DL packets to RAN via different QoS Flows. UPF may also provide dependency information between PDU Sets to RAN via GTP-U header.

5.	The RAN handles the DL packets based on N2 SM information received in step 2 considering the above different QoS handling of the PDU Set. In addition, if dependency information is provided and a PDU Set has been dropped, the other PDU Sets depends on this dropped PDU Sets should also be dropped.

#### 6.X.3.2 PDU Set Importance/dependency information identification on UPF

Real-time Transport Protocol (RTP) are widely used for data with real-time characteristics, such as interactive audio and video.

To support differentiated QoS handling for PDU Sets in 5GS, the UPF should firstly identify the PDU Sets (corresponding to a frame or video slice), then identify the importance/dependency of different PDU Set.

NOTE x:	Solution X (S2-2202408) for KI#4 described how to identify PDU Set in UPF. This clause will focus on how to further identify the importance/dependency information of different PDU Sets.

##### 6.X.3.2.1	Identification of importance/dependency information of PDU Set as a video frame.

If the PDU Set represents a video frame, the identification of importance/dependency information of the PDU Set can be realized via following options.

Option#1. Identification based on RTP header extension

The RTP header extension defined in draft-ietf-avtext-framemarking [y] is as following:

Figure 6.x.3.2.1-1: Frame Marking RTP header extension for non-scalable streams

In the above RTP header extension (see clause 3.1 of draft-ietf-avtext-framemarking [y]):

-	The I field named as Independent Frame reflects whether a frame can be decoded independent of temporally prior frames or not. 1 means this frame can be decoded independent of temporally prior frames, e.g. intra-frame, IDR frame of H.264/H.265 or VP8/9 keyframe.

-	The D field named as Discardable Frame which reflects whether a frame is droppable or not. 1 means the sender knows this frame can be discarded, and still provide a decodable media stream.

UPF can based the above I field or D field to identify the important PDU Set/frame.

If scalable streams (i.e. different temporary, spatial, quality layers) is used, the RTP header extension is defined as following:

Figure 6.x.3.2.1-2: Frame Marking RTP header extension for scalable streams

The TID field named as Temporal ID identifies the temporal layer/sub-layer encoded, starting with 0 for the base layer, and increasing with higher temporal fidelity. Higher values of TID identify higher temporal layers with higher frame rates as defined in [y].

The LID field named as Layer ID identifies the spatial and quality layer encoded, starting with 0 for the base layer, and increasing with higher fidelity. Higher values of LID identify higher spatial and/or quality layers with higher resolutions and/or bitrates as defined in [y].

UPF can based on the TID and LID field to identify the independent PDU Set/base layer.

Option#2. Identification based on NALU header

For each RTP packet, a NAL unit header is included following RTP header. For RTP payload carrying H.264 video slices, according to RFC 6184 [x], the NAL unit header is as following:

Figure 6.X.3.2.1-3: NAL unit header format

In the NAL unit header, the NRI value indicates the relative transport priority, as determined by the encoder. According to clause 5.3 of RFC6184 [x], a value of 00 indicates that the content of the NAL unit is not used to reconstruct reference pictures. Such NAL units can be discarded without risking the integrity of the reference. Values greater than 00 indicate that the decoding of the NAL unit is required to maintain the integrity of the reference pictures. The highest transport priority is 11, followed by 10, and then by 01; finally, 00 is the lowest.

Therefore, UPF can use the NRI value to identify the importance information of NAL unit carrying the PDU Set/slice. In case the frame includes only one slice, the method can be used to detect the importance of the frame.

If scalable streams are used, the RTP NAL unit header is defined in RFC 6190 [z] can be used to identify the importance/dependency information of PDU Set in different media layers.

The scalable streams use NAL unit type 14, 15, and 20 for prefix NAL unit, subset sequence parameter set and coded slice in scalable extension respectively as described in Figure 6.X.3.2.1-3. NAL unit type 14 and 20 indicate the presence of three additional octets in the NAL unit header, as shown below.

Figure 6.X.3.2.1-4: additional octets in the NAL unit header

The PRID field named as priority_id specifies a priority identifier for the NAL unit. A lower value of PRID indicates a higher priority.

The DID field named as dependency_id indicates the inter-layer coding dependency level of a layer representation. At any PDU Set, a layer representation with a given dependency_id may be used for inter-layer prediction for coding of a layer representation with a higher dependency_id, while a layer representation with a given dependency_id shall not be used for inter-layer prediction for coding of a layer representation with a lower dependency_id. The dependency_id gives the details on the dependency relationship between different layers. For example, a layer of DID=n will be decoded based on another layer of DID=n-1.

UPF can determine importance information based on PRID field and/or determine dependency information based on the DID field.

NOTE x:	Option 2 depends on the readability of NALU header in RTP payload. If RTP payload is encrypted, e.g. SRTP is used, option 2 will not applicable.

Option#3. Identification based on assistance information from AF

The media application (i.e., AF) may provide to 5GS dependency information of PDU sets in the form of GOP structure. 5GS derives the importance/dependency relationship between different PDU sets using the GOP structure.

NOTE x:	Option#3 applies to the scenario where dependency information of PDU Set is static.

As an example, such frame dependency information can be indicated using GOP structure (N, M), where N indicates the number of frames in one GOP and M indicates the distance between two anchor frames (I or P). For example, if M=3 and N=12, the GOP structure is IBBPBBPBBPBBI.

Option#4. Identification based on traffic pattern

In general, different PDU Sets have different traffic patterns, e.g. size of one PDU Set, number of packets within one PDU Set. For example, the size of I frame is generally 2-3 times larger than P frame. Therefore, the traffic patterns can be used to identify the frame type and hence the importance information of PDU Set.

NOTE x:	Accuracy of the detection in option 4 depends on the implementation of UPF.

##### 6.X.3.2.2	Identification of importance/dependency information of PDU Set as a video slice

When the PDU Set stands for a video slice, the RTP Payload as defined in RFC 6184 [x] can be used to identify the priorities between different video slices. Details can be found in option#2 of clause 6.X.3.2.1.

### 6.X.4	Impacts on services, entities and interfaces

AF:

-	Optionally provide importance/dependency assistance information of PDU Sets to 5GC.

PCF:

-	Optionally receive importance/dependency assistance information from AF;

-	Generate different QoS parameters and optionally dependency related instructions) for different PDU Sets.

SMF:

-	Assign different QoS Flows for different PDU Sets.

-	Instruct UPF to identify importance/dependency information of PDU Sets and map PDUs of different PDU Sets into corresponding QoS Flow accordingly.

-  Optionally receive dependency related instructions from PCF and distribute to RAN.

UPF:

-	Identify importance/dependency information of PDU Sets for received DL packets and map the PDUs into corresponding QoS Flow based on SMF instructions.

-	Optionally provide dependency info between different PDU Sets via GTP-U header.

RAN:

-	Optionally receive dependency information of PDU sets and dependency related instructions, identify the dependency information of GTP-U header and perform differentiated PDU Set level packet handling/treatment accordingly.

NOTE: The details of RAN behaviour is up to RAN WG.

# * * * * End of changes * * * *