3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202526

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	new solution to address PALS key issues

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract: A new solution to address PALS key issues: Enabling NPN as hosting network for providing access to localized services, and Enabling access to localized services via a specific hosting network.

1. Introduction/Discussion

A new solution to address PALS key issues: Enabling NPN as hosting network for providing access to localized services, and Enabling access to localized services via a specific hosting network.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-08 V0.1.0.

# * * * * First change * * * *

6.X	Solution #X: Exposure enhancements to support providing access to localized services

6.X.1	Introduction

Editor's Note:	This clause lists the key issue(s) addressed by this solution, and briefly the main principles of the solution.

This solution aims at address key Issue #3 (Enabling NPN as hosting network for providing access to localized services) and Key Issue #5 (Enabling access to localized services via a specific hosting network), in particular:

-	How localized service agreements (i.e. a service agreement for a localized service) for a specific occasion (time and location) are automatically established and terminated,

-	What is required to enable communication between a network operator deploying a hosting network and a localized services provider: Investigate which type of interaction (e.g., configuration of the hosting network, information reporting) is needed, in such relation to enable the localized services provider for making the best use of the hosting network,

-	How and whether the home network, determine the service availability of a hosting network, and interacts with hosting network to authorize home network's subscribers to access home network services via the hosting network, at certain time and location, coverage of the hosting network and services offered by the hosting network,

-	How to enable UE to access both home network services and localized services via the hosting network, and seamless service continuity for home network services and localized services when UE moves between different networks providing the same services. This includes how to configure UE with information enabling the UE to be aware of services that can be accessed via a specific network (e.g. home network or hosting NPN).

-	How home network determines the need to steer or instruct the UE, and how the home network steers or instructs the UE to select a hosting network for obtaining home network services or localized services or select a network for a specific service which is available from both hosting and home network.

The main principles of this solution are that:

-	The localized service provider can act as the AF to manage the localized service agreements among Hosting Network operator, localized service provider, and Home Network or Credentials Holder via exposure interface;

-	The localized service provider can act as the AF to manage the subscribed localized service information in subscription data within the Home Network or Credentials Holder or Hosting network via network exposure. This can be performed on a demand basis;

-	The network selection information for the target Hosting Network is provisioned/signalled to the UE when the UE is registered in the Home Network; the network selection information is used for discovery and selection of the target Hosting Network; Optionally, the network selection information can be signalled to the UE under certain conditions e.g. when UE enters specific area and/or after certain time;

-	When the UE registers in the Hosting Network, the Home Network or the Credentials Holder performs primary authentication and authorization for the UE;

-	When the UE accesses to the localized service via Hosting Network, the Hosting Network initiates Network Slice-Specific Authentication and Authorization or PDU Session Secondary Authentication and Authorization to authorize the UE access to the localized service;

-	When the UE accesses to the localized service via Hosting Network, the Hosting Network uses the time and location allowed for the UE to perform access control to the localized service;

6.X.2	Functional Description

Editor's Note:	This clause further details the solution principles and any assumptions made.

Providing access to localized services refers to the capability to provide access to a hosting network (PNI-NPN or SNPN) and a set of services offered by the hosting network provider, other mobile network operators (PLMN or SNP) and 3rd party application providers. The services may be localized (i.e. provided at specific/limited area) and may be bounded in time.

The UE’s with or without prior subscription to the hosting network can access to the localized services via hosting network using access credentials from Home Network or Credentials Holder once the UE has subscribed successfully to the localized services of the hosting network. Then this solution assumes that:

-	The localized service provider acting as the application function manages the service agreements among Hosting Network operator, localized service provider, and Home Network or Credentials Holder via network exposure. The service agreements include:

-	Availability of the localized service (service identification, service parameters [DNN, S-NSSAI, QoS], service authorization methods [NSSAA or PDU Session SAA], service access methods [LBO or HR], time and location); and

-	List of Supported Home Networks or Credentials Holders; and

-	List of Supported Hosting Networks;

-	UE/User can subscribe to the localized service for a specific Hosting Network via out-of-3GPP means, e.g. online store, portal:

-	The UE should be a subscriber of the supported Home Network or should have credentials from the supported Credentials Holder in the service agreement; and

-	The target Hosting Network should be one in the list of supported Hosting Networks; and

-	The localized service provider should know localized service information selected by the UE/User, e.g., subscribed DNN, S-NSSAI, QoS, service authorization method, service access method, time and location, this information selected by the UE/Use should be covered by the Availability of the localized service in the service agreement. The localized service should also know the information of the subscribed Hosting Network and Home Network or Credentials Holder; and

-	The UE/User can obtain the service credentials assigned by the localized service, the service credentials can be used to authenticate and authorize the UE access to the localized service;

-	The localized service provider acting as the application function, updates subscription data in the subscribed Home Network or Credentials Holder or Hosting network via network exposure, so the subscription data can contain the localized service information subscribed by a single UE or a group of UEs:

-	If the UE will access the Hosting Network using credential from Credentials Holder with AAA Server, then the localized service updates the subscription data in the subscribed Hosting Network, so the subscription data contains the indication that primary AA by AAA-S is required as well as the subscribed localized service information e.g., subscribed DNN, S-NSSAI, QoS, service authorization method, service access method, time and location.

-	If the UE will access the Hosting Network using credential from Credentials Holder with AUSF and UDM, then the localized service updates the subscription data in the subscribed Home Network, so the subscription data contains the subscribed Hosting Network information as well as the subscribed localized service information e.g., subscribed DNN, S-NSSAI, QoS, service authorization method, service access method, time and location.

-	If the UE will access the Hosting Network using credentials from Home Network, then the localized service updates the subscription data in the subscribed Home Network, so the subscription data contains the subscribed Hosting Network information as well as the subscribed localized service information e.g., subscribed DNN, S-NSSAI, QoS, service authorization method, service access method, time and location.

-	The Home Network updates UE with the network selection information for the subscribed Hosting Network (e.g. service identification, Hosting Network ID, temporal validity condition, spatial validity condition) when subscribed time starts and/or when UE enters specific area indicated by the subscribed location contained in the subscription data:

-	The temporal validity condition is derived from the subscribed time for the localized service, this indicates the time period when the network selection information is valid;

-	The spatial validity condition is derived from the subscribed location for the localized service, this indicates the area where the network selection information is valid;

-	The UE moves in the subscribed location for the localized service and the temporal validity condition is still valid, and starts discovery and selection of the Hosting Network.

-	The UE registers in the Hosting Network with the access credentials and accesses to the localized services using the service credentials.

-	The UE is authenticated and authorized by the Home Network or the Credentials Holder;

-	The UE can be provisioned with information related with the subscribed localized service (Allowed NSSAI, service authorization method, URSP rules);

-	The UE setups the connectivity to the localized service by establishing a PDU Session with the DNN, S-NSSAI associated with the localized service, the Hosting Network can allocate resources for the PDU Session according to the subscribed QoS or service access method;

-	With the subscribed service authorization method, the Hosting Network performs the service authorization using the indicated authorization method;

-	With the subscribed time and location, the Hosting Network performs access control to the localized service;

-	When the UE moves out the subscribed location for the localized service or the temporal validity condition becomes out-dated,

-	The UE de-registers from the Hosting Network, the UE may delete the network selection information for the subscribed Hosting Network and information related with the subscribed localized service;

-	The Hosting Network de-registers the UE and releases network resources reserved for the UE;

-	The localized service provider may delete the localized service information from subscription data in the Home Network or Credentials Holder or Hosting network;

Figure 6.X.2-1 depicts the scenarios for support of providing access to localized services.

Figure 6.X.2-1: Support of providing access to localized services via exposure enhancements

6.X.3	Procedures

Editor's Note: This clause describes procedures and information flows for the solution.

6.X.4	Impacts on services, entities, and interfaces

Editor's Note: This clause lists impacts to services, entities, and interfaces.

# * * * * End of changes * * * *