3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202539

Elbonia, April 6– 10, 2022	(revision of S2-220XXXX)

Source:	Huawei, HiSilicon

Title:	Remove the EN on KI#4

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This paper removes the EN on KI#4.

1. Introduction

At the S2#149E meeting one EN is added at the KI#4 related to possible update the content of the key issue per the progress of the Rel-17 work. There are no further progress on the multi NSACF issue at the S2#149E meeting. It is proposed to remove that EN.

2. proposal(s)

It is proposed to capture the following changes vs. TR23.700-41.

# * * * * First change * * * *

5.4	Key Issue #4: Support of NSAC involving multi service Area

5.4.1	Description

For one S-NSSAI, there is only one configured global Maximum allowed number value for NSAC. It is possible more than one service area is associated with one S-NSSAI, e.g. to split a PLMN into multi-service areas. This impacts the use cases below as there will be more than one NSACF handling the UE:

-	Multi NSACF deployed within one PLMN: For NSACF deployment more than one service area are defined within one PLMN. For each service area one NSACF or NSACF set is selected for slice admission control. This include the control of the maximum allowed number of UE or PDU session.

-	Roaming: when one user resides at the visit PLMN, the NSAC (Maximum PDU session) may be controlled by the NSACF in the VPLMN (e.g. for LBO PDU session), or the NSACF in the HPLMN (e.g. for HR PDU session).

-	EPS interworking: when the user establishes a HR PDN connection at the EPS network and move to 5GS later, the NSACF(Maximum UE number) selected by the SMF+PGW-C and AMF may be different.

This key issue addresses the above cases, to ensure consistent NSAC handling against the configured global Maximum allowed number. The following aspects will be covered by the key issue:

-	UE Registration.

-	PDU Session establishment.

-	Session continuity when UE move across the service area.

# * * * * End of change * * * *