SA WG2 Meeting #150E (e-meeting)	S2-2202565

April 6 – 12, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	KI#2, Sol#2: Update to add EN

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract of the contribution: This paper proposes to update Solution #2: Procedures for MOCN network sharing.

Discussion

Solution #2: Procedures for MOCN network sharing in TR 23.700-47v0.1.0 is based on the additional identifier (i.e. identifier of the broadcast MBS service) provided by AF during MBS session Create procedure.

We believe that the additional identifiers from different AFs must not overlap each other. Otherwise, NG-RAN cannot know that broadcast service from AF#1 and broadcast service from AF#2 are different when the same identifier is provided by both AFs and MBS session create procedures for both broadcast services with same parameters, e.g. same MBS service area are performed around or at the same time.

Therefore, it is proposed to add the following EN to Sol#2.

Editor's note:	How different AFs use different additional identifiers is FFS.

Proposal

It is proposed to agree the following changes into TR 23.700-47v0.1.0.

* * * * Start of 1st Change * * * *

6.2	Solution #2: Procedures for MOCN network sharing

6.2.3.1	General

NOTE:	The message names in the procedures below are descriptive. It is assumed that the names are updated with corresponding SBI based names where applicable during the normative phase.

In general the proposal is based on the additional identifier (i.e. identifier of the broadcast MBS service) provided by AF during MBS session Create procedure. The identifier of the broadcast MBS service is non-PLMN specific, which would be included and same when AF sends requests for establishing the broadcast MBS session for the same broadcast MBS service to different PLMNs. The MB-SMF includes the received identifier in the N2 MB-SM container, and provided to NG-RAN node.

Editor's note:	It will be confirmed by the RAN WG that whether the additional identifier is needed,

Editor's note:	Support of the encrypted content reception is FFS.

Editor's note:	How different AFs use different additional identifiers is FFS.

* * * * End of Changes * * * *