3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202619

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	Revision of Solution #2 Burst arrival time adaptation for KI#6

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: Providing a revision of solution#2 by resolving an EN.

1. Introduction/Discussion

In SA2#149e meeting, Solution#2 has ENs. Among them, the following needs to be solved.

Editor's note:	It is FFS whether there is need to perform timing translation for burst arrival time offset, e.g. between the external clock time and 5GS time for the case that the AF is time synchronized using an external clock.

External clocks have different from 5GS clock, and the difference is reported to the SMF by the UPF.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-25.

# * * * * First change * * * *

6.2	Solution #2: Burst arrival time adaptation

6.2.1	Introduction

This solution enables the network to adjust the burst arrival time by signaling positive or negative offset values (e.g. +3 ms) to the AF so that the AF can adjust the burst sending time accordingly.

6.2.2	Functional Description

This solution is based on the following principles:

-	As part of requesting QoS for a flow, AF includes burst size, flow direction and periodicity and may also indicate support of Burst arrival time adaptation to PCF (via NEF if AF is outside the trust domain).

Editor's note:	The solution description should also include the case where AF provides an absolute BAT when requesting QoS for a flow.

-	PCF forwards the support of Burst arrival time adaptation indication to SMF together with a PCC rule and other parameters (burst size, flow direction, burst periodicity).

-	SMF creates TSCAI based on the received periodicity and flow direction. Instead of including a burst arrival time, SMF includes support of Burst arrival time adaptation indication in TSCAI and signals TSCAI to NG-RAN as described in clause 4.3.3.2 of TS 23.501 [2].

-	Upon reception of DL packets from the UPF via user plane for a QoS Flow, if NG-RAN has received the indication of support of Burst arrival time adaptation in TSCAI for the given QoS Flow and NG-RAN determines the need to adapt the burst arrival time, NG-RAN sends an indication to SMF including a burst arrival time offset value. The burst arrival offset can take positive or negative values.

Editor's note:	It is FFS how it is possible to determine in advance whether a new flow with a given delay requirement can be admitted.

NOTE:	NG-RAN determines a relative burst arrival time offset value in reference to the current Burst Arrival Time experienced by RAN (i.e. in reference to when RAN currently receives bursts). Since it is a relative offset it can also be applied by the AF for adapting when it sends bursts, i.e. for the AF to adapt the burst sending time (see further below).

Editor's note:	It is FFS how the NG-RAN determines the accurate offset value and whether extra information is needed for such determination.

Editor's note:	It is FFS how often the NG-RAN should re-send the offset value to the SMF, when the NG-RAN determines a new offset for the given QoS Flow.

-	The burst arrival time offset value is signaled from SMF to AF via PCF/NEF. An external clock is different from the 5GS clock, and the offset between them is reported to the SMF by the UPF. Before the SMF report the arrival time offset value to AF, the SMF needs to translate the burst arrival time offset from 5GS time basis to the external clock time basis.

-	For downlink flows AF adapts the burst sending time based on the received offset.

6.2.3	Procedures

Existing procedures are reused (PDU session modification to signal support of Burst arrival time adaptation indication in TSCAI to NG-RAN; Notification control as defined in clause 5.7.2.4.1a of TS 23.501 [2]. to signal burst arrival time offset value to PCF/AF).

Editor's note:	It is FFS whether TSCTSF is involved in the procedures.

6.2.4	Impacts on services, entities and interfaces

-	AF: Support of sending burst arrival time adaptation indication and receiving burst arrival time offset.

-	NEF, PCF, SMF: Support of signalling burst arrival time adaptation indication and burst arrival time offset.

-	NG-RAN: Support of receiving burst time arrival adaptation indication, determining and signalling burst arrival time offset.

# * * * * End of changes * * * *