3GPP WG SA WG2 Meeting #150E e-meeting	S2-2002633

Elbonia, April 6th – 12th, 2022	(revision of S2-2xxxxxx)

Source:	China Mobile

Title:	New Solution on Horizontal Federated Learning among Multiple NWDAFs Instances

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3/ Rel-18

Abstract of the contribution: This contribution proposes a new Solution on enhancements on supporting Federated Learning in 5GC in FS_eNA_Ph3.

- Discussion

This contribution proposes a new solution, focusing on Horizontal Federated Learning, for Key Issue #8: Supporting Federated Learning in 5GC.

2.	Proposal

It is propose a new solution on enhancements on supporting Federated Learning in 5GC.

* * * * First Change * * * *

6.0 Mapping Solutions to Key Issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  | 8 Supporting Federated Learning in 5GC |  |  |
| X |  | X |  |  |

* * * * Second Change * * * *

6.X	Solution #X: Horizontal Federated Learning among Multiple NWDAFs

6.X.1	Description

This solution is proposed to address Key Issue #8: Supporting Federated Learning in 5GC.

Current enablers for network automation architecture by NWDAF still faces some major challenges as follows:

-	User data privacy and security (protected by e.g. GDPR) has become a worldwide issue, it is also difficult for NWDAF to collect UE level network data.

-	With the introduction of MTLF in R17, various data from wide area is needed to train an ML model for NWDAF containing MTLF. However, it is difficult for NWDAF containing MTLF to collect all the raw data from distributed data source in different areas, because heavy signaling load may exist if it centralized all the data into the NWDAF containing MTLF.

In order to address the challenges, Federated Learning (also called Federated Machine Learning) technique in NWDAF containing MTLF could be adopted to train an ML model, in which there is no need for raw data transferring (e.g. centralized into one NWDAF) but only need for cooperation among multiple NWDAFs (MTLF) distributed in different areas i.e., sharing of ML model and of the learning results among multiple NWDAFs (MTLF).

This solution is mainly focusing on horizontal Federated Learning among Multiple NWDAFs, whose procedures are illustrated in the following clauses.

6.X.2	Procedures

6.X.2.1	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.x.2.1-1: General procedure for Federated Learning among Multiple NWDAF Instances

1-3.	Client NWDAF registers its NF profile (Client NWDAF Type (see TS 23.502 [3] clause 5.2.7.2.2), Address of Client NWDAF, Support of Federated Learning capability information, Analytics ID(s)) into NRF.

4-6.	Server NWDAF discovers one or multiple Client NWDAF instances which could be used for Federated Learning via the NRF to get IP addresses of Client NWDAF instances by invoking the Nnrf_NFDiscovery_Request (Analytics ID, Support of Federated Learning capability information) service operation.

It is assumed an Analytics Id is preconfigured for a type of Federated Learning. Thus, the NRF can realize the Server NWDAF is requesting to perform federated learning based on the pre-configuration. And the NRF responds to the central NWDAF the IP address of multiple NWDAF instances which support the Analytics Id.

NOTE：	The analytic ID(s) supporting Federated Learning can be configured by operator.

7a:	Each client NWDAF communicates licensing conditions for its data and training infrastructure to participate in the federated learning task. These conditions can be based on policies set-up based on how sensitive the data is, how much compute is expected to be needed to perform local training, who will get the use the trained model, etc.

7b:	Based on the response from NRF, Server NWDAF selects which NWDAF clients will participate based on its desired license model.

7c.	Server NWDAF sends a request to the selected Client NWDAFs that participate in the Federated learning according to steps 7a and 7b including some parameters (such as initial ML model, data type list, maximum response time window, etc.) to help the local model training for Federated Learning.

8.	Each Client NWDAF collects its local data by using the current mechanism in clause 6.2, TS 23.288 [5].

9.	During Federated Learning training procedure, each Client NWDAF further trains the retrieved ML model from the server NWDAF based on its own data, and reports the results of ML model training to the Server NWDAF, e.g. the gradient.

10.	The Server NWDAF aggregates all the local ML model training results retrieved at step 9 such as the gradient to update the global ML model.

11.	The Server NWDAF sends the aggregated ML model information (updated ML model) to each Client NWDAF for next ground model training.

12.	Each Client NWDAF updates its own ML model based on the aggregated model information (updated ML model) distributed by the Server NWDAF at step 11.

NOTE:	The steps 8-12 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is finished, the globally optimal ML model or ML model parameters could be distributed to the Client NWDAFs for the inference.

6.x.2.2	Procedure for usage of Federated Learning in Abnormal Behaviour

Figure 6.x.2.2-1: Procedure for usage of Federated Learning in Abnormal Behaviour

1.	A Server NWDAF determines to train an Abnormal Classifier using the input data defined in Table 6.7.5.2-1, TS 23.288 [5]). The Server NWDAF discovers two Client NWDAFs i.e. Client NWDAF 1 in Area A, Client NWDAF 2 in Area B that are participating in the license model as defined in steps 7 in figure 6.x.2.1-1.

2.	Client NWDAF 1 and Client NWDAF 2 collect raw data from SMF and AMF, respectively. The data collection procedures from AMF and SMF are defined in step 2-3, clause 6.7.5.4, TS 23.288 [5].

3-4.	Client NWDAF 1 and Client NWDAF 2 collect Exception information from AF, respectively. The Exception information is defined in clause 6.7.5.2, TS 23.288 [5].

Repeat from step 5-8 until the training termination condition (e.g. maximum number of iterations) is fulfilled:

5.	During Federated Learning training procedure, based on the local raw data, Client NWDAF 1 and Client NWDAF 2 further train the retrieved Abnormal Classifier from the Server NWDAF, respectively.

6.	Client NWDAF 1 and Client NWDAF 2 respectively report the information of local training results for Abnormal Classifier (e.g. volume of the local dataset, parameters for update of the Abnormal Classifier) to the Server NWDAF.

7.	Server NWDAF aggregates all the local training results for Abnormal Classifier to derive the aggregated Abnormal Classifier information.

8.	Server NWDAF sends the aggregated Abnormal Classifier information to Client NWDAF 1 and Client NWDAF 2, and then, Client NWDAF 1 and Client NWDAF 2 update the its own Abnormal Classifier based on the aggregated Abnormal Classifier information distributed by the Server NWDAF.

After the training procedure is finished, the globally optimal Abnormal Classifier or Abnormal Classifier parameters could be distributed to the Client NWDAF 1 and Client NWDAF 2 as well as the new Client NWDAFs, if requested, for the inference.

6.X.3	Impacts on Existing Nodes and Functionality

NWDAF:

-	Registration into NRF;

-	New Data Analytics (i.e. new Analytics ID) for Federated Learning, which can be configured by operator

-	As a server NWDAF,

-	Discover request to NRF with the analytics id corresponding to a federated learning

-	Client NWDAF instances discovery;

-	Receiving licensing condition from Client NWDAF to check the Client NWDAF's participation in the federated learning task;

-	Initial Federated Learning parameters determination and distribution;

-	Collection of local ML model information from the Client NWDAF instances;

-	Aggregates local model information to update ML model, e.g. for gradient calculation, reporting to the Server NWDAF;

-	As a Client NWDAF,

-	Receive aggregated model information (updated ML model) from Server NWDAF and perform local ML model update.

-	Report the licensing conditions to the Server NWDAF.

NRF:

-	Server NWDAF and Client NWDAF registration and discovery.

* * * * End of change * * * *