3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202640

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	New KI: Interactions between NWDAF and MDAF for data collection and analytics

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract: It proposes a new key issue of Interactions between NWDAF and MDAS/MDAF to leverage MDAS/MDAF functionality for data collection and analytics.

1. Discussion

This contribution is related to WT#3.4:

WT#3.4: Study whether and how interactions between NWDAF can leverage MDAS/MDAF functionality for data collection and analytics

In SA5 Management Data Analytics (MDA) Technical Specification (TS 28.104), the architecture and interfaces are defined between MDA and NWDAF on the following aspects:

- NWDAF may leverage MDA reports for control purposes.

- CN Domain MDA may consume the analytics results provided by NWDAF.

To leverage the analytics from MDAS/MDAF for NWDAF can save the traffic for data collection and reduce the computation for data analysis. Therefore, the use cases for NWDAF to consume MDAS/MDAF analytics should be studied.

2. Text Proposal

It is proposed to adopt the following text within the TR.

# * * * * First change (All new text) * * * *

## 5.X	Key Issue #X: Interactions between NWDAF and MDAS/MDAF to leverage functionality for data collection and analytics.

### 5.X.1	General description

In SA5 Management Data Analytics (MDA)Technical Specification (TS 28.104), the architecture and interfaces are defined between MDAS/MDAF and NWDAF on the following aspects:

- The NWDAF may leverage MDA reports for control purposes.

- CN Domain MDAS/MDAF may consume the analytics results provided by the NWDAF.

Both NWDAF and MDAS/MDAF have the capability of data collection and analysis, the interaction with the MDAS/MDAF can reduce the traffic and computation resources(cpus, mems) for data collection and analytics efficiently and effectively.

Therefore, the following aspects are needed to be studied for interactions between the NWDAF and the MDAS/MDAF:

- 	Identify the use case that requires the MDAS/MDAF analytics for the NWDAF. What information is required from MDAS/MDAF?

- 	Whether and How to collect information from MDAS/MDAF? Whether new Event ID(s) are required for data collection from MDAS/MDAF?

-	Whether and how to enhance system architecture and interfaces between the MDAS/MDAF and the NWDAF?

# * * * * End of changes * * * *