3GPP TSG-WG SA2 Meeting #150E e-meeting	S2-2202662

e-Meeting, 6th-12th Apr, 2022

Title:	[Draft] Reply LS on Mapped NSSAI

Response to:	LS S2-2201936 / C1-222095 on Mapped NSSAI

Release:	Rel-17

Work Item:	5GProtoc17

Source:	SA2

To:	CT1

Cc:	None

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu dot Jinguo at ZTE dot com dot cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 would like to thank CT1 for their LS on Mapped NSSAI.

SA2 discussed the scenario mentioned in the CT1 LS and would like to provide the following clarifications:

- If the UE only uses standard S-NSSAI values, then the same S-NSSAI values can be used in VPLMN as in the HPLMN. In this case there is no S-NSSAI mapping between VPLMN and HPLMN.

- If the VPLMN and HPLMN have an SLA to support non-standard S-NSSAI values in the VPLMN, the NSSF of the VPLMN maps the Subscribed S-NSSAIs values to the respective S-NSSAI values to be used in the VPLMN.

- The S-NSSAIs in the NSSP of the URSP rules contain only HPLMN S-NSSAI values. The mapped S-NSSAI is used in the UE to associate the S-NSSAIs in URSP rules with the Allowed NSSAI of the VPLMN.

- The mapped S-NSSAI can be provided to the UE in either Allowed NSSAI or Configured NSSAI of the VPLMN.

Regarding the questions asked by the CT1, please see the answer below:

Question 1: Could SA2 confirm that it is indeed optional for the VPLMN to provide the mapped S-NSSAI for VPLMN S-NSSAI 2?

SA2 Answer: Yes.

Question 2: If the answer to question 1 is yes, could SA2 further clarify the conditions under which it is optional to provide the mapped S-NSSAI.

SA2 Answer: The mapped S-NSSAI is provided to the UE when the VPLMN and HPLMN have an SLA to support non-standard S-NSSAI values in the VPLMN, and if the AMF has never provided the mapped S-NSSAI to the UE before, or if the AMF needs to update the mapped S-NSSAI

Question 3: If the answer to question 1 is yes, could SA2 further clarify if it is mandatory for the network to provide the mapped S-NSSAI to the UE if the mapped S-NSSAI is known to the network?

SA2 Answer: See answer above.

Question 4: If the answer to question 3 is no, could SA2 further clarify how the UE determines the mapped S-NSSAI in that case?

SA2 Answer: See answer above

2. Actions:

To: CT1

ACTION:

SA2 asks kindly CT1 to take the above information into consideration

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #151E                               May 16-20, 2022

TSG-SA2 Meeting #152                                 Aug 17-26, 2022