3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202673

Elbonia, 6th – 10th Apr, 2022	(revision of S2-220xxxx)

Source:	ZTE

Title:	FS_eLCS_Ph3 New Solution to KI#2

Document for:	Approval

Agenda Item:	9.x

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract: This paper introduces a new solution to key issue #2

1. Introduction/Discussion

This paper propose a new solution to KI#2.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71 V0.1.0.

# * * * * First change * * * *

6.X	Solution #X: Direct communication between LMF and RAN node

6.X.1	Introduction

This solution aims to address the key issues#2: enhanced positioning architecture for NPN deployment. Particularly this solution address the following questions:

-	How to realize low latency positioning procedure under NPN deployment.

-	How to achieve reliable and secure location result delivery and exposure, e.g. UE location not exposed to the public network.

6.X.2	Functional Description

The following is the architecture figure in this solution

Figure 6.x.2: Reference architecture for Location Services

In this solution, the LMF has Nx interface with the RAN node and Ny interface with the GMLC. The Nx interface is to exchange directly the Network Positioning Message between the RAN and LMF for network assisted or network based positioning procedure, and obtaining Network Positioning Message for Non-UE Associated Network Assistance Data procedure, without going through the AMF. The Ny interface is used to exposure the UE location to GMLC directly without going through the AMF.

The TNL association between the RAN node and the LMF is pre-established via configuration, or dynamically established per request. For UE associated Network Positioning Message the AMF provides the RAN NGAP UE ID to LMF. The LMF allocates LMF NGAP UE ID and establishes NGAP association between the RAN node and LMF.

The GMLC notification address is provided from AMF to LMF. The LMF notifies the UE location estimation towards the GMLC directly.

Editor Note: It is FFS how to handle the Network Positioning Messages for UE Assisted and UE Based Positioning

6.X.3	Procedures

6.X.3.1	Network Assisted Positioning Procedure

Figure 6.x.3-1: Mobile Terminated Network Assisted Positioning Procedure

1.	Based on configuration the TNL association may be pre-established between the RAN node and the AMF.

2.	The GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF to request the current location of the UE. The service operation includes the SUPI, and client type and may include the required QoS and Supported GAD shapes.

3.	If the UE is in CM IDLE state, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [19] to establish a signalling connection with the UE.

4.	The AMF selects the LMF and invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE. This service operation may include the RAN node ID, the TNL Address and the RAN NGAP UE ID. This service operation may also include the GMLC notification address to receive the UE location.

5.	The LMF response Nlmf_Location_DetermineLocation response.

6.	The LMF establishes the TNL association towards the RAN node if it does not exist.

7.	The LMF sends a Network Positioning message in the N2 Transport message via the TNL association to the RAN node to request position related information from the RAN node. The LMF allocates an LMF NGAP UE ID and includes both the RAN NGAP UE ID and the LMF NGAP UE ID in the N2 Transport message

8.	The target RAN node returns any position related information to the LMF in a Network Positioning message included in an N2 Transport message. The N2 Transport message includes the LMF NGAP UE ID.

9.	The LMF invokes the Nlmf_EventExposure_Notification service operation towards the GMLC notification address received from AMF in step 4.

6.X.3.2	Obtaining Non-UE Associated Network Assistance Data

Figure 6.x.3.2: Non-UE Associated Network Assistance Data procedure

1.	The LMF establishes the TNL association towards the RAN node if it does not exist.

2.	The LMF sends a Network Positioning message in the Non UE associated N2 Transport message via the TNL association to the RAN node to request position related information from the RAN node.

3.	The target RAN node returns any position related information to the LMF in a Network Positioning message included in an Non UE associated N2 Transport message.

6.X.4	Impacts on services, entities, and interfaces

LMF:

- Establish TNL association and NGAP association with the RAN node

- Exchange the Network Positioning message with RAN node over the TNL association

- Send the UE location to the GMLC

AMF:

- Provides TNL address and RAN NGAP UE ID to the LMF

RAN node:

- Establish TNL association and NGAP association with the LMF

- Exchange the Network Positioning message with LMF over the TNL association

# * * * * End of changes * * * *