3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202674

Elbonia, 6th – 10th Apr, 2022	(revision of S2-220xxxx)

Source:	ZTE

Title:	FS_eLCS_Ph3 New Solution to KI#3

Document for:	Approval

Agenda Item:	9.x

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract: This paper introduces a new solution to key issue #3

1. Introduction/Discussion

This paper propose a new solution to KI#3.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71 V0.1.0.

# * * * * First change * * * *

6.X	Solution #X: LMF selection based on LMF ID

6.X.1	Introduction

This solution aims to address the key issues#3: Local Area Restriction for an LMF and GMLC. In this solution the LMF discovery and selection mechanism is enhanced to use LMF ID as input parameter.

6.X.2	Functional Description

In this solution, the GMLC is configured with an LMF ID of the LMF located within the same local area of the GMLC. The LMF may register itself in the NRF. The GMLC provides the LMF ID to AMF in Namf_Location_ProvidePositioningInfo service operation. The AMF uses the LMF ID to discover the LMF via NRF query or local configuration and invokes the Nlmf_Location_DetermineLocation service operation to request the current location of the UE.

6.X.3	Procedures

Figure 6.x.3-1: 5GC-MT-LR procedure

1.	The external location services client sends a request to the GMLC for a location for the target UE identified by an GPSI or an SUPI.

2.	The GMLC invokes a Nudm_UECM_Get service operation towards the home UDM of the target UE to be located with the GPSI or SUPI of this UE.

3.	The UDM returns the network addresses of the current serving AMF.

4.	The GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF to request the current location of the UE. The service operation includes the SUPI, and client type and may include the required QoS and Supported GAD shapes. The service operation may also include the LMF ID if it is configured in the GMLC.

5.	If the UE is in CM IDLE state, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [19] to establish a signalling connection with the UE.

6.	The AMF selects the LMF based on the LMF ID received from GMLC. The selection may use a NRF query or local configuration.

7-11. Same steps 7-11 in TS 23.273 clause 6.1.1.

6.X.4	Impacts on existing entities and interfaces

GMLC:

- Configured with LMF ID in the same location area

- Send the LMF ID to AMF in Namf_Location_ProvidePositioningInfo service operation

AMF:

- Select the LMF according to the LMF ID received from the GMLC

# * * * * End of changes * * * *