3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202683

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	New Solution on ML Model storage in ADRF

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes a new solution for Key Issue #4 "How to Enhance Data collection and Storage".

1. Discussion

This contribution is related to KI#4: How to Enhance Data collection and Storage, it is to be studied whether and how the ADRF should store types of data other than historical data and analytics (e.g. ML models, analytics context) for network analytics.

According to the definition of Rel-17, the 5G System architecture allows ADRF to store and retrieve the collected data and analytics. And an NWDAF containing AnLF is locally configured with (a set of) IDs of NWDAFs containing MTLF and the Analytics ID(s) supported by each NWDAF containing MTLF to retrieve trained ML models.

However, the storage location of the trained ML models is not defined yet. This proposal proposes to store the trained ML models in ADRF, and the ML model can be passed via DCCF/MFAF to reduce signalling and data traffic. When DCCF/MFAF is deployed, ML model consumers(e.g. NWDAF containing AnLF) can subscribe/require trained ML models via DCCF/MFAF.

2. Text Proposal

It is proposed to agree the following key issue into TR 23.700-81.

# * * * * First change (All new text) * * * *

6.0 Mapping Solutions to Key Issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 4 Enhance Data collection and Storage |  |  |  |
| X | X |  |  |  |

# * * * * Second change(All new text) * * * *

6.X	Solution #X: ML model storage in ADRF

6.X.1	Description

This solution addresses Key Issue #4 "How to Enhance Data collection and Storage".

An NWDAF containing MTLF may be consumed by multiple ML model consumers at the same time, resulting in repeated consumption signalling and data traffic of ML models with the same Analytics ID.

When the DCCF/MFAF is deployed in the network, the NWDAF containing MTLF can store trained ML models into ADRF via DCCF/MFAF. Meanwhile, the NWDAF containing MTLF can register the trained ML model of the Analytics ID it supports into DCCF, the registered ML profile may include the ID of NWDAF containing MTLF, Analytics ID(s), ML model information, the ML model consumers (e.g. NWDAF containing AnLF) can subscribe/request the needed ML model from ADRF via DCCF.

6.X.2	Procedures

Figure 6.X.2-1: Procedure for ML model storage in ADRF

1.	An NWDAF containing MTLF instance has trained ML Model.

2.	The NWDAF containing MTLF requests to register ML Model profile (Analytics ID, NWDAF ID, ML model filter information) to DCCF by invoking the Ndccf_MLModelManagement_Register.

- The DCCF responds to the NWDAF containing MTLF with a Ndccf_MLModelManagement_Register Response.

NOTE :	The procedure of NWDAF discovery and selection for trained ML models via NRF is defined in TS 23.288[4].

- The NWDAF containing MTLF sends the trained ML Model to the ADRF by invoking the Nadrf_MLModelManagement_StorageRequest (containing a (set of) file address of the trained ML model and/or ML model filter information and/or ML model) service operation or the NWDAF containing MTLF the sends trained ML Model to the ADRF via DCCF.

- The ADRF stores the trained ML Model sent by the NWDAF containing MTLF. The ADRF may based on implementation, determines whether the same trained ML Model is already stored or being stored based on the information sent in step 4 by the NWDAF containing MTLF and, if the trained ML Model is already stored or being stored in the ADRF, the ADRF decides to not store again the trained ML Model sent by the NWDAF containing MTLF.

- The ADRF sends Nadrf_MLModelManagement_StorageRequest Response message to the NWDAF containing MTLF indicating that the trained ML Model is stored, including when the ADRF may have determined at step 5 that the trained ML Model is already stored.

- The ML model consumer (i.e. an NWDAF containing AnLF) subscribes or requests a (set of) trained ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nadrf_MLModelProvision_Subscribe or Nadrf_MLModelInfo_Request service operation.

- The ADRF notifies the ML model consumer with the trained ML Model Information (containing a (set of) file address of the trained ML model) by invoking Nadrf_MLModelProvision_Notify or Nadrf_MLModelInfo_Request response service operation.

6.X.3	Impacts on Existing Nodes and Functionality

NWDAF containing MTLF:

-	Supports sending the trained ML model to ADRF.

-	Supports registering the ML Model profile to DCCF.

ADRF:

-	Supports Nadrf_MLModelManagement service to store the ML models.

-	Supports Nadrf_MLModelProvision or Nadrf_MLModelInfo_Request service to send the ML models.

DCCF:

-	Supporst Ndccf_MLModelManagement service to register the ML models.

# * * * * End of changes * * * *