3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2202687

# Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0109 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of PCC procedure for 5MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The currently described PCC procedures for 5MBS are unnecessarily complicated. It is however possible to simplify and streamline them more, so that the behavior of NEF, AF and MB-SMF is closer to the one in the 5MBS procedures without PCC. The NEF – MB-PCF interaction can be moved up so that an early check/authorization of the AF request by the MB-PCF becomes possible. This only requires that a single MB-PCF is configured for the whole network (e.g. in NRF) which should be necessary anyway to enable network wide policies and admission control for MBS sessions in the MB-PCF. When the MB-SMF is subsequently interacting with the MB-PCF, the MB-PCF has already the MBS session info available and can therefore directly provide the PCC rules in the response. With this the number of interactions between MB-SMF and MB-PCF can be reduced considerably. Furthermore, the early NEF – MB-PCF interaction has the advantage that, in case of a MB-PCF authorization failure (which should be more likely when many different AFs ask for the creation of an MBS session), all of the CN internal interactions (i.e. NEF to MB-SMF and MB-SMF to MB-PCF) are saved. Furthermore, assuming that the scenario of direct AF – MB-SMF interaction corresponds to “simpler” MBS sessions while the NEF scenario can support all kinds of MBS sessions and consequently, the potentially more complex and extensive MBS session info would not need to be passed through the MB-SMF in the successful MB-PCF authorization situation. Furthermore, for location dependent MBS, it can be ensured that the MBS session info is only provided once to the MB-PCF by the NEF instead of repeatedly by every involved MB-SMF. All procedures (i.e. MBS Session Creation, Update and Deletion) can be updated accordingly. To address the scenario with direct interaction between AF and MB-SMF appropriately, the structure of the procedures should depend on the scenario, i.e. whenever NEF is involved, the NEF – MB-PCF interaction takes place (so that an early check/authorization by MB-PCF is possible) while for the direct AF – MB-SMF interaction, the information relevant for the MB-PCF is provided by the MB-SMF (so that the AF procedure is less complex and the AF need to only interact with the MB-SMF). With this “conditional” execution of the NEF – MB-PCF interaction (as well as the “conditional” sending of MBS session information to MB-PCF via the MB-SMF) the need to support options in the procedures can be avoided and at the same time, the AF is only required to interact with a single NF, i.e. either the NEF or the MB-SMF for the management of the MBS session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following updates and changes are proposed for the MBS session management procedures: - The structure of the PCC procedures for 5MBS is changed so that the NEF – MB-PCF interaction happens before the NEF – MB-SMF interaction. - The PCC procedures for 5MBS are further simplified by reducing the MB-SMF – MB-PCF interaction to a single request response. The conditional provisioning of the MBS session information and if available, the MBS service area information, to the MB-PCF is clarified. - The parameters provided by the AF are now listed individually and it is clarified that MBS session information is only comprising the service requirements and media information. - The MBS Session ID is changed to a mandatory parameter for the MBS session creation procedure and the possibility for allocating the TMGI together with the creation of the MBS session is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessarily complicated PCC procedures for 5MBS leading to high standard maintenance and implementation efforts. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2, 7.1.1.3, 7.1.1.5, 7.1.1.6, 7.1.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.1.1.2	MBS Session Creation without PCC

This procedure is used by the AF to start the MBS Session towards 5GC and consist of TMGI allocation, and MBS session creation, and they apply to both multicast and broadcast communications unless otherwise stated.

For multicast, MBS session establishment procedure triggered by UE join requests may follow the MBS session creation procedure to reserve resources towards NG-RAN. For broadcast, the MBS session start procedure to reserve resources towards NG-RAN is triggered by the MBS session creation procedure.

For both broadcast and multicast communication, the TMGI allocation may be separated from the MBS Session creation request.

For multicast communication, TMGI allocation procedure is applicable if TMGI is used as MBS Session ID.

Figure 7.1.1.2-1: MBS Session Creation without PCC

Steps 1 to 6 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1.	AF sends Nnef_TMGI_Allocate Request (TMGI number) message to NEF/MBSF to request allocation of a TMGI(s) to identify new MBS session(s).

NOTE 1:	Depending on the network deployment and use case, MB-SMF may receive requests from AF directly, or via NEF, or via MBSF, or via NEF and MBSF.

2.	NEF checks authorization of AF.

NOTE 2:	NEF is not required if AF is in trusted domain.

3.	NEF/MBSF discovers and selects an MB-SMF using NRF or based on local configuration,

4.	NEF/MBSF sends an Nmbsmf_TMGI_Allocate Request (TMGI number) message to the MB-SMF.

5.	MB-SMF allocates TMGI(s) and returns the TMGI(s) to the NEF/MBSF via the Nmbsmf_TMGI_Allocate response (TMGI(s), expiration time).

6.	The NEF or MBSF responds to the AF by sending an Nnef_TMGI_Allocate Response (TMGI(s), expiration time).

7.	The AF may perform a Service Announcement towards UEs. The AF informs UEs about MBS Session information with MBS Session ID, e.g., TMGI, SSM, and possibly other information e.g., MBS service area, session description information, etc.

The MBS service area information can be Cell ID list, TAI list, geographical area information or civic address information. Amongst them, Cell ID list and TAI list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

The UE needs to be aware if the service is broadcast or multicast to decide if JOIN is to be performed.

Editor's note:	How to do service announcements requires SA WG4 /WG6 coordination.

8.	AF of content provider may provide description for an MBS session to NEF via a Nnef_MBSSession_Create request (MBS Session ID, service type, MBS session information, [MBS service area], [Any UE indication], [start and end time of the MBS session], [MBS session state], [ingress transport address request indication], [FSA ID(s)]). The AF may provide as MBS Session ID an SSM or, if step 1-6 has been executed before, the identifier that has been allocated for the MBS session (i.e., TMGI). The AF provides the service type (i.e. either multicast service or broadcast service) and MBS session information which may include service requirements and media information. The AF may provide Any UE indication (indicating whether a multicast MBS session is "open to any UEs"), start and end time of the MBS session and MBS session state (active/inactive). In addition, the AF request may also indicate whether the allocation of an ingress transport address is requested.

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

For broadcast communication, the AF may determine MBS FSA ID(s) for the broadcast session based on business agreements and include them in the description of the MBS session.

9	NEF/MBSF checks authorization of content provider.

10.	NEF/MBSF discovers MB-SMF candidates and selects MB-SMF as ingress control node, possibly based on MBS service area. If a TMGI is included in step 8, NEF/MBSF finds MB-SMF based on that TMGI.

11.	NEF/MBSF sends Nmbsmf_MBSSession_Create Request (MBS Session ID, service type, MBS session information, [MBS service area], [Any UE indication], [start and end time of the MBS session], [MBS session state], [ingress transport address request indication], [FSA ID(s)]) to MB-SMF, to request MB-SMF to reserve ingress resources for a MBS distribution session, The NEF/MBSF forwards all parameters it has received from the AF in step 8. If the MBSF decides to insert an MBSTF into the user plane for the MBS session, it also indicates that the allocation of an ingress transport address is requested even if this was not requested in step 8. The request also includes the Any UE indication if provided in step 8.

If a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF allocates a TMGI.

For broadcast communication, if no MBS FSA ID(s) have been received, the MB-SMF selects MBS FSA ID(s) for the broadcast session based on local configuration.

12.	If a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF updates its NF profile at the NRF with the serving MBS Session ID. If an MBS service area was received in step 11, the MB-SMF updates its NF profile at the NRF with that information.

NOTE 3:	If TMGI is used to represent an MBS Session, MB-SMF does not need to update NRF if the TMGI range(s) supported by an MB-SMF is already included in the MB-SMF profile when MB-SMF register itself into NRF.

13.	The MB-SMF derives the required QoS parameters locally under consideration of the MBS session information.

14	MB-SMF selects the MB-UPF. If the allocation of an ingress transport address was requested in step 11, the MB-SMF requests the MB-UPF to reserve user plane ingress resources. If multicast transport of the MBS data towards RAN nodes is to be used, the MB-SMF also request the MB-UPF to reserve for the outgoing data a tunnel endpoint and the related identifiers (source IP address, SSM and GTP Tunnel ID) and to forward data received at the user plane ingress resource using that tunnel endpoint.

If the allocation of an ingress transport address was not requested in step 11, the MB-SMF provides the SSM received as MBS session ID to the MB-UPF and requests the MB-UPF to join the corresponding multicast tree from the content provider. The MB-SMF may also defer the configuration to join the corresponding multicast tree e.g. based on information that the session is inactive, service requirements and MBS start/end time until receiving the first query for the MBS session as part of the establishment procedure in clause 7.2.1.3, or until receiving a request to activate the MBS session via the MBS Session Update procedure in clause 7.1.1.6.

15.	If requested, MB-UPF selects an ingress address (IP address and port) and a tunnel endpoint for the outgoing data and provides it to MB-SMF.

16.	MB-SMF indicates the possibly allocated ingress address to the NEF/MBSF. MB-SMF may include TMGI if it is allocated in step 11. For broadcast communication, the MB-SMF includes any MBS FSA ID(s) selected in step 11. It also indicates the success or failure of reserving transmission resources.

17.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.1 to request the allocation of resources to for the transmission of the broadcast session.

18.	[Optional] If the MBSF decides to use an MBSTF, the NEF/MBSF provides the ingress address received in step 16 towards the MBSTF as DL destination. If the allocation of an ingress transport address was requested in step 8, the MBSF requests the MBSTF to allocate the user plane ingress resources. If the allocation of an ingress transport address was not requested in step 8, the MBSF provides the SSM received as Multicast session ID in step 8 and requests the MBSTF to join the corresponding multicast tree from the content provider.

19.	[Conditional on step 19]	If requested, the MBSTF selects an ingress address (IP address and port) and provides it to NEF/MBSF.

20.	The NEF/MBSF-C indicates the possibly allocated ingress address and other parameters (e.g. TMGI) to the AF via an Nnef_MBSSession_Create response ([TMGI], [Allocated ingress address])). If MBS Session ID is not provided in step 8, or the MBS Session ID is SSM, the NEF/MBSF provides the allocated TMGI. If AF requested the allocation of an ingress transport address, the message also includes the allocated ingress address. For broadcast communication, the message also includes any MBS FSA ID(s) received in step 17.

21.	Same as step 7. The AF may also perform a service announcement at this stage.

22.	For multicast communication, depending on configuration UEs can join the MBS Session as specified in clause 7.2.1.

NOTE:	Steps 18-22 can be executed in parallel to step 17.

# * * * * Second change * * * *

#### 7.1.1.3	MBS Session Creation with PCC

Figure 7.1.1.3-1: MBS Session Creation with PCC

Steps 1 to 7 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1. to 9.:	Same as in Figure 7.1.1.2-1. Step 1 may include an application ID.

10. NEF/MBSF discovers the MB-PCF.

11. The NEF/MBSF sends an Npcf_MBSPolicy_Authorization_Create Request (MBS session ID, MBS session information, MBS service area) to the MB-PCF.

12. The MB-PCF determines whether the request is authorized and if the request is authorized, the MB-PCF derives the required QoS parameters based on the received MBS session information and determines whether this QoS is allowed. If the required QoS is allowed, the MB-PCF generates the PCC rules and the Policy information for the entire MBS session (see clause 6.10) and keeps them together with the MBS session ID.

13.	The MB-PCF sends an Npcf_MBSPolicy_Authorization_Create Response to the NEF/MBSF.

If the request is not authorized or the required QoS is not allowed, the MB-PCF indicates so in the response to the NEF/MBSF which in turn informs the AF about it (by sending the Nnef_MBSSession_Create Response) and ends this procedure.

14. NEF/MBSF discovers MB-SMF candidates and selects MB-SMF as ingress control node, possibly based on MBS service area. If a TMGI is included in step 8, NEF/MBSF finds MB-SMF based on that TMGI.

15. Same as step 11 in Figure 7.1.1.2-1 with the difference that the MBS session information is only present if the Nmbsmf_MBSSession_Create Request is directly sent by the AF (because the NEF/MBSF has already provided the MBS session information to the MB-PCF in step 11, otherwise).

16.	The MB-SMF sends an Npcf_MBSPolicyControl_Create Request (MBS session ID) for the MBS session towards the MB-PCF.

If the Nmbsmf_MBSSession_Create Request was directly received from the AF, the MB-SMF provides also the MBS session information and if available, the MBS service area information, to the MB-PCF.

17. The MB-PCF identifies the PCC rules and the Policy information for the entire MBS session (see clause 6.10) corresponding to the MBS session ID received from the MB-SMF.

If the MB-PCF does not have any information corresponding to the MBS session ID (which happens only if the AF interacts directly with the MB-SMF), the MB-PCF performs the same actions as described in step 12 using the MBS session information and if available, MBS service area information, received from the MB-SMF.

The MB-PCF responds with Npcf_MBSPolicyControl_Create Response (PCC rules, Policy information for the entire MBS session, see clause 6.10) with policies for the MBS Session ID.

NOTE 1:	If there is no service requirement, the MB-PCF may create the MBS policy according the operator's policy.

If the MB-PCF has received MBS session information from the MB-SMF in the request (which happens only if the AF interacts directly with the MB-SMF) and the request is not authorized or the required QoS is not allowed, the MB-PCF indicates so in the response to the MB-SMF which in turn informs the AF about it (by sending the Nmbsmf_MBSSession_Create Response) and ends this procedure.

18. Same as step 12 in Figure 7.1.1.2-1.

19.-27.	Same as steps 14-22 in Figure 7.1.1.2-1.

# * * * * Third change * * * *

#### 7.1.1.5	MBS Session Deletion with PCC

This procedure is used by the AF to release the MBS Session. This procedure may also include TMGI de-allocation. The procedures apply to both multicast and broadcast communications unless otherwise stated. This procedure releases the reserved resources in both 5GC and NG-RAN.

Figure 7.1.1.5-1: MBS Session Deletion with PCC

1.-3.	Same as in Figure 7.1.1.4-1.

4. 	NEF/MBSF sends an Npcf_MBSPolicyAuthorization_Delete Request (MBS Session ID) to the MB-PCF.

5. 	The MB-PCF sends an Npcf_MBSPolicyAuthorization_Delete Response to the NEF/MBSF.

6. 	Same as step 4 in Figure 7.1.1.4-1.

7.	The MB-SMF sends an Npcf_MBSPolicyControl_Delete Request (MBS Session ID) to the MB-PCF.

8. 	The MB-PCF deletes all information related to the received MBS Session ID and sends an Npcf_MBSPolicyControl_Delete Response to the MB-SMF.

9-18.	Same as steps 5-14 in Figure 7.1.1.4-1.

# * * * * Fourth change * * * *

#### 7.1.1.6	MBS Session Update without PCC

This procedure is used by the AF to update the MBS service area and/or MBS Session information. Updating MBS Session information may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s). The procedure applies to both multicast and broadcast communications unless otherwise stated.

For local multicast services and location dependent multicast services, the AF may perform a Service Announcement towards UEs to update the MBS service area before the MBS Session Update procedure is started or after the MBS Session Update procedure is completed.

Figure 7.1.1.6-1: MBS Session Update without PCC

1.	AF of content provider initiates MBS Session Update to a NEF/MBSF, e.g. to update MBS service area and/or update service requirement, update media information or to activate or deactivate an MBS session. AF may provide updated information for an MBS session (identified by MBS session ID) by sending an Nnef_MBSSession_Update Request (MBS Session ID, [MBS session information], [MBS service area], [MBS session state (active/inactive)]). MBS session information may include service requirements, and media information.

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

2.	NEF checks authorization of AF.

3.	NEF/MBSF sends Nmbsmf_MBSSession_Update Request to MB-SMF forwarding the updated information received from the AF in step 1.

4.	The MB-SMF derives any updated QoS parameters locally under consideration of the updated MBS session information. This may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s).

5.-6.	MB-SMF may need to update MB-UPF, e.g. if new MBS QoS Flow is to be created, or existing MBS QoS Flow is to be deleted.

7.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.2.5 (for service activation/deactivation), 7.2.6 (for QoS Flow updates and service area updates).

8.	If an MBS service area is being updated, the MB-SMF stores the new service area in its profile at the NRF.

9.	MB-SMF responds to the NEF/MBSF with a Nmbsmf_MBSSession_Update Response.

10.	NEF/MBSF responds to the AF with a Nnef_MBSSession_Update Response.

# * * * * Fifth change * * * *

#### 7.1.1.7	MBS Session Update with PCC

For local multicast services and location dependent multicast services, the AF may perform a Service Announcement towards UEs to update the MBS service area before the MBS Session Update procedure is started or after the MBS Session Update procedure is completed.

Figure 7.1.1.7-1: MBS Session Update with PCC

1.-2.	Same as in Figure 7.1.1.6-1.

3. 	NEF/MBSF sends an Npcf_MBSPolicy_Authorization_Update Request (MBS session ID, [MBS session information], [MBS service area]) to the MB-PCF forwarding the updated information received from the AF in step 1.

4. 	The MB-PCF determines whether the update is authorized and if the update is authorized, the MB-PCF derives the update for the QoS parameters based on the received MBS session information and determines whether this new QoS is allowed. If the new QoS is allowed, the MB-PCF updates the PCC rules and the Policy information for the entire MBS session (see clause 6.10) accordingly.

5. 	The MB-PCF sends an Npcf_MBSPolicy_Authorization_Update Response to the NEF/MBSF.

If the update is not authorized or the new QoS is not allowed, the MB-PCF indicates so in the response to the NEF/MBSF which in turn informs the AF about it (by sending the Nnef_MBSSession_Update Response) and ends this procedure.

6. 	The NEF/MBSF sends Nmbsmf_MBSSession_Update Request to the MB-SMF forwarding the updated information received from the AF in step 1.

Updated MBS session information is only present if the Nmbsmf_MBSSession_Update Request is directly sent by the AF (because the NEF/MBSF has already provided the updated MBS session information to the MB-PCF in step 3, otherwise)

7. 	The MB-SMF sends an Npcf_MBSPolicyControl_Update Request (MBS session ID) for the MBS session towards the MB-PCF.

If the Nmbsmf_MBSSession_Update Request was directly received from the AF, the MB-SMF provides also the updated MBS session information (if received) and the updated MBS service area information (if received), to the MB-PCF.

8. 	The MB-PCF identifies any updated PCC rules and any updated Policy information for the entire MBS session (see clause 6.10) corresponding to the MBS session ID received from the MB-SMF.

If the MB-PCF has received updated MBS session information from the MB-SMF (which happens only if the AF interacts directly with the MB-SMF), the MB-PCF performs the same actions as described in step 4 using the MBS session information and if available, MBS service area information, received from the MB-SMF.

The MB-PCF responds with Npcf_MBSPolicyControl_Update Response (updated PCC rules, updated Policy information for the entire MBS session, see clause 6.10) with updated policies for the MBS Session ID.

If the MB-PCF has received updated MBS session information from the MB-SMF in the request (which happens only if the AF interacts directly with the MB-SMF) and the update is not authorized or the updated QoS is not allowed, the MB-PCF indicates so in the response to the MB-SMF which in turn informs the AF about it (by sending the Nmbsmf_MBSSession_Update Response) and ends this procedure.

9.-14.		Same as steps 5-10 in Figure 7.1.1.6-1.

# * * * * End of changes * * * *