SA WG2 Meeting #150-e	S2-2202701

# 06 - 12 Apr 2022, 2022; Elbonia               	       (revision of S2-22xxxxx)

Source: 	China Telecom

Title: 	KI#4, New Sol: Support standardized and operator-specific traffic categories in URSP

Document for: 	Approval

Agenda Item: 	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: solutions for KI 4

1 Discussion

The SID FS_eUEPO in Key issue#4: Support standardized and operator-specific traffic categories in URSP, has following aspects will be considered:

-	 Whether the existing URSP design can be used to support the traffic category. If not, how to support traffic category in the traffic descriptor of a URSP rule

This paper proposes to leverage Application descriptors (short for ‘APP ID’) in URSP TD as specified in TS 23.503 [xx] clause 6.6.2 “UE Route Selection Policy information” for supporting Key issue#4.

2 Proposal

It is proposed to update 23.700-85 as follows

FIRST CHANGE

6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |  |  |  |  |  |
| Solution #X: Support standardized and operator-specific traffic categories in URSP |  |  |  | X |  |  |  |  |  |

NEXT CHANGE

6.X	Solution #X: Support standardized and operator-specific traffic categories in URSP

6.X.1	Description

In the current specification, application descriptors (short for ‘APP ID’) is defined as one TD in URSP rules. Application descriptors is used to identify the Application(s) that is(are) running on the UE's OS as specified in TS 23.503 clause 6.6.2 “UE Route Selection Policy information”.

To resolve the KI#4, application descriptors as one TD in URSP rules is to leverage for supporting standardized and operator-specific traffic categories in URSP. The special value of application descriptors can be defined or used for standardized and operator-specific traffic categories, and other value of application descriptor can be used for non-special application(s).

For example:

- Assuming that there are standardized and operator-specific traffic categories like “vedio”, “games”, “streaming”, , and “enterprise”;

- So the following application descriptors value can be defined or used for standardized and operator-specific traffic categories:

| Special value of application descriptors (short for ‘APP ID’) | standardized and operator-specific traffic categories |
| --- | --- |
| APP ID= aaaa | video |
| APP ID= bbbb | games |
| APP ID= cccc | streaming |
| APP ID= dddd | enterprise |
| …… | …… |

So the solution to reuse the application descriptors as one TD in URSP rules to define special value for standardized and operator-specific traffic categories in URSP has meet the Key issue#4.

6.X.2	Procedures

The solution reuses existing procedures.

6.X.3	Impacts on Existing Nodes and Functionality

None.

END OF CHANGES