3GPP TSG-SA WG2 Meeting #150E (e-meeting)	S2-2202704

# April 6 – 12, 2022, Elbonia													     (revision of S2-22xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.285 | CR | 0064 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on parameter for NR Tx Profile selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC_Ph2 |  |  |  |  |  | Date: |  |  | 2022-03-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Regarding Policy/parameters for NR PC5, description about NR Tx Profile is missing. Therefore, description about the policy/parameter for NR Tx Profile selection needs to be added under "4) Policy/parameters for NR PC5" in clause 4.4.1.1.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §4.4.1.1.2: - Add description about the policy/parameter for NR Tx Profile selection under "4) Policy/parameters for NR PC5". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description about the policy/parameter for NR Tx Profile selection is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.4.1.1.2	Policy/Parameter provisioning

The following information for V2X communications over PC5 reference point is provisioned to the UE:

# 1)	Authorization policy:

-	When the UE is "served by E-UTRAN":

-	PLMNs in which the UE is authorized to perform V2X communications over PC5 reference point.

-	When the UE is "not served by E-UTRAN":

-	Indicates whether the UE is authorized to perform V2X communications over PC5 reference point when "not served by E-UTRAN".

# 2)	Radio parameters for when the UE is "not served by E-UTRAN":

-	Includes the radio parameters with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed. These radio parameters (e.g. frequency bands) are defined in TS 36.331 [9]. The UE uses the radio parameters to perform V2X communications over PC5 reference point when "not served by E-UTRAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 1:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

3)	Policy/parameters:

-	The mapping of Destination Layer-2 ID(s) and the V2X services, e.g. PSIDs, ITS-AIDs or AIDs of the V2X application.

NOTE 2:	PLMN operators coordinate to make sure Destination Layer-2 ID(s) for different V2X services are configured in a consistent manner.

NOTE 3:	To pre-configure a UE with the provisioning parameters, at least the "not served by E-UTRAN" parameters of 1) and 2), and the parameters of 3) need to be included.

-	The mapping of ProSe Per-Packet Priority and packet delay budget for V2X communication (autonomous resources selection mode).

-	The list of V2X services, e.g. PSIDs, ITS-AIDs or AIDs of the V2X applications, with Geographical Area(s) that require privacy support.

-	The mapping of service types (e.g. PSIDs, ITS-AIDs or AIDs) to V2X frequencies (see TS 36.300 [10] for further information) with Geographical Area(s).

-	The mapping of service types (e.g. PSIDs, ITS-AIDs or AIDs) to Tx Profiles (see TS 36.300 [10] for further information).

-	The list of V2X services, e.g. PSIDs, ITS-AIDs or AIDs of the V2X applications, allowed to use a specific PPPR value.

4)	Policy/parameters for NR PC5:

-	Same as specified in TS 23.287 [30] clause 5.1.2.1 item 6) Policy/parameters when NR PC5 is selected, and the policy/parameter for NR Tx Profile selection as specified in TS 23.287 [30] clause 5.1.2.1 item 3) Policy/parameters for PC5 RAT selection and for PC5 Tx Profile selection.

Alternatively, the above parameters in 2) and 3) may be configured on the UE through the V1 reference point from the V2X Application Server. The V2X Control Function may also obtain these parameters through the V2 reference point from the V2X Application Server.

The parameters provisioned by the V2X Control Function take precedence over the configured parameters on the UE.

NOTE 4:	The V2 reference point procedure is not specified in this Release.

* * * * End of Changes * * * *