3GPP TSG-SA WG2 Meeting #150E	S2-2202712

# E-Meeting, 6th – 12th April 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0110 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MBS security function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The TS23.247 just refers to SA3 specification for MBS security function. But, is is not clear when their security mechanism applies and which entities have the functionality. Per TS 33.501 the defined MBS security is applied to only when the MBSF/MBSTF are used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that MBS security is implemented in MBSF/MBSTF and so MBS security is applies to only configuration option 2 and 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Which entities are used or which configuration scenarios are applied for 5MBS is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.8, 5.3.2.11, 5.3.2.12, 6.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******Start of 1st change*******

6.13	MBS Security function

Security function may be needed to protect MBS related signalling/data. Detailed descriptions of security requirements, procedures and handling for 5G Multicast/Broadcast Service (MBS) are provided in TS 33.501 [20].

MBS security function is implemented in the MBSF/MBSTF so that it may apply only when MBSF/MBSTF are used (i.e. Configuration option 2 and 3). For configuration option 1 how to support MBS security is out of 3GPP scope.

*******Start of 2nd change*******

5.3.2.8	UE

In addition to the functions defined in TS 23.501 [5], the UE may perform the following functions to support MBS:

-	Reception of multicast data using PTM/PTP.

-	Reception of broadcast data using PTM.

-	Handling of incoming MBS QoS flows.

-	Support of signalling for joining and leaving multicast MBS session.

-	MBS resource management support at AS layer.

-	Reception of notification in CM-IDLE state and CM-CONNECTED with RRC Inactive state for multicast data transmission.

NOTE :	UE functionality for MBS security is provided in TS 33.501 [20].

*******Start of 3rd change*******

5.3.2.11	MBSF

The MBSF performs the following functions to support MBS:

-	Service level functionality to support MBS, and interworking with LTE MBMS

-	Interacting with AF and MB-SMF for MBS session operations, determination of transport parameters, and session transport.

-	Selection of MB-SMF to serve an MBS Session.

-	Controlling MBSTF if the MBSTF is used.

-	Determination of destination IP multicast address for the MBS session if IP multicast address is sourced by MBSTF.

NOTE 1:	MBSF functionality related to service and MBS data handling (e.g. encoding) is to be determined with SA WG4.

NOTE 2:	MBSF functionality for MBS security is provided in TS 33.501 [20].

5.3.2.12	MBSTF

The MBSTF performs the following functions to support MBS if deployed:

-	Media anchor for MBS data traffic if needed.

-	Sourcing of IP Multicast if needed.

-	Generic packet transport functionalities available to any IP multicast enabled application such as framing, multiple flows, packet FEC (encoding).

-	Multicast/broadcast delivery of input files as objects or object flows.

NOTE 1:	MBSTF functionality related to MBS data handling (e.g. encoding) is to be determined with SA WG4.

NOTE 2:	MBSTF functionality for MBS security is provided in TS 33.501 [20].

*******End of changes*******