3GPP TSG-SA2 Meeting #150-E	S2-2202721

# Elbonia, 6 April 2022 – 14 April 2022									    	(Revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3452 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NW-TT Identification and Clock Drift Report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIOT |  |  |  |  |  | Date: |  |  | 2022-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If there are multiple NW-TTs configured in the UPF, each NW-TT is associated with a network instance, in such case the NW-TT can be identified by the network instance. If the SMF asks the UPF to provide the clock drift between a TSN time domain number and 5GS clock, the UPF does not know which NW-TT connects to the TSN time domain and UPF may ask all the NW-TTs one by one to check which NW-TT connects to the TSN time domain.If the the network instance is provided, the UPF can directly ask the NW-TT associatd with the network instance to provide the clock drift information. If the SMF wants to get all clock drifts from a NW-TT associated network instance, but the current results is that the SMF gets all the clock drifts from all the NW-TTs in the UPF. If the SMF does want to get all clock drifts from NW-TTs in the UPF, the SMF does not provide the network instance and TSN time domain number in the provision request on the clock drift. |  |  |  |  |  |  |  |  |
| Summary of change: |  | N4 Association Setup procedure can be used to request the UPF to measure and report the clock drift between the external time and 5GS time for one or more external time domains by provisioning External Clock Drift Report and providing the corresponding Time Domain number(s) and network instance (if available ) as specified in TS 29.244 [69]. If there are multiple NW-TTs configured in the UPF as described in clause 4.4.8.3 of TS 23.501 [2], the SMF includes the network instance to request the associated NW-TT with the network instance in the UPF. The SMF may include network instance and omit the Time domain number in the request, in this case the UPF shall report the clock drift for all Time domains connected to the NW-TT associated with the network instance; The SMF may omit the Time domain number and network instance in the request; in this case the UPF shall report the clock drift for all Time domains the UPF is connected to. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NW-TT in the UPF cannot be identified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST CHANGE

#### 4.4.3.1	N4 Association Setup Procedure

The N4 Association Setup procedure is used to setup an N4 association between the SMF and the UPF, to enable the SMF to use the resources of the UPF subsequently to establish N4 Sessions. The SMF and UPF may exchange the supported functionalities on each side during these procedures.

The setup of an N4 association is initiated by the SMF. SMF and UPF may additionally support an N4 association initiated by UPF.

The SMF should only establish an N4 association with a UPF that supports F-TEID allocation at the UPF.

The SMF initiates the N4 Association Setup procedure to request to setup an N4 association towards a UPF prior to establishing a first N4 session on this UPF.

When receiving an N4 Association Setup Request, the UPF shall send an N4 Association Setup Response.

N4 Association Setup procedure can be used to request the UPF to measure and report the clock drift between the external time and 5GS time for one or more external time domains by provisioning External Clock Drift Report and providing the corresponding Time Domain number(s) and network instance (if available ) as specified in TS 29.244 [69]. If there are multiple NW-TTs configured in the UPF as described in clause 4.4.8.3 of TS 23.501 [2], the SMF includes the network instance to request the associated NW-TT with the network instance in the UPF. The SMF may include network instance and omit the Time domain number in the request, in this case the UPF shall report the clock drift for all Time domains connected to the NW-TT associated with the network instance; The SMF may omit the Time domain number and network instance in the request; in this case the UPF shall report the clock drift for all Time domains the UPF is connected to.

Figure 4.4.3.1-1: N4 association setup procedure initiated by SMF

The UPF may initiate the N4 Association Setup procedure to request to setup an N4 association towards a SMF prior to establishing a first N4 session on this UPF.

When receiving an N4 Association Setup Request, the SMF shall send an N4 Association Setup Response.

Figure 4.4.3.1-2: N4 association setup procedure initiated by UPF

END OF CHANGES