SA WG2 Meeting #150e	S2-2202766

# April 6th – 12th, 2022; Elbonia               		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	23.700-41: Solution for KI#1; Support of Network Slice Service continuity using SSC mode 3

Document for: 	Approval

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_ph3/ Rel-18

Abstract of the contribution:

# 1 Discussion

This pCR proposes a solution to KI#1 in TR 23.700-41, regarding the support of service continuity

# 2 Proposal

It is proposed to update 23.700-41 as follows

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| Solution X: Support of Network Slice Service continuity using SSC mode 3 | x |  |  |  |  |  |

NEXT CHANGE (2) ALL text is new

## 6.X	Solution #X: Support of Network Slice Service continuity using SSC mode 3

## 6.X.1	Description

This solution considers a UE which is in CM-CONNECTED mode and arrives at an area where it needs to be handed over to a cell outside the current RA where a PDU session in a certain slice cannot continue as the slice is no longer available for any of the reasons documented in the KI#1.

This solution relies on using SSC mode 3 in a network-controlled manner. the next clause provides the necessary details.

### 6.X.2	Procedures

The call flow below in figure 6.x.2-1 proposes using SSC mode 3 to provide the service continuity solution. A single NG-RAN node may also be involved, two NG-RAN nodes are shown for generality and only the Xn Handover case is shown but this can apply also to NG Handover.

Source NG-RAN node 1

SMF

2. UE has ongoing PDU session 1 of slice 10

UE

Target NG-RAN node 2

3. HO request (pdu session 1)

AMF

Handover command (PDU session 1 temporary accepted)

7. NAS PDU Session Modification Command (end slice 10, new slice 11)

Handover complete

8. Establish resources for PDU session 2 on slice 11

9. Release pdu session 1 of slice 10

SSC mode 3 Timer expiry

10. UE Configuration Update (new Allowed NSSAI= slice 11 only)

4. PSR (end slice 10, new slice 11)

5. update (end slice 10, new slice 11)

Register Accept (Allowed NSSAI (slices 10, 11)

6.Register Request

1: UE Allowed NSSAI (slices 10, 11)

6.x.2-1: service continuity upon slice change

0.	NG-RAN nodes have been configured with slice re-mapping of slice 10 to 11 due to any cause.

1.	 AMF has sent the UE Allowed NSSAI to the serving NG-RAN node and the UE per existing procedures

2. 	UE has ongoing PDU session 1 of slice 10.

3. 	Source NG-RAN triggers Handover to target NG-RAN. The target NG-RAN node 2 informs during the HO procedure the source NG-RAN node 1 that it accepts the PDU session 1 of slice 10 temporarily due to slice re-mapping action which it has been configured with for slice 10. It also indicates the new slice 11 for the PDU session.

4. 	At handover completion, the target NG-RAN indicates to AMF in Path Switch Request that PDU session 1 of slice 10 needs to be terminated and a new PDU session is to be setup with slice 11.

5. 	AMF sends an Update message to SMF indicating end of PDU session 1 of slice 10 and remapping to slice 11. The SMF triggers step 7.

6. 	The UE performs the post-handover registration (as Source and Target NG RAN nodes have different slice support, they don’t belong to the same registration area for the UE) requesting Slice 10 and slice 11 as indicated in the handover command. Because the AMF has received (end slice 10) at step 4, the AMF still includes the slice 10 in the Allowed NSSAI towards the UE at this step (the slice is indeed still temporarily available until it receives from SMF notification of the final release of PDU session 1 of slice 10 at step 9).

7. 	In reaction to step 5, the SMF triggers towards the UE the NAS PDU Session Modification Command (Cause, PCO (PDU Session Address Lifetime value), end slice 10, new slice 11) to invoke SSC mode 3. The (end slice 10, new 11) may be included towards the UE to prompt the UE to setup the new PDU session 2 with slice 11.

8.	The UE triggers the establishment of PDU session 2 with slice 11 according to SSC mode 3 procedure as per existing procedures described in 23.502 § 4.3.2.2.1.

9. 	At the expiry of SSC mode 3 timer, the SMF triggers the release of the PDU session 1 of slice 10 according to SSC mode 3 procedures (existing procedures described in 23.502 § 4.3.2.2.1).

10.	Upon notification from the SMF that PDU session of no longer supported slice 10 has been released, the AMF sends a final the UCU (UE Configuration Update) message in order to update the Allowed NSSAI towards the NG-RAN and the UE. In this example, the new Allowed NSSAI is slice 11.

### 6.X.3	Impacts on services, entities and interfaces

The solution has the following impacts:

NG-RAN node:

-	configuration to perform slice remapping for certain slices and related behaviour as per above procedure

AMF

-	temporarily allow slices while SSC3 completes for the PDU session and do UE configuration update to provide the final allowed NSSAI when the SSC mode 3 new PDU session is up and running and old PDU session leg released.

UE

-	trigger SSC mode 3 when the PDU session is temporarily accepted in the new slice as specified in the handover command.

-	perform registration procedure by taking into account the indicated slice as replacement of the ongoing slice.

-	performs SSC mode 3 with the slice indicated to replace the ongoing slice.

AMF: noneEND OF CHANGES