3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202814

April 6th – 12th, 2022, Elbonia	(revision of S2-220xxxx)

Source:	China Mobile, China Southern Power Grid

Title:	New Key Issue: Efficient N19 tunnel management for VN group communication

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing a Key Issue for FS_GMEC based on the Work Task 2.1.

1. Introduction/Discussion

The Work Task 2.1 of FS_GMEC is as following:

-	Work Task 2.1: Support group communication for a 5G VN which supports multiple SMFs, including support of SMF redundancy for reliability of the 5G VN group communication.

When multiple SMFs are involved to serve a 5GVN, multiple UPFs controlled by these different SMFs may be involved to enable communication of the 5GVN. In this case, how to establish the N19 tunnels between these UPFs (different topology may be established) is not clear. After that, how to update the configuration on these UPFs due to UE mobility also needs further study.

In Rel-16, if the 5G VN group members’ PDU sessions are served by different PSA UPFs and N19 based forwarding is applied, the SMF shall create a group-level N4 session with each involved UPF to enable N19 based forwarding. However, with the increase of involved UPFs, especial when the 5G VN group is served by multiple SMFs, the number of N19 tunnels and corresponding group-level N4 sessions maintained by SMF(s) will increase significantly.

If two UPF involved, one N19 tunnel and two group-level N4 sessions will be managed by the SMF.

If three UPF involved, up to three N19 tunnel and six group-level N4 sessions will be managed by the SMF.

If four UPF involved, up to six N19 tunnel and twelve group-level N4 sessions will be managed by the SMF.

As illustrated above, if N UPF involved in the 5G VN group, then up to N*(N-1)/2 N19 tunnel and N*(N-1) group-level N4 sessions will be managed by the SMF. For each time a new and Nth UPF involved, the SMF shall establish up to N-1 N19 tunnel and create/manage 2*(N-1) group-level N4 sessions. Obviously, when the involved UPF increased, the SMF load will be increased significantly and the maintenance for VN group routing and forwarding rules will be quite complex. There is a high probability that more UPFs will be involved when multiple SMFs are required for the VN group communication.

So, it is necessary to study the efficient solutions to solve the N19 tunnel management issue mentioned above along with the Key issue #4.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.1.0.

# * * * * First change * * * *

5.X	Key Issue #X: Efficient N19 tunnel management for VN group communication

5.x.1	Description

In Rel-16, if the 5G VN group members’ PDU sessions are served by different PSA UPFs and N19 based forwarding is applied, the SMF shall create a group-level N4 session with each involved UPF to enable N19 based forwarding. However, with the increase of involved UPFs, especial when the 5G VN group is served by multiple SMFs, the number of N19 tunnels and corresponding group-level N4 sessions maintained by SMF(s) will increase significantly.

If two UPF involved, one N19 tunnel and two group-level N4 sessions will be managed by the SMF.

If three UPF involved, up to three N19 tunnel and six group-level N4 sessions will be managed by the SMF.

If four UPF involved, up to six N19 tunnel and twelve group-level N4 sessions will be managed by the SMF.

As illustrated above, if N UPF involved in the 5G VN group, then up to N*(N-1)/2 N19 tunnel and N*(N-1) group-level N4 sessions will be managed by the SMF. For each time a new and Nth UPF involved, the SMF shall establish up to N-1 N19 tunnel and create/manage 2*(N-1) group-level N4 sessions. Obviously, when the involved UPF increased, the SMF load will be increased significantly and the maintenance for VN group routing and forwarding rules will be quite complex. There is a high probability that more UPFs will be involved when multiple SMFs are required for the VN group communication.

For this Key Issue, it will be studied how to increase the N19 tunnel management efficiency and reduce the SMF management overhead, including:

-	Which architectural enhancements, if any, are needed to reduce the SMF management overhead for N19 tunnel management;

-	What interface/protocol enhancements, if any, are needed to increase the N19 tunnel management efficiency.

# * * * * End of changes * * * *