3GPP TSG-WG SA2 Meeting #150E meeting	S2-2202861

Elbonia, April 06 – 12, 2022	(revision of S2-220xxxx)

Source:	Lenovo, Motorola Mobility

Title:	Solution for WT#2: providing UE with partially available S-NSSAI per TA

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3/Rel-18

Abstract of the contribution: This solution proposes to provide to the UE with partially available S-NSSAI within RA.

# 1. Discussion

The present solution proposes to provide to the UE with "partially available S-NSSAI within RA" information. The format of the "partially available S-NSSAI within RA" information depend on the following conditions:

-	If the UE requests to register with an S-NSSAI, which is not available in the current TA but it is available in other TAs of the RA, the AMF includes the S-NSSAI in the list of rejected S-NSSAIs with a reject cause partially not available and associated list of TAs where the S-NSSAI is not available.

-	If the UE requests to register with an S-NSSAI, which is available in the current TA but not in all TAs of the RA, the AMF includes the S-NSSAI in "partially allowed S-NSSAI" and associates the S-NSSAI with a list of TAs where the S-NSSAI is available.

# 2. Proposal

It is proposed to add the following new key issue to 3GPP TR 23.700-41.

* * * First Change * * * *

## 6.X	Solution #X: Providing UE with partially available S-NSSAI within RA

### 6.X.1	Introduction

This solution targets KI#Y on "whether and how to initiate a registration for a rejected S-NSSAI that was rejected in a first TA of the RA but may be available in another TA of the RA".

### 6.X.2	Functional Description

The present solution is based on the following principles:

1.	The UE may indicate to the AMF the support of the feature of partial S-NSSAI availability within RA.

2.	The AMF/NSSF know which S-NSSAIs are supported in the TAs included in the AMF service area. The AMF/NSSF may determine, e.g. based on the UE capabilities, the UE current TA (from which the UE sends the Registration Request) and the S-NSSAIs of the requested NSSAI, to provide the UE with "partially available S-NSSAI within RA" information. The "partially available S-NSSAI within RA" information may be in the form of:

a.	partially rejected S-NSSAIs: if the UE requests to register with an S-NSSAI, which is not available in the current TA but it is available in other TAs of the RA, the AMF includes the S-NSSAI in the list of rejected S-NSSAIs with a reject cause partially rejected and associated with a list of TAs where the S-NSSAI is not available.

b.	partially allowed S-NSSAIs: if the UE requests to register with an S-NSSAI, which is available in the current TA but not in all TAs of the RA, the AMF includes the S-NSSAI in partially allowed S-NSSAI and associates the S-NSSAI with a list of TAs where the S-NSSAI is available.

3.	The UE stores the partially rejected NSSAI or the partially allowed NSSAI (e.g. as described in point 2a and 2b). The UE NAS requests the UE AS layer to indicate when the UE enters a cell which is part of a TA associated with partially rejected S-NSSAI or partially allowed S-NSSAI. The following applies:

a.	When an S-NSSAI is partially rejected and the UE enters a TA where the S-NSSAI is available, the UE (based on internal prioritization and application needs) may determine to request registration with the S-NSSAI.

b.	When an S-NSSAI is partially allowed and the UE enters a TA where the S-NSSAI is unavailable and

1)	if there is established PDU Session to this S-NSSAI, the UE initiates a procedure to suspend the user plane resources of the PDU Session.

2)	if there is no PDU Session established, the UE omits the PDU Session establishment request to this S-NSSAI as long as the UE is located in TA where the S-NSSAI is unavailable.

### 6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

UE impacts:

-	Receive and store the partially rejected NSSAI or the partially allowed NSSAI and the associated list of TA where the S-NSSAI is available/unavailable.

-	Track the current location in TA where the S-NSSAI is partially rejected or partially allowed.

AMF/NSSF impacts:

-	Ability to determine whether the UE should be provided with partially rejected or partially allowed S-NSSAI and the corresponding list of TAs where the S-NSSAI is available/unavailable in the RA.