SA WG2 Meeting #S2-145E	S2-2202888

06 – 12 April , 2022, E-meeting

Source:	Xiaomi

Title:	FS_eNPN_Ph2: architecture assumption

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_ph2/Rel-18

Abstract of the contribution: This paper proposes an architecture assumption that, the localized service is available in the hosting network, since it is out scope of 3GPP to establish the temporary relationship between Hosting network operator and localized service provider.

Discussion

Today, how the service provider to make its services available in one PLMN, is out scope of 3GPP, whether needs a service agreement or not is business operation, no impact on 3GPP. So does the relation between Hosting operation and localized service provider. There are some existing ways to make the localized service available in the Hosting network, e.g., Localize Service APP installed in UE.

Proposal: The localized service is available in the hosting network, whether needs the temporary relationship between hosting network operator and localized service provider is out scope of 3GPP.

Proposal

It is proposed to add the following architecture assumption to the TR 23.700-08 "Study on enhanced support of Non-Public Networks; Phase 2"

*** Start of changes ***

4	Architectural Assumptions and Requirements

Editor's note:	This clause will list general architectural assumptions and principles for this study.

-	Solutions shall build on the 5G System architectural principles as in TS 23.501 [3], including flexibility and modularity for newly introduced functionalities.

-	Functionality to enable regulatory services like emergency services are assumed to re-use existing architecture mechanisms with no or limited impact.

NOTE:	Any impacts to PWS or LI would be handled by other responsible WGs i.e. CT1 and SA3-LI respectively.

-	Hosting network can be NPN, i.e. SNPN, or PNI-NPN.

-	Home network can be NPN or PLMN.

-	The term "home network" does not imply a roaming relationship.

-	Only subscribers of a public network can roam into a PLMN.

-	The localized service is available in the hosting network, whether needs the temporary relationship between hosting network operator and localized service provider is out scope of 3GPP.

*** End of changes ***