SA WG2 Meeting #S2-145E	S2-2202892

06 – 12 April , 2022, E-meeting

Source:	Xiaomi

Title:	architecture Assumption

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	5G_PIN /Rel-18

Abstract of the contribution: This paper proposes to a new architecture assumption that when a PIN is created at the same time the PEMC is created as well.

Discussion

Observation 2: in TR 23.700-88, PEMC is defined to manage PIN, e.g., to add PEGC, PINE into PIN. It means when the PIN is created, the PEMC should be also ready for this PIN at same time. If there is no PEMC when a PIN is created, then there is no way to add other PINE, PEMC, PEGC into the PIN.

Key Issue #3: Management of PIN and PIN Elements

5.3.1		Description

This key issue intends to support the management of the PIN, including the management of different types of PIN Elements and the configuration of the PIN. Both the network operator and authorized 3rd party, i.e. PIN Element with Management Capability (PEMC) could create and configure the PIN and its elements.

After a PIN has been created, PEMC can add a PEGC into the PIN, or remove a PEGC from the PIN, as well as add a PIN Element into the PIN and associate it to some PEGCs that have already been added into the PIN, or remove a PIN Element from the PIN.

Conclusion: when a PIN is created, the PEMC is also created for this PIN at same time.

Proposal

It is proposed to update the following changes for the definition of PIN in the TR 23.700-88.

*** Start of changes ***

## 4.2	Architectural Assumptions

This study has following architectural assumptions:

-	Only a 3GPP UE can act as PEGC and/or PEMC.

-	There are one or more PEGCs in a PIN.

-	There are one or more PEMCs in a PIN.

-	The PIN Elements assumes to use non-3GPP access (e.g. WIFI, Bluetooth) for direct communication, the PEMC can use 5G ProSe Direct Communication for direct communication with PEGC.

-  At least there is a PEMC when the PIN is created

NOTE:	In this release the 5G-RG is considered outside the scope of the study and consequently not part of PIN.

Editor’s note: It is FFS whether data traffic of PINE over control plane is in scope of this study.

*** End of changes ***