SA WG2 Meeting #150e	S2-2202904

# Elbonia, Apr 6-12, 2022               			            (revision of S2-220xxxx)

Source: 	Apple

Title: 	New Sol for KI#X: Slice-specific implicit deactivation timers

Document for: 	Approval

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes a solution for Key Issue #X – Improved Network control of the UE behaviour control in TR 23.700-41.

# 1 Discussion

S2-22xxxxx includes the following Key Issue proposal: Improved Network control of the UE behaviour control:

"There is currently no standardized means for the operator to set a policy for deregistration of a network slice or tear down of a PDU session (e.g. the operator cannot control the time when a PDU session is released after it is last used by an application, nor can the operator decide when to deregister a UE from a network slice after there are no more PDU sessions established over it)."

This solution proposes a slice-specific implicit registration deactivation timer for registration control and a slice-specific implicit PDU session deactivation timer for PDU session control.

# 2 Proposal

It is proposed to update TR 23.700-41 as follows:

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | X |
| X |  |  |  |  | X |

NEXT CHANGE

## 6.X	Solution #X: Slice-specific implicit deactivation timers

### 6.X.1	Functional Description

#### 6.x.1.1	Registration control

During registration procedure, the UE indicates support for slice-specific implicit registration deactivation timer capability in 5GMM Core Network Capability. On receiving the Registration Request from the UE with this capability, AMF chooses a value for slice-specific implicit registration deactivation timer based on subscription and/or current load on the S-NSSAI and sets the Allowed NSSAI list along with slice specific implicit registration deactivation timer duration values for those slices that are high in demand or resource constrained. Based on subscription, different UEs may get assigned different values for the timer (e.g., premium subscribers can get assigned a longer value).

Both the UE and AMF start the slice-specific implicit registration deactivation timer when there are no PDU sessions with active user plane resources on the slice that is associated with the implicit timer.

When the UE establishes a PDU session with active user plane resources or activates a user plane connection for an established PDU Session, the slice-specific implicit registration deactivation timer is stopped.

When the timer expires at UE and AMF, both UE and AMF perform Implicit Deregistration procedure locally for the slice without any explicit deregistration procedure.

If necessary, AMF can adjust the slice-specific implicit registration deactivation timer duration using the UE Configuration Update procedure (clause 4.2.4 of TS 23.502 [x]) depending on the network resource usage/load balance.

#### 6.x.1.1	PDU session control

During PDU session establishment procedure, the UE indicates support for slice-specific implicit PDU session deactivation timer capability in 5GSM Core Network Capability. On receiving the PDU Session Establishment Request from the UE with this capability, the SMF chooses a value for slice-specific implicit PDU session deactivation timer based on subscription and/or current load on the S-NSSAI and sets the slice specific implicit PDU session deactivation timer duration values for those slices that are high in demand or resource constrained. Based on subscription, different UEs may get assigned different values for the timer (e.g., premium subscribers can get assigned a longer value).

Both the UE and SMF start the slice-specific implicit PDU session deactivation timer when the user plane connection is deactivated for the PDU session.

When the UE activates a user plane connection for the PDU Session, the slice-specific implicit PDU session deactivation timer is stopped.

When the timer expires at UE and SMF, both UE and SMF locally release the PDU session without any explicit PDU session release procedure.

If necessary, SMF can adjust the slice-specific implicit PDU session deactivation timer duration via network requested PDU Session Modification procedure (clause 4.3.3.2 of TS 23.502 [x]) depending on the network resource usage/load balance.

### 6.X.2	Procedures

The solution relies on existing procedures defined in TS 23.502 [x].

### 6.X.3	Impacts on Existing Nodes and Functionality

UE:

- Supports slice-specific implicit registration deactivation timer.

- Supports slice-specific implicit PDU session deactivation timer.

AMF:

- Supports slice-specific implicit registration deactivation timer, sets value via Registration procedure, adjusts value via UE Configuration Update procedure.

SMF:

- Supports slice-specific implicit PDU session deactivation timer, sets value via PDU session establishment procedure, adjusts value via network requested PDU Session Modification procedure.

END OF CHANGES