3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202928

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	MBS-based UAV remote ID broadcast

Document for:	Approval

Agenda Item:	9.5

Work Item / Release:	FS_UAS_Ph2 / Rel-18

Abstract: this is a new solution proposal on MBS-based UAV remote ID broadcast.

- Introduction

This is a new solution to address key issue 2.

- Discussion

Rel-17 defines MBS feature, which is a network-controlled mechanism for broadcasting. In case of UAV ID broadcast, MBS-based solution is proposed in this paper.

With the solution, network can be aware of the UAV status, not only the connection status, but also the flight status. It is helpful to the operator to manage the UAV service and coordinate with the USS.

3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-58.

# * * * * First change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |  |  |  |
| #X |  | X |  |  |  |  |  |

# * * * * Second change (all texts new)* * * *

6.X	Solution #X: MBS-based UAV remote ID broadcast

6.X.1	Key Issue mapping

This solution address key issue#2: Support of Broadcast Remote ID

6.X.2	Description

When UAV UE registers to the network, based on rel-17 conclusion, the AMF is aware of the UAV subscription, and interacts with the USS for authentication an authorization.

Overview of the solution:

- USS/UTM knows the UAV location based on UAV tracking;

- USS/UTM to broadcast remote identification via MBS system in specific area;

- The broadcast session establishment may take place by the USS/UTM(server) at any time, or triggered by a UAV registration (after UUAA procedure)

6.X.3	Procedures

Figure 6.x.3-1 MBS-based UAV remote ID broadcast

Step 0 broadcast session is established using mechanism defined in TS 23.247.

Step 1 UAV UE registers to the network, and establishes user plane connection with USS/UTM (server).

Step 2 UAV UE reports its remote ID to the server, via user plane.

Step 3 if the broadcast session is not yet established, triggered by step 2, USS/UTM requests the 5G network to establish a broadcast session using mechanism defined in TS 23.247.

After the broadcast session is established, USS/UTM(server) starts to broadcast the remote ID of UAV.

6.X.4	Impacts on Services, Entities, and Interfaces

Editor's note: This clause captures impacts on existing 3GPP services, entities, and interfaces.

# * * * * End of changes * * * *