3GPP TSG-SA WG2 Meeting #150E (e-meeting)	S2-2202971

6 – 12 April 2022, Elbonia	(revision of)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0227 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Periodic and Triggered 5GC-MT-LR Procedure in RRC INACTIVE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph2 |  |  |  |  |  | Date: |  |  | 2022-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN2 has sent an LS to SA2 in S2-2201948 / R2-2203949 requesting support for Low Power Periodic and Triggered 5GC-MT-LR Procedures with SDT for a UE in RRC INACTIVE state. The LS states the following. RAN2 has technically endorsed the attached "Low Power Periodic and Triggered 5GC-MT-LR Procedures with SDT" for DL-only and RAT-Independent positioning, for UL-only positioning, and for UL+DL positioning. These procedures are analogous to the "Low Power Periodic and Triggered 5GC-MT-LR Procedures" specified in clause 6.7 of TS 23.273 but making use of the NR Small Data Transmission (SDT) feature in TS 38.300 in RRC_INACTIVE state instead of the LTE EDT feature in RRC_IDLE state in TS 36.300 currently used in clause 6.7 of TS 23.273. RAN2 believes that these procedures may be best captured in TS 23.273 as an alternative to Figure 6.7.1-1 when the Event Reports can be sent using NR SDT. Accordingly, RAN2 would like to ask SA2 whether support can be provided for the attached procedures in TS 23.273 and, if so, to add support to TS 23.273 as part of Release 17. RAN2 would also like to inform SA2 about the following agreements made: Agreements: Exposure of the RRC state of the UE to the LPP layer of the UE for RRC_INACTIVE UL and DL positioning will not be specified. This does not exclude cross-layer behaviour in implementations. The RRC state of the UE is not exposed to the LMF for INACTIVE UL and DL positioning. RAN2 will not make additional effort to make the gNB aware of when to transit the UE to RRC_INACTIVE (left to gNB implementation and RAN3 solution).. A new procedure is needed in TS 23.273 to support each of the three types of UE positioning included in the RAN2 LS. |
| Summary of change: |  | A single combined procedure is added in TS 23.273 to support UL positioning, DL positioning and UL+DL positioning. While 3 separate procedures might have been added instead, the significant overlap between the 3 procedures would have resulted in significantly more change to 23.273. In addition, there would be a risk of inconsistency between 3 separate procedures regarding common aspects and the differences between the procedures at the UE and LMF would have been more difficult to show. Detailed RAN specific aspects of the procedures in the LS (e.g. concerning precise use of RRC, NRPPa and LPP) have been converted to a description consistent with the scope of 23.273 but without omitting important high level requirements such as use of SDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may not be possible to support positioning of a UE in RRC INACTIVE state in a power efficient manner as defined by RAN2 such that the UE remains in RRC INACTIVE state and does not transition into RRC CONNECTED state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.7.1, 6.7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | An alternative equivalent CR to this one is provided in S2-2202973 in which 3 separate procedures are used instead of a single combined procedure. Only one of these CRs should be approved. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

APN	Access Point Name

APN-NI	APN Network Identifier

EDT	Early Data Transmission

E-SMLC	Evolved Serving Mobile Location Centre

GMLC	Gateway Mobile Location Centre

HGMLC	Home GMLC

LCS	LoCation Services

LDR	Location Deferred Request

LMF	Location Management Function

LPI	LCS Privacy Indicator

LRF	Location Retrieval Function

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

NI-LR	Network Induced Location Request

PMD	Pseudonym mediation device functionality

POI	Privacy Override Indicator

SDT	Small Data Transmission

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TWAP	Trusted WLAN Access Point

VGMLC	Visited GMLC

**** Next Change ****

### 6.7.1	Event Reporting with no change of LMF

Figure 6.7.1-1 summarizes the initiation and reporting of location events for a deferred 5GC-MT-LR procedure for Periodic or Triggered Location Events using low power event reporting. This procedure is only applicable to a UE with E-UTRA access to 5GC.

Figure 6.7.1-1: Low Power Periodic and Triggered 5GC-MT-LR Procedure with no change of LMF

1.	Steps 1-23 for the deferred 5GC-MT-LR procedure for periodic or triggered location events in clause 6.3.1 are performed with the following exceptions.

-	At step 14, the AMF includes an indication in the Nlmf_Location_DetermineLocation Request service operation that the UE supports and is allowed to use Control Plane CIoT 5GS Optimisation as described in TS 23.501 [18] clause 5.31.4.

-	At step 16, if the AMF indicates the UE supports and is allowed to use Control Plane CIoT 5GS Optimisation at step 14 and if the LMF decides to allow Control Plane CIoT 5GS Optimisation, the LMF includes an indication in the LCS Periodic-Triggered Invoke Request that the UE is allowed to use Control Plane CIoT 5GS Optimisation to send Event Reports. The LMF may also include criteria indicating when Control Plane CIoT 5GS Optimisation may be used to send Event Reports. The criteria may include a maximum duration for sending Event Reports using Control Plane CIoT 5GS Optimisation, a maximum number of consecutive Event Reports to be sent using Control Plane CIoT 5GS Optimisation or both.

NOTE 1:	As part of negotiating 5G network behaviour during registration, a UE indicates in a Registration Request whether Control Plane CIoT 5GS Optimisation is supported for location event reporting. This indication may be passed to the LMF by the AMF at step 14 for clause 6.3.1.

2.	The UE determines whether to report the event detected at step 22 in clause 6.3.1 using Control Plane CIoT 5GS Optimisation or using a NAS signalling connection. If the UE is currently in CM CONNECTED state or is not accessing E-UTRA connected to 5GC, the UE shall use a NAS signalling connection. Otherwise, the determination shall be based on criteria received from the LMF at step 14 in clause 6.3.1 when criteria are received. When the criteria include a maximum duration for sending event reports using Control Plane CIoT 5GS Optimisation, the UE shall report the event using a NAS signalling connection if the UE has not used a NAS signalling connection to report events during an immediately preceding time interval equal to the maximum duration. When the criteria include a maximum number of consecutive Event Reports to be sent using Control Plane CIoT 5GS Optimisation, the UE shall report the event using a NAS signalling connection if the UE has used Control Plane CIoT 5GS Optimisation to report each of the N previous events, where N equals the maximum number of consecutive Event Reports. In other cases, Control Plane CIoT 5GS Optimisation may be used to report the event if supported by the 3GPP access type. If use of a NAS signalling connection is determined, steps 24-31 for the procedure in clause 6.3.1 are performed and steps 3-11 below are skipped. If use of Control Plane CIoT 5GS Optimisation is determined, steps 3-11 below are performed.

3.	If the UE and ng-eNB node both support EDT, the UE sends an RRCEarlyDataRequest message to the ng-eNB node and includes a NAS control plane service request.

Otherwise, the UE established an RRC connection with the ng-eNB node and sends the NAS control plane service request. The NAS control plane service request includes an event report message which includes the information described in step 25 in clause 6.3.1 (e.g. the type of event being reported and any location measurements or location estimate obtained at step 23 in clause 6.3.1). The control plane service request also includes the deferred routing identifier received in step 16 in clause 6.3.1. The UE also includes a NAS Release Assistance Indication (NAS RAI) in the NAS message. The NAS RAI indicates a single response is expected.

NOTE 2:	Event reporting using Control Plane CIoT 5GS Optimisation requires a single Event report acknowledgment from the LMF.

4.	The ng-eNB node forwards the NAS message to the AMF in an N2 Initial UE message and includes an "EDT Session" indication if EDT was used at step 3.

5.	The AMF checks the integrity of the NAS message and deciphers its contents. The AMF then invokes an Namf_Communication_N1MessageNotify service operation to forward the event report to either the serving LMF or any suitable LMF as described for step 25 for clause 6.3.1. The AMF includes an indication of Control Plane CIoT 5GS Optimisation in the service operation and the serving cell ID.

6.	The AMF returns a NAS Service Accept to the ng-eNB node which is encapsulated in an N2 Downlink NAS Transport message.

7.	If EDT was used at step 3, the ng-eNB node establishes an RRC connection to the UE.

8.	The ng-eNB node sends an RRC DL Information Transfer message to the UE and includes the NAS Service Accept message received in step 6.

9.	The LMF invokes an Namf_Communication_N1N2MessageTransfer operation to return an acknowledgment for the event report as described for step 26 for clause 6.3.1.

10.	The AMF forwards the acknowledgment to the ng-eNB node in a NAS DL NAS Transport message which is encapsulated in an N2 Downlink NAS Transport message. The AMF also includes an "end indication" in the N2 message.

NOTE 3:	If the AMF determines more data or signalling may be pending for the UE, the AMF does not include the "end indication" at step 10 and step 12 below is not performed.

11.	The ng-eNB node sends an RRC DL Information Transfer message to the UE and includes the NAS message received in step 10.

12.	If the "end indication" was received in step 10, the ng-eNB node releases the RRC connection.

13.	If a location estimate is needed for event reporting, the LMF determines the UE location using the location measurements and/or location estimate received in the event report at step 5 and/or the serving cell ID provided by the AMF at step 5.

NOTE 4:	The LMF does not attempt to obtain additional location measurements from the UE or from the NG-RAN.

14.	Steps 28-31 for clause 6.3.1 are performed to send the event report to the external client or AF and to continue event detection at the UE.

**** Next Change ****

### 6.7.X	Event Reporting in RRC INACTIVE state

Figure 6.7.X-1 summarizes the initiation and reporting of location events for a deferred 5GC-MT-LR procedure for Periodic or Triggered Location Events when a UE is in RRC INACTIVE state. This procedure is only applicable to a UE with NR access to 5GC.

Figure 6.7.X-1: Low Power Periodic and Triggered 5GC-MT-LR Procedure in RRC INACTIVE state

1.	Steps 1-21 for the deferred 5GC-MT-LR procedure for periodic or triggered location events in clause 6.3.1 are performed with the following differences.

-	At step 16 in clause 6.3.1, the LMF indicates to the UE whether UL, DL, RAT independent or UL+DL positioning will be used for subsequent location reporting events when the UE is in RRC INACTIVE state. If UL positioning will be used, the LMF does not include an LPP positioning message in the LCS Periodic-Triggered Invoke Request sent to the UE at step 16. If DL positioning or RAT independent positioning will be used, the LMF includes an LPP positioning message in the LCS Periodic-Triggered Invoke Request sent to the UE at step 16, where the LPP positioning message requests DL location measurements, RAT independent location measurements or a location estimate based on these location measurements. If UL+DL positioning will be used, the LMF includes an LPP positioning message in the LCS Periodic-Triggered Invoke Request sent to the UE at step 16, where the LPP positioning message identifies an UL-DL positioning method and requests DL location measurements for this positioning method. The LMF also includes a deferred routing identifier in the LCS Periodic-Triggered Location Invoke which is an identification of the LMF. If no location of the UE is needed for event reporting or if a location based on a Cell ID will suffice for the location QoS, the LMF follows the procedure for DL and RAT independent positioning but does not include an LPP positioning message in the LCS Periodic-Triggered Invoke Request sent to the UE at step 16. The UE will then not obtain DL or RAT independent location measurements at step 4 or include an LPP positioning message in the supplementary services event report message sent at step 5.

NOTE 1:	A deferred routing identifier indicating a default LMF cannot be included in the LCS Periodic-Triggered Location Invoke sent at step 16 in clause 6.3.1 because for event reporting at steps 5-21 below. the LMF needs to know which type of positioning was indicated at step 1 in order to correctly support these subsequent steps. However, a default LMF would not know which type of positioning had been indicated at step 1.

2.	The UE enters RRC INACTIVE state some time before an event is detected at step 22 or step 31 in clause 6.3.1. If the UE is not in RRC INACTIVE state when an event is detected at step 22 or step 31 in clause 6.3.1, then the UE follows the procedure described for steps 22-31 in clause 6.3.1 to report the event to the LMF and to the LCS Client or AF, depending on the type of LPP message or lack of an LPP message received at step 16 in clause 6.3.1.

NOTE 2:	The LMF is not aware of whether the UE is in RRC INACTIVE state. This allows the LMF to follow the procedure described here or the procedure described in clause 6.3.1 for event reporting. With the procedure described here, a UE that was initially in RRC INACTIVE state can remain in RRC INACTIVE state after the procedure is complete. With the procedure in clause 6.3.1, a UE that was initially in RRC INACTIVE state could be moved into RRC CONNECTED state during the procedure in clause 6.3.1.

3.	The UE monitors for and detects a trigger event as described for step 22 in clause 6.3.1.

4.	If DL or RAT independent positioning was indicated at step 1, the UE obtains the location measurements or location estimate requested in the LPP message received in step 1. If UL positioning or UL+DL positioning was indicated at step 1, this step is skipped.

5.	The UE sends an RRC Resume Request with small data transmission (SDT) to a receiving gNB node in the NG-RAN. The RRC Resume Request includes an RRC UL Information Transfer message containing an UL NAS TRANSPORT message that includes a supplementary services event report message. If DL or RAT independent positioning was indicated at step 1, the UE includes an LPP positioning message in the supplementary services event report message that includes the location measurements or location estimate obtained at step 4. If UL+DL positioning was indicated at step 1, the UE includes an LPP positioning message in the supplementary services event report message that includes a request for an UL Configuration to support the UL+DL positioning method indicated to the UE at step 1. If UL positioning was indicated at step 1, the UE does not include an LPP positioning message in the supplementary services event report message. The event report message also includes other information described in step 25 in clause 6.3.1 (e.g. the type of event being reported). The UL NAS TRANSPORT message also includes the deferred routing identifier received in step 16 in clause 6.3.1.

6.	The receiving gNB node forwards the UL NAS TRANSPORT message to the serving AMF in an N2 Uplink NAS Transport message.

NOTE 3:	If the receiving gNB node is not the anchor gNB node for the UE, the UL NAS TRANSPORT message may be forwarded to the serving AMF via the anchor gNB node.

7.	The AMF checks the integrity of the NAS message and deciphers its contents. The AMF then invokes an Namf_Communication_N1MessageNotify service operation to forward the event report to the serving LMF as described for step 25 for clause 6.3.1.

8.	If UL or UL+DL positioning was indicated to the UE at step 1, the LMF uses the Network Assisted Positioning Procedure in clause 6.11.2 to request the receiving gNB node to provide an UL configuration to the UE at step 12. The receiving gNB node then determines an UL Configuration to be used later at step 12 and indicates this to the LMF. The LMF also uses the Non-UE Associated Network Assistance Data procedure in clause 6.11.3 to request UL location measurements of the UE by NG-RAN gNB nodes.

NOTE 4:	Because the UE is in RRC INACTIVE state and thus in CM CONNECTED state, step 2 of the Network Assisted Positioning Procedure in clause 6.11.2 whereby the AMF performs a Network Triggered Service Request will not be performed to support UL or UL+DL positioning. This avoids a transition of the UE into RRC CONNECTED state.

9.	The LMF invokes an Namf_Communication_N1N2MessageTransfer operation to return an acknowledgment for the event report as described for step 26 for clause 6.3.1.

NOTE 5:	If there is a change of LMF according to clause 6.4, the acknowledgment includes a deferred routing identifier for the new LMF as described in clause 6.4.

10.	The AMF forwards the acknowledgment to the receiving gNB node in a DL NAS TRANSPORT message which is encapsulated in an N2 Downlink NAS Transport message.

NOTE 6:	If the receiving gNB node is not the anchor gNB node for the UE, the DL NAS TRANSPORT message may be forwarded to the receiving gNB node via the anchor gNB node.

11.	The receiving gNB node sends a Subsequent DL SDT message to the UE and includes the NAS message received in step 10.

12.	The receiving gNB node sends an RRC Release message to the UE to keep the UE in RRC INACTIVE state and includes the UL Configuration determined at step 8 when UL or UL+DL positioning is used. If DL or RAT independent positioning was indicated to the UE at step 1, the LMF and UE skip steps 13 to 21 and proceed to step 22.

13.	The UE transmits UL positioning signals according to the UL Configuration received at step 12. If UL+DL positioning was indicated at step 1, the UE also obtains DL location measurements as requested at step 1. The NG-RAN gNB nodes that were requested to obtain UL location measurements at step 8 obtain the UL location measurements of the UL positioning signals transmitted by the UE.

14.	The NG-RAN gNB nodes transfer the UL location measurements obtained at step 13 to the LMF according to the Non-UE Associated Network Assistance Data Procedure in clause 6.11.3. If UL positioning was indicated to the UE at step 1, the LMF and UE skip steps 15 to 21 and proceed to step 22. If UL+DL positioning was indicated to the UE at step 1, the LMF and UE continue with steps 15 to 21.

15.	The UE sends an RRC Resume Request with small data transmission to the receiving gNB node. The RRC Resume Request includes an RRC UL Information Transfer message containing an UL NAS TRANSPORT message that includes a supplementary services event report message. The event report message indicates that this is a second event report associated with the first event report sent at step 5 and includes an LPP positioning message that includes the DL location measurements obtained at step 13. The UL NAS TRANSPORT message also includes the deferred routing identifier received in either step 16 in clause 6.3.1 or step 11 in this procedure if there was a change of LMF.

16.	The receiving gNB node forwards the UL NAS TRANSPORT message to the serving AMF in an N2 Uplink NAS Transport message.

NOTE 7:	If the receiving gNB node is not the anchor gNB node for the UE, the UL NAS TRANSPORT message may be forwarded to the serving AMF via the anchor gNB node.

17.	The AMF checks the integrity of the NAS message and deciphers its contents. The AMF then invokes an Namf_Communication_N1MessageNotify service operation to forward the event report to the serving LMF as described for step 25 for clause 6.3.1.

18.	The LMF invokes an Namf_Communication_N1N2MessageTransfer operation to return an acknowledgment for the event report as described for step 26 for clause 6.3.1.

19.	The AMF forwards the acknowledgment to the receiving gNB node in a DL NAS TRANSPORT message which is encapsulated in an N2 Downlink NAS Transport message.

NOTE 8:	If the receiving gNB node is not the anchor gNB node for the UE, the DL NAS TRANSPORT message may be forwarded to the receiving gNB node via the anchor gNB node.

20.	The receiving gNB node sends a Subsequent DL SDT message to the UE and includes the NAS message received in step 19.

21.	The receiving gNB node sends an RRC Release message to the UE to keep the UE in RRC INACTIVE state.

22.	Following step 21 when UL+DL positioning was indicated at step 1, step 14 when UL positioning was indicated at step 1, or step 12 when DL positioning was indicated at step 1, the UE remains in RRC INACTIVE state.

23.	If a location estimate is needed for event reporting, the LMF determines the UE location using one of the following: for DL or RAT independent positioning, the LMF uses the location measurements or location estimate received at step 7 or the cell ID provided by the AMF at step 7; for UL positioning, the LMF uses the location measurements received at step 14; and for UL+DL positioning, the LMF uses the UL location measurements received at step 14 and the DL location measurements received at step 17.

NOTE 9:	The LMF does not attempt to obtain additional location measurements from the UE or from the NG-RAN.

24.	Steps 28-31 for clause 6.3.1 are performed to send the event report to the LCS client or AF.

**** End of Changes ****