SA WG2 Meeting #150E (e-meeting)	S2-2203119

April 06 – 12, 2022, Elbonia

Source:	Orange

Title:	KI#2, New Sol: 5GC awareness of URSP rule evaluation and verification of network slice usage

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This paper proposes a new solution for Key Issue #2.

# Proposal

It is proposed to agree the following changes into TR 23.700-85 on FS_eUEPO.

* * * * Start of changes * * * *

## 6.X	Solution #X: 5GC awareness of URSP rule evaluation and verification of network slice usage

### 6.X.1	Description

This solution addresses Key Issue #2: "5GC awareness of URSP enforcement".

This solution is intended to be used when a specific network slice has been deployed for the exclusive usage of an application service provider. Business agreements between the MNO and the application service provider guarantee that the network slice only carries traffic from the applications of the application service provider.

When the evaluation of URSP rules triggers the establishment of a new PDU Session, the UE includes in the PDU Session Establishment Request the Traffic Descriptor of the matched URSP rule, and the application identity. The SMF may verify that the indicated application identity is trusted and that the S-NSSAI is allowed for this application according to operator policy. If this verification fails, the SMF rejects the PDU Session Establishment Request with a proper cause value.

Editor’s note: User privacy consideration is FFS. User consent is needed for application identity reporting and the detailed descriptions and procedures of user consent is FFS.

Editor’s note: Details on the application identity and its verification are FFS.

Editor’s note: The case where the evaluation of URSP rules does not trigger the establishment of a new PDU Session is FFS.

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.3	Impacts on services, entities and interfaces

Editor's note: This clause captures impacts on existing 3GPP nodes and functional elements.

* * * * End of changes * * * *