3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2203138

Elbonia, April 6 – 12, 2022	(revision of S2-2200934, 2465r01)

Source:	Huawei, HiSilicon

Title:	KI#3 Solution: Path switching from PC5 path to Uu path

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: This contribution introduces a new solution to solve the path switching from PC5 path to Uu path.

# 1. Introduction/Discussion

This contribution introduces a new solution to solve the path switch from PC5 to Uu for KI#3 in TR 23.700-33.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change * * * *

# 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  |  | 3 |  |
| X |  |  | X |  |

# * * * * Second change (all new) * * * *

## 6.X	Solution #X: Path switching from PC5 path to Uu path

### 6.X.1	Description

This solution resolves Key Issue #3 for direct communication path switching from PC5 path to Uu path.

This solution uses the make-before-break mechanism to reduce interruption when path switch from PC5 to Uu. The two UEs perform the Uu path preparation procedure for the switched service, and then may release the PC5 connection. During the Uu path preparation procedure phase, the two UEs may negotiate, using the ProSe layer, the Uu QoS based on PC5 QoS via the PC5 connection in order to ensure consistent service experience, and optionally share the IP address used for the Uu path to each other to achieve the switch of service transmission.

### 6.X.2	Procedures

Figure 6.X.2-1: high-level procedure for path switch from PC5 to Uu

1.	Triggered by an AF request, the PCF may provide the path switch Policy/parameters for Proximity Services to the UE (including UE1 and UE2) by using the procedure as defined in clause 4.2.4.3 in TS 23.502 [8]. The path switch Policy/parameters may include whether the specific service (e.g., Service A) is allowed to switch from PC5 to Uu.

2.	UE1 and UE2 have an established PC5 connection and are transferring service data with each other for Service A. Before the PC5 connection establishment, UE1 and UE2 may get the path selection policy, which indicates that the PC5 path is preferred for Service A.

3.	When e.g., the PC5 signal level is lower than the configured threshold or PC5 QoS for Service A cannot be satisfied, UE1 sends the path switch request for Service A to UE2. The request message includes the Service A identifier. Before UE1 sends the request, UE1 checks the path switch Policy/parameters to make sure that Service A is allowed to switch from PC5 to Uu.

4.	UE2 sends the path switch response to UE1 that indicates that Service A can be switched to Uu path. The response message includes the Service A identifier. Before UE2 sends the response, UE2 checks the path switch Policy/parameters to make sure that Service A is allowed to switch from PC5 to Uu.

NOTE: PC5 connection is still valid to transfer the PC5 signalling in step 3 and step 4.

5.	UE1 and UE2 perform the Uu path preparation procedure. In particular, UE1 and UE2 each trigger PDU Session establishment/modification procedure, if needed, to make the Uu path ready for Service A transmission. During this procedure UE1 and UE2 can get the IP address of the PDU session that will be used for the Service A, and UE1 and UE2 can respectively request the Uu QoS 1 (used for Uu path of UE1) and Uu QoS 2 (used for Uu path of UE2) for Service A .

Before UE1 and UE2 request the Uu QoS for Service A, UE1 decides the Uu QoS 1 and Uu QoS 2 requirements for Service A based on PC5 QoS requirement for Service A and sends Uu QoS 2 requirements to UE2 via the PC5 connection. Uu QoS 1 and Uu QoS 2 requirements can be decided based on the configured mapping of PC5 QoS parameter to Uu QoS parameter or based on UE1 implementation.

Optionally, UE1 and UE2 can also share the IP/port addresses used for Uu path with each other via the PC5 connection, to achieve the switch of the Service A transmission.

6.	UE1 and UE2 transmit the data of the Service A via the Uu path.

7.	After the step 6, UE1 and UE2 may release the PC5 connection, using the existing Layer-2 link release over PC5 reference point, see TS 23.304 [3], clause 6.4.3.3.

Editor’s Note: how to support Ethernet traffic and Unstructured traffic is FFS.

### 6.X.3	Impacts on services, entities and interfaces

UE:

-	Support path switch procedure following the policy, including Uu path preparation procedure.

PCF:

-	Composition of path switch policy.

# * * * * End of changes * * * *