3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2203452

Elbonia, April 6 – 12, 2022	(revision of S2-2202135r01)

Source:	Huawei, HiSilicon

Title:	Solution on support of onboarding over untrusted non-3GPP access in SNPN

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract: This contribution would like to propose a new solution on support of R17 functionalities such as onboarding over untrusted non-3GPP access in SNPN.

1. Introduction/Discussion

Clause 5.30.2.8 and Annex D, clause D.3 of TS 23.501 [3] specify how the UE can access SNPN services via a PLMN. Solution #2 defines how the UE can access SNPN services via Untrusted non-3GPP access network using the credentials owned by the SNPN.

This solution defines how the UE can access SNPN via Untrusted non-3GPP access network for Onboarding.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-08.

# * * * * First change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| X |  | X |  |  |  |  |

# * * * * Second change * * * *

6.X	Solution #X: Support of onboarding over untrusted non-3GPP access in SNPN

6.X.1	Introduction

Editor's Note:	This clause lists the key issue(s) addressed by this solution, and briefly the main principles of the solution.

This solution aims at addressing key Issue #2 about support of Non-3GPP access for SNPN. In particular, this solution mainly focuses on how to support functionalities defined in R17 eNPN such as onboarding and remote provisioning over untrusted non-3GPP access. For UE accessing SNPN using credentials owned by the SNPN, Solution #2 specified in clause 6.2 can be applied.

This solution assumes that access to PVS is restricted inside the ON-SNPN and the PVS is not accessible from the public internet directly over the “untrusted non-3GPP access network”.

6.X.2	Functional Description

Editor's Note:	This clause further details the solution principles and any assumptions made.

Before the UE registers to an SNPN over untrusted non-3GPP access for Onboarding, it shall select a N3IWF in the SNPN which supports Onboarding. Additionally, the SNPN shall support the Default Credentials Server belonging to a group identified by GINs.

Editor’s Note: How the UE can select non-3GPP access network that supports access to the N3IWF in the SNPN which supports Onboarding is FFS.

Therefore, clause 6.3.6.2a of TS 23.501[3] can be applied with following clarifications and additions:

- The configured N3IWF FQDN may consist of GIN that identifying a group the DCS belongs to.

- The FQDN constructed by the UE GIN that identifies a group the DCS belongs to and the Visited Country FQDN, indicating the query is for SNPN and performing a DNS query for the resulting FQDN. The example of N3IWF FQDN consisting of GIN is shown below:

n3iwf.5gc.GIN999123456789ABCDE..pub.3gppnetwork.org

- After UE selects a N3IWF that supports Onboarding, it shall include an Onboarding indication in the AN parameters included in the EAP-Res/5G-NAS message which are sent to N3IWF during registration procedure as specified in clause 4.12.2.2 in TS 23.502 [4]. The selected N3IWF shall select an AMF that supports Onboarding based on the Onboarding indication included in the AN parameters.

Editor’s Note: Whether GIN needs to be in the FQDN for N3IWF that supports onboarding is FFS.

Editor’s Note: Which authority will respond to the DNS query for FQDN that contains GIN and support onboarding is FFS.

6.X.3	Procedures

Editor's Note: This clause describes procedures and information flows for the solution.

6.X.4	Impacts on services, entities, and interfaces

Editor's Note: This clause lists impacts to services, entities, and interfaces.

UE impact:

-	Ability to construct an FQDN consist of GINs used for selecting a preferred SNPN that supports onboarding.

-	Ability to include Onboarding indication in the AN parameter sent to the N3IWF during registration procedure.

N3IWF impact:

-	Ability to select an AMF that supports Onboarding based on the Onboarding indication included in the AN parameters.

# * * * * End of changes * * * *