3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202126

Elbonia, April 6th – 12th, 2022	(merge of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New solution for KI#7: 5GS assistance to FL member selection

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract: It proposes a new solution for KI#7 on 5GS assisted FL member selection for FS_AIMLsys.

1. Introduction/Discussion

This contribution is related to KI#7 "5GS Assistance to Federated Learning Operation":

On assistance to selection of UEs for FL operation:

-	Whether, how and what information provided by 5GC to the AF can help the AF to select and manage the group of UEs which will be part of FL operation.

-	Whether, how and what information is required by the 5GC in order to assist the AF for selecting and managing the group of UEs which will be part of FL operation.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-80.

# * * * * First change * * * *

6.X	Solution #X: 5GC information assistance to FL member selection

6.X.1	Description

This solution addresses Key Issue #7 "5GS Assistance to Federated Learning Operation" on assistance to selection of UEs for FL operation.

This solution is based on the principles as follows:

-	The FL group management is under the control of the AF; and,

-	The 5GC NF just provides some network information to help the AF to select and manage the group of UEs which will be part of FL operation; and,

-	How the AF selects and manages the group of UEs which will be part of FL operation based on the network information is out of 3GPP scope.

To request the network information from the 5GC NF, the AF may provide Area of Interest and/or UE group ID to reduce the signalling of network information exposure. Take the FL assisted Word Query Suggestion Model as an example, the AF for Gboard on Android (i.e. the Google Keyboard) may provide TAC (Type Allocation Code) as an External UE group ID to 5GC NF to request the UE’s network information if the UE is an Android terminal. Furthermore, there are hundreds of millions of Android UEs in the 5G network, so to save the signalling of network information exposure, the AF may also provide an AoI to 5GC NF to request the UE’s network information if the UE is located in the AoI or about to enter the AoI.

The assistance network information can be provided by 5GC NF to assist FL member selection by AF is shown in Table 6.X.1-1.

Table 6.X.1-1: Information can be provided by 5GC NF for FL member selection

| Network Information | 5GC NF | Description | Actions of AF (examples) |
| --- | --- | --- | --- |
| UE locations (1..max) | AMF | UE positions | AF only selects the UEs who are located in the requested AoI to join the FL group. |
| >UE location |  | TA or cells that the UE enters (NOTE 1) |  |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |  |
| CM state | AMF | The NAS signalling Connection state of the UE with the AMF, including CM-IDLE state and CM-CONNECTED state | AF only selects the UEs who are in Connection state to join the FL group. |
| UE Reachability | AMF | Whether the UE is reachable | When the number of the connected UEs is not enough, AF may select the UEs who are in IDLE state but are reachable to join the FL group. |
| UE related analytics | NWDAF | Statistics or prediction on UE Mobility and Abnormal behaviour. | 1) AF only selects the UEs who are located in (UE mobility statistics) or about to enter (UE mobility predictions) the requested AoI to join the FL group. 2) AF does not select the UEs who are abnormal to join the FL group. |
| NOTE 1: UE location includes either the last known location or the current location, under the conditions defined in Table 4.15.3.1-1 in TS 23.502 [x]. |  |  |  |

To subscribe the UE locations, CM state and UE Reachability from AMF, the Event IDs i.e. "Location Report", "Connectivity state changes", "UE reachability status" as defined in clause 5.2.2.3.1 of TS 23.502 [4] are used respectively. Instead of the "Location Report", the "UE moving in or out of a subscribed "Area Of Interest"" event can also be used to monitor if a UE is located in the requested AoI.

To subscribe the UE Mobility and Abnormal behaviour from NWDAF, the Analytics ID i.e. "UE Mobility" as defined in clause 6.7.2 of TS 23.288 [6] and "Abnormal behaviour" as defined in clause 6.7.5 of TS 23.288 [6] are used respectively.

6.X.2	Procedures

Step 1a~Step 5a. The procedure in clause 6.1.1.2 of TS 23.288 [6] for analytics subscribe by AFs via NEF are re-used.

Step 1b~Step 5b. The procedure in clause 4.15.3.2.3 of TS 23.502 [4] for event notifications subscribe by AFs via NEF are re-used.

Editor's note:	Whether the enhancement of the current procedures for Analytics Exposure or Event Exposure is FFS.

6.X.3	Impacts on services, entities and interfaces

AF:

-	Support to subscribe the Event IDs i.e. "Location Report", "Connectivity state changes", "UE reachability status", and/or "UE moving in or out of a subscribed "Area Of Interest"" by AMF.

-	Support to subscribe the Analytics IDs i.e. "UE Mobility" and "Abnormal behaviour" by NWDAF.

# * * * * End of changes * * * *