SA WG2 Meeting #150e	S2-2202204

# April 06th – 12th, 2022; Elbonia               		  (revision of S2-2201083)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	new solution for WT1: URSP solution to support roamers access to EHE in a VPLMN

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_EDGE_Ph2/ Rel-18

Abstract of the contribution: new solution for WT1: URSP solution to support roamers access to EHE in a VPLMN

# Discussion

The FS_EDGE_Ph2 SID contains

WT1)	Improvements to roaming, to support access to EHE in a VPLMN

# 2 Proposal

It is proposed to update TR 23.700-48 as follows

## 6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |  |  |  |  |  |
| Solution X: URSP solution to support roamers access to EHE in a VPLMN | x |  |  |  |  |  |  |  |  |

NEXT CHANGE (2) ALL text is new

## 6.X	Solution #X: URSP solution to support roamers access to EHE in a VPLMN

### 6.X.1	Description

UE not served by their home network should where possible (when allowed by their home network) benefit from traffic offload and Edge Computing capabilities deployed by their serving network.

A solution is to leverage TS 23.548 [FFS] clause 6.6 “Support of AF Guidance to PCF Determination of Proper URSP Rules” and clause 4.15.6.10 of TS 23.502 [FFS] “Application guidance for URSP rules determination mechanisms” as defined for R17. This is to guide UE for traffic to be offloaded in the VPLMN to use a specific DNN and slice deployed in LBO mode while the rest of he traffic can us HR PDU session(s).

The VPLMN acting as an AF of the HPLMN i.e. an AF belonging to the VLMN reaching the NEF of a roaming partner indicates the (VPLMN) DNN and slices related with traffic offload / EC that it supports (per TS 23.548 [FFS] §6.6 “Support of AF Guidance to PCF Determination of Proper URSP Rules” and clause 4.15.6.10 of TS 23.502 ” application guidance for URSP rules determination mechanisms”); this indication can be leveraged by the PCF of the HPLMN to (if authorized by HPLMN policies) provide dedicated URSP rules applicable to relevant users of the HPLMN when they are roaming in that VPLMN.  These URSP may have a validity condition corresponding to the VPLMN location but should refer to S-NSSAI(s) defined by the HPLMN.

The H-PCF may, based on local policies, determine whether to always send the URSP rules immediately to the UE or to send these rules only when the UE is served by the corresponding serving PLMN, e.g. waiting to be triggered from a V-PCF of that PLMN in step 3 of TS 23.502 [FFS] Clause 4.6.11.

The PCF of the HPLMN when it sends such URSP update to a UE, needs, as for R17, to ensure that the DNN, S-NSSAI provided in the URSP rules sent to the UE is allowed in LBO mode in the user subscription (in UDM/UDR) even when the UE is roaming in that VPLMN.

NOTE: This solution can also apply for UEPO WT1.

The VPLMN may provide Geographical or VPLMN location (e.g. VPLMN TAI or cells) related validity conditions for the URSP to apply. When the VPLMN AF provides the HPLMN with VPLMN validity conditions, the VPLMN location information (e.g. VPLMN TAI or cells) contains a VPLMN ID that may be used by the H-PCF to understand that this information is specific for a VPLMN/

### 6.X.2	Procedures

The solution reuses existing procedures

### 6.X.3	Impacts on Existing Nodes and Functionality

- The VPLMN and the HPLMN as part of the roaming agreement need to ensure that a VPLMN AF can contact the NEF of the HPLMN to use the API defined in clause 4.15.6.10 of TS 23.502 [FFS] “Application guidance for URSP rules determination mechanisms”