SA WG2 Meeting #150E (e-meeting)	S2-2202412

April 6 – 12, 2022, Elbonia

Source:	LG Electronics

Title:	KI#7, New Sol: Providing 5GS Assistance information to AF for Federated Learning Operation

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This paper proposes a new solution for KI#7 "5GS Assistance to Federated Learning Operation".

Discussion

Regarding Key Issue #7: 5GS Assistance to Federated Learning Operation captured in clause 5.7 of TR 23.700-80v0.1.0, this paper proposes a new solution.

Proposal

It is proposed to agree the following changes into TR 23.700-80v0.1.0.

* * * * Start of 1st Change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Z |  |  |  |  |  |  | X |

* * * * Start of Next Change * * * *

!! All New Texts !!

6.Z	Solution #Z: Providing 5GS Assistance Information to AF for Federated Learning Operation

6.Z.1	Description

This is a solution for key issue #7 "5GS Assistance to Federated Learning Operation".

The outline of the proposed solution for providing 5GS assistance information to AF for Federated Learning operation is as below:

-	This solution introduces a new NF, AIML NF which is supported by NEF and used for external exposure to assist 5G AIML services. (e.g. receives request from AF (i.e. AI/ML Application Server) and response with 5GS assistance information for FL operation)

-	A dedicated NEF may be deployed to provide only the AIML NF functionality, i.e. to assist AIML Federated Learning operation specific features/APIs and NEF features/AIPs that are specified for capability exposure towards AI/ML Application Server.

-	AI/ML Application Server requests assistance information for FL operation. (e.g. UE list, QoS references, geographic area, minimum number of UEs for FL operation, Preferred FL time periods, whether to request the lists of candidate UEs for FL operation, preferred time periods for FL operation, etc). For example, some FL operation should be performed during a certain time or some FL operations may not need to be performed immediately. In this case, AI/ML Application Server can provide preferred FL operation time period. Also, based on the training data size, AI/ML Application Server can provide expected FL operation time duration. The AIML NF/NEF can derive recommended time periods for FL operation by considering the preferred FL operation time periods and the expected FL operation time duration, and by predicting the network condition such as user data congestion time.

-	AIML NF/NEF processes the request from AI/ML Application Server by interacting with other NFs such as NWDAF, UDM, etc., and generates assistance information (e.g. lists of candidate UEs for recommended time periods) for FL operation based on the collected information, and provides the assistance information for FL operation requested by AI/ML Application Server.

6.Z.2	Procedures

6.Z.2.1	Procedure for Providing 5GS Assistance Information for Federated Learning Operation

Figure 6.Z.2.1-1 shows the procedure for providing 5GS assistance information to AI/ML Application Server for Federated Learning operation.

Figure 6.Z.2.1-1: Procedure for providing 5GS assistance information for Federated Learning operation

1.	AI/ML Application Server invokes Nnef_FLAssistanceInfo_Request to request 5G assistance information with specific conditions for FL operation. The AI/ML Application Server requests FL assistance information for the UE list included in the request. It also indicates that these UEs are authorized by AI/ML Application Server (application side) for FL operation.

The input parameters of the Nnef_FLAssistanceInfo_Request service operation can be as below:

-	AF Identifier

-	UE List (GPSIs, External Group ID, or IP addresses)

-	AF-Service-Identifier

-	External Application Identifier

-	QoS Reference

-	Whether to request the list of candidate UEs

-	Time when the assistance information is needed

-	Expected FL operation time duration (Expected time duration by AI/ML Application Server based on the training information such as training model and training data size)

-	Preferred FL operation time periods (Time periods that the AI/ML Application Server want to perform FL operation)

-	Whether to request the recommended FL operation time periods

-	Traffic volume for downloading global model and uploading training results

-	DNN and S-NSSAI

-	Expected UE Moving Trajectory for each UE in the UE list (e.g. A planned path of movement)

-	Area of Interest (FL operation for the UEs in the area of interest)

-	Minimum number of UEs needed for FL operation

2.	If the request from AI/ML Application Server is authorized, AIML NF/NEF determines whether the UEs provided by AI/ML Application Server are authorized by 5GC to support AIML FL operations from UDM by invoking Nudm_SDM_Get service operation. After successful response is received, the AIML NF/NEF may subscribe to be notified of the Subscription data modification using Nudm_SDM_Subscribe.

3.	For the authorized UEs for AIML FL operation by 5GC, AIML NF/NEF determines appropriate analytics information to derive assistance information such as lists of candidate UEs or recommended areas with recommended time periods that can satisfy the request from AI/ML Application Server.

AIML NF/NEF may request analytics information to NWDAF in order to collect analytics information of the requested UEs (e.g. UE Communication analytics, UE Mobility analytics, User Data Congestion analytics, QoS Sustainability analytics). If the AI/ML Application Server did not provide some of input parameters, AIML NF/NEF can derive them based on local configuration or based on analytics such as UE Mobility analytics.

4.	The NWDAF collects the data for the requested Analytics IDs as specified in TS 23.288 [6].

5.	The NWDAF derives the requested analytics as specified TS 23.288 [6].

6.	The NWDAF provides analytics response to the AIML NF/NEF.

7.	AIML NF/NEF collects all the analytics information from NWDAF and derives assistance information that can satisfy the request from AI/ML Application Server taking into account, e.g. the QoS, User Data Congestion, UE mobility, and the preferred FL time periods for FL operation, etc.

8.	AIML NF/NEF responds to the AI/ML Application Server with the derived assistance information.

The output parameters of the Nnef_FLAssistanceInfo_Request service operation can be as below:

-	Set of the 5GS Assistance information requested by AI/ML Application Server. Each 5GS Assistance information can contain:

-	List of candidate UEs for FL operation

-	Recommended time period to perform FL operation

-	Recommended area for FL operation

The lists of candidate UEs for recommended FL operation can be different according to the recommended time period and/or the recommended area for FL operation.

If the "time when the assistance information is needed" was provided in step 1, AIML NF/NEF takes this into account in responding to the AI/ML Application Server.

Based on the received the 5GS assistance information, the AI/ML Application Server can select and manage the group of UEs, and determine the start time for FL operation.

6.Z.3	Impacts on services, entities and interfaces

UDM:

-	New subscription information whether the UE is authorized for 5G AI/ML FL operation

AIML NF/NEF:

-	New NF added to assist application AI/ML FL operation.

-	New service operation to assist application AI/ML FL operation.

AI/ML Application Server:

-	Support new service operation provided by AIML NF/NEF.

* * * * End of Changes * * * *