3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202540

Elbonia, April 6–10, 2022	(revision of S2-220XXXX)

Source:	Huawei, HiSilicon, China Mobile

Title:	KI#4: Hierarchical NSACF Architecture for maximum UE/PDU Session number control

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This paper introduce the hierarchical NSACF architecture for maximum UE/PDU number control.

1. Introduction

NSACF is introduced at Rel-17 for maximum number of UE and PDU session control. For one S-NSSAI it is possible that multi NSACF are deployed. Each NSACF performs NSAC independently. Due to no standard mechanism is defined session continuity is not guaranteed if UE moves across service area. It is proposed to introduce the hierarchical NSACF architecture to enhance the UE/PDU Session number control and solve those identified issue.

2. proposal

It is proposed to capture the following changes vs. TR23.700-41.

# * * * * 1st change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 | KI#4 |
| - Hierarchical NSACF Architecture for Maximum UE/PDU Session number control |  |  |  | X |

# * * * * 2nd change * * * *

6.X	Solution #X:  Hierarchical NSACF Architecture for Maximum UE/PDU Session number control

6.X.1	Introduction

This is a solution to Key Issue #4, "Support of NSAC involving multi service Area".

As defined in Rel-17 an NSACF is deployed on a service area basis, which can be one NSACF instance or one NSACF Set. Each NSACF performs maximum number of registered UE or established PDU session number control independently. It is possible that UE registration or PDU session establishment is rejected by the network due to the maximum number of UE/PDU session is reached at the current serving NSACF even the maximum number may still be available at other NSACF. This also impacts the session continuity when the UE moves across the service area.

Thus how to enhance the NSAC mechanism when multi NSACFs are deployed at the network need be considered.

6.X.2	Functional Description

The hierarchical NSACF architecture for UE number control is shown as the Figure 6.X.2-1. For an S-NSSAI, one NSACF acting as Primary NSACF is introduced. Other NSACFs take the same role as the existing NSACF, i.e. serving one service area. The slice SLA attributes, i.e. the global maximum number values, is only configured at the Primary NSACF. The global maximum number value is shared among different service area(s). The Primary NSACF registers its NF profile to the NRF, which can be discovered by other NF.

Figure 6.X.2-1: Hierarchical NSACF Architecture for UE number control

Nxx:	Reference point between Primary NSACF and NSACF.

With the replacement of the AMF with SMF, same architecture is used for maximum PDU session number control.

To improve the signalling efficiency the Primary NSACF may allocate partial of the global maxim number value to the NSACF, i.e. the local maximum number. When the AMF or SMF interact with the NSACF, if the local maximum number at the NSACF is reached, the NSACF interact with the Primary NSACF.

6.X.3	Procedures

6.X.3.1	Registration management Procedures

Figure 6.X.3.1-1: NSAC check of the maximum number of UEs

The enforcement of maximum number of UEs registered for an S-NSSAI is performed as following:

-	Step 1-2, same as the step 1-2 defined in clause 4.2.11.2 of TS23.502 [xx].

-	Step 3, the NSACF performs NSAC for the indicated S-NSSAI.

If the update flag parameter from the AMF indicates increase,

- If the local maximum number of UEs is available, the NSACF execute same as step 3 defined in clause 4.2.11.2 of TS23.502[xx]. Step 4-7 are skipped.

- If the local maximum number of UEs is reached, the NSACF interact with the Primary NSACF. Step 4-7 are executed.

If the update flag parameter from the AMF indicates decrease,

- If the UE entry to be deleted is stored at the NSACF, i.e. the UE entry with the same UE ID, NF ID and Access type is stored at the NSACF, the NSACF execute same as step 3 defined in clause 4.2.11.2 of TS23.502[xx]. Step 4-7 are skipped.

- If the UE entry to be deleted is not stored at the NSACF, the NSACF interact with the Primary NSACF. Step 4-7 are executed.

-	Step 4, if the Primary NSACF has not been discovered before, the NSACF discovers and selects the Primary NSACF per the NF type.

-	Step 5, the NSACF invokes Nnsacf_NSAC_NumberOfUEsUpdate_Request to the Primary NSACF.

The NSACF forwards the update request to the Primary NSACF. If the update flag parameter from the AMF indicates increase, the NSACF also include the local maximum number of UEs, which is the previous value received from Primary NSACF, i.e. the configured one before.

-	Step 6, the Primary NSACF performs NSAC for the indicated S-NSSAI.

If the update flag parameter from the NSACF indicates increase and the local maximum number is received, per operator’s policy, the Primary NSACF may want to delegate the following NSAC update request done at the NSACF. In this case the Primary NSACF increases the local maximum number of UEs allocated to the NSACF and ignores the received UE ID information. Otherwise per the received UE ID information the UE entry stored at the Primary NSACF is updated for the related UE ID, NF ID and Access type.

NOTE: 	The UE entry managed by the Primary NSACF is used to support the session continuity when the UE moves to the new service area and local maximum number is reached at the target NSACF.

If the update flag parameter from the NSACF indicates decrease, the Primary NSACF updates the UE entry for the related UE ID, NF ID and Access type.

-	Step 7, the Primary NSACF returns the Nnsacf_NSAC_NumberOfUEsUpdate_Response. If the local maximum number of UEs is increased by the Primary NSACF, the updated local maximum number of UEs is also included, i.e. the updated configured value.

-	Step 8, the NSACF checks the response from primary NSACF and determines whether it need update the UE entry stored at the NSACF.

If the local maximum number of UEs is received from Primary NSACF, the NSACF replaces the local maximum number of UEs with the received updated value and update the UE entry for the related UE ID, NF ID and Access type. In other case the NSACF forwards the response to the AMF.

-	Step 9, same as the step 4 defined in clause 4.2.11.2 of TS23.502 [xx].

6.X.3.2	PDU Session management Procedures

Figure 6.X.3.2-1: NSAC check of the maximum number of PDU Sessions

The enforcement of maximum number of PDU Session established for an S-NSSAI is performed as following:

-	Step 1-2, same as the step 1-2 defined in clause 4.2.11.4 of TS23.502 [xx].

-	Step 3, the NSACF performs NSAC for the indicated S-NSSAI.

If the UE entry update at the NSACF is possible, e.g. Adding the associated PDU session ID for increase case or removing the associated PDU session ID for decrease case, same as step 3 defined in clause 4.2.11.4 of TS23.502[xx] is executed. Step 4-8 are skipped. Otherwise the NSACF interact with the Primary NSACF.

-	Step 4, if the Primary NSACF has not been discovered before, the NSACF discovers and selects the Primary NSACF per the NF type.

-	Step 5, the NSACF invokes Nnsacf_NSAC_NumberUpdate_Request to the Primary NSACF. The message includes the S-NSSAI, requested local maximum PDU session number, i.e. increasing the local maximum PDU session number.

-	Step 6, the Primary NSACF checks the global maximum PDU session number and determine whether accept or reject the requested the local maximum PDU session number.

-	Step 7, the Primary NSACF returns the Nnsacf_NSAC_NumberUpdate_Response. The response include an allocated local maximum PDU session number, i.e. the updated configured value.

-	Step 8, the NSACF replaces the local maximum PDU session number with the received allocated local maximum PDU session number value. If the allocated local maximum PDU session number is increasing, the NSACF creates or adds the associated PDU session ID into the UE entry. Otherwise the NSACF rejects the permission to establish the PDU session.

-	Step 9, same as the step 4 defined in clause 4.2.11.4 of TS23.502 [xx].

6.X.3.3	Redistribution of local maximum number

Figure 6.X.3.3-1: Redistribution of local maximum number

At any time the Primary NSACF may update the allocated local Maximum number of UE or PDU session configured at the NSACF as following:

-	Step 1-2, The Primary NSACF subscribes the slice event exposure service from the NSACF. This is to get the current registered number of UE or established PDU session number. Per the subscription, the NSACF notifies the registered number of UE or established PDU session number to Primary periodically or above the related threshold per the event subscribed.

-	Step 3, Per the received current registered UE/PDU session number and operator’s policy, the Primary NSACF decides to update the local maximum UE/PDU session number value configured at the NSACF, i.e. the configured value.

-	Step 4, the Primary NSACF invokes Nnsacf_NSAC_NumberUpdate_Request to the NSACF. The message includes the allocated local maximum number.

-	Step 5, the NSACF replaces the local maximum number with the received allocated local maximum number value.

-	Step 6, the NSACF returns the Nnsacf_NSAC_NumberUpdate_Response.

6.X.3.4	Session continuity handling

For maximum number of UE control, the NSACF discovered by the AMF (or SMF+PGW-C) is deployed as the following:

-	Different service area within one PLMN: the NSACF is deployed in each service area.

-	Roaming: the NSACF is located at the VPLMN.

- EPS interworking: when the UE camps at the EPS network, the SMF+PGW-C select the NSACF at the serving PLMN or the NSACF at the HPLMN depending on whether the PDU session is LBO PDU session or HR PDU session. When the UE camps at the 5GS network, the AMF selects the NSACF at the camping service area of the serving PLMN.

In above all case there is only one Primary NSACF instance or one NSACF Set, which is located at the HPLMN.

When UE moves across different service area, different NSACF may be interacted to perform the maximum UE number control. If local Maximum number is not reached at the NSACF, the NSACF accepts the UE registration at the new service area. If local Maximum number is reached, the NSACF forwards the NSAC request to the Primary NSACF. The Primary NSACF updates the UE ID entry per the received UE ID information until the maximum number at the Primary NSACF is reached. Thus even if the local maximum number at one NSACF is reached, the session continuity is still supported.

6.X.4	Impacts on services, entities and interfaces

The following impacts are foreseen by this solution:

NSACF:

-	A new NSACF type, i.e. Primary NSACF, is introduced. Compare to the Rel-17 NSACF function, it manages the global Maximum number value and distribute global Maximum number to NSACF additionally.

-	Support the update of the local Maximum number per the instruction from Primary NSACF.

-	Determines whether the UE ID entry update is to be performed at the NSACF or Primary NSACF.

# * * * * End of changes * * * *