SA WG2 Meeting #150E (e-meeting)	S2-2202763

April 06 – 12, 2022, Elbonia

Source:	Orange

Title:	New key issue for objective #2: Improved support of RAs including TAs supporting Rejected S-NSSAIs

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes a KI for objective #2

# Discussion

This paper proposes Key Issue for FS_eNS_Ph3 objective #2

2.	Study whether and how to initiate a registration for a rejected S-NSSAI that was rejected in a first TA of the RA but may be available in another TA of the RA.

# Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of changes (all new text) * * * *

## 5.X	Key Issue #X: Improved support of RAs including TAs supporting Rejected S-NSSAIs

### 5.X.1	Description

In 5GS specifications up to Rel-17, a UE that registers with a S-NSSAI which is not supported in the current TA, but is supported in some of the other TAs of the RA, receives a Rejected S-NSSAI indicating this S-NSSAI is not supported in the RA. The UE therefore can only attempt to request this S-NSSAI again when it camps in a cell outside the current RA, despite this S-NSSAI being supported in some TAs of the RA. To avoid delaying the UE to be able to request this S-NSSAI when it camps in a TA that supports the S-NSSAI, the only solution available today is to restrict the RA to only include the TAs where this S-NSSAI is not supported. This however may be suboptimal from the standpoint of signalling load and power consumption.

This Key Issue will study how to trigger the UE to perform Registration when the UE enters a TA that supports a S-NSSAI that was included in the Rejected NSSAI, even when this TA is part of the RA.

* * * * End of Changes * * * *