3GPP SA WG2 Meeting #150E	S2-2202768

Elbonia, April 06 - 12, 2022

Source:	Qualcomm Incorporated

Title:	TR 23.700-05: KI#4, new solution on support of roaming of mobile base station relays

Document for:	Discussion/Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR

Abstract of the contribution: This contribution proposes a solution to enable roaming of mobile base station relays.

# 1.	Proposal

It is proposed to agree the following changes to TR 23.700-05

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[x]	3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[y]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[z]	3GPP TS 38.401: "NG-RAN; Architecture description".

NEXT CHANGE

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  |  | x |  |  |

NEXT CHANGE

## 6.X	Solution #X: Enabling roaming of mobile base station relays

### 6.X.1	General

This solution addresses Key Issue#4 as defined in clause 5.4.

### 6.X.2	Functional description

The high-level points of the proposed solution are:

1)	The mobile base station relay may be pre-configured with one or more of the following:

a)	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a relay;

NOTE 1: The geographic area format used for V2X policy in TS 24.588 [y] can be re-used. A geographic area is encoded as a list of coordinates, each corresponding to a summit of a polygon.

b)	a list of PDU session parameters, e.g. specific DNN and S-NSSAI, and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for relay configuration;

2)	When the mobile base station relay moves out of coverage of the HPLMN:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a relay as described in 1-a), the relay attempts to select the PLMNs in the list, in priority order;

-	otherwise, the mobile base station relay performs PLMN selection as specified in TS 23.122 [y];

3)	Upon selecting a VPLMN as described in 2), the mobile base station relay initiates a registration procedure and indicates in the Registration Request message that it intends to operate:

a)	as a relay on behalf of the VPLMN (i.e. operating the radio resource of the VPLMN and connecting to a CU of the VPLMN); or

b)	as a relay on behalf of the HPLMN (i.e. operating the radio resources of the HPLMN and connecting to a CU of the HPLMN);

NOTE 2: The mobile base station relay can operate as a relay on behalf of the HPLMN within coverage of the VPLMN, provided such operation is authorized by the VPLMN, for instance to provide coverage extension for the HPLMN in specific areas.

4) The AMF in the VPLMN either:

a)	accepts the Registration Request of the mobile base station relay; or

b)	rejects the Registration Request of the mobile base station relay with a 5GMM cause value indicating that the mobile base station relay is not allowed to operate as a relay in the VPLMN;

5) If the Registration Request of the mobile base station relay is accepted and:

a)	the mobile base station relay intends to operate as a relay on behalf of the VPLMN, the mobile base station relay either:

i)	performs an IAB node integration procedure with the VPLMN as specified in TS 38.401 [z] clause 8.12.1. If the mobile base station relay is pre-configured with the PDU session parameters for the OAM PDU session for the VPLMN or the geographic area as described in 1-b), the UE uses these parameters to establish the OAM PDU session during the procedure; or

NOTE 3: When performing i), the PDU sessions of any UE camped on the mobile base station relay will be interrupted.

ii)	performs an IAB inter-donor full migration procedure, where the source donor CU is in the HPLMN and the target donor CU is in the VPLMN.

NOTE 4: The IAB inter-donor full migration procedure is to be specified by RAN.

NOTE 5: When performing ii), the PDU sessions if any UE camped on the mobile base station relay will be transferred from the HPLM to the VPLMN. This assumes that inter-PLMN handover between the HPLMN and the VPLMN is supported.

b)	the mobile base station relay intends to operate as a relay on behalf of the HPLMN, the IAB-node MT at the mobile base station relay establishes a PDU session with the VPLMN and uses the resulting IP connectivity to establish an F1 interface toward a CU in the HPLMN.

### 6.X.3	Signalling flows

#### 6.X.3.1	Signalling flow for roaming of mobile base station relay operating on behalf of VPLMN using IAB node integration procedure

Figure 6.X.3.1-1: Signalling flow for roaming of mobile base station relay operating on behalf of VPLMN using IAB node integration procedure

0.	The mobile base station relay may be pre-configured with:

-	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a relay

-	a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for relay configuration

1.	The mobile base station relay moves out of coverage of its HPLMN and selects a VPLMN as follows:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a relay, the relay attempts to select the PLMNs in the list, in priority order

-	otherwise, the mobile base station relay performs PLMN selection as specified in TS 23.122 [y]

2. The mobile base station relay sends a Registration Request message with an indication that the mobile base station relay intends to operate as a relay on behalf of the VPLMN.

3. Optionally, the VPLMN AMF may check whether the mobile base station relay is authorized to operate as a relay on behalf of the VPLMN.

4. If the AMF determines that the mobile base station relay is authorized to operate as a relay on behalf of the VPLMN, the AMF sends a Registration Accept message to the mobile base station relay.

5. The mobile base station relay perform an IAB node integration procedure with the VPLMN as specified in TS 38.401 [z] clause 8.12.1. If the mobile base station relay is pre-configured with the PDU session parameters for the OAM PDU session for the VPLMN or the geographic area (see Step 0), the UE uses these parameters to establish the OAM PDU session during the procedure.

#### 6.X.3.2	Signalling flow for roaming of mobile base station relay operating on behalf of VPLMN using IAB inter-donor full migration procedure

Figure

6.X.3.2-1: Signalling flow for roaming of mobile base station relay operating on behalf of VPLMN using IAB inter-donor full migration procedure

0.	The mobile base station relay may be pre-configured with:

-	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a relay

-	a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for relay configuration

1.	The mobile base station relay moves out of coverage of its HPLMN and selects a VPLMN as follows:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a relay, the relay attempts to select the PLMNs in the list, in priority order

-	otherwise, the mobile base station relay performs PLMN selection as specified in TS 23.122 [y]

2. The mobile base station relay sends a Registration Request message with an indication that the mobile base station relay intends to operate as a relay on behalf of the VPLMN.

3. Optionally, the VPLMN AMF may check whether the mobile base station relay is authorized to operate as a relay on behalf of the VPLMN.

4. If the AMF determines that the mobile base station relay is authorized to operate as a relay on behalf of the VPLMN, the AMF sends a Registration Accept message to the mobile base station relay.

5. The mobile base station relay performs an IAB inter-donor full migration procedure, where the source donor CU is in the HPLMN and the target donor CU is in the VPLMN.

#### 6.X.3.3	Signalling flow for roaming of mobile base station relay operating on behalf of HPLMN

Figure 6.X.3.3-1: Signalling flow for roaming of mobile base station relay operating on behalf of HPLMN

0.	The mobile base station relay may be pre-configured with:

-	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a relay

-	a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for relay configuration

1.	The mobile base station relay moves out of coverage of its HPLMN and selects a VPLMN as follows:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a relay, the relay attempts to select the PLMNs in the list, in priority order

-	otherwise, the mobile base station relay performs PLMN selection as specified in TS 23.122 [y]

2. The mobile base station relay sends a Registration Request message with an indication that the mobile base station relay intends to operate as a relay on behalf of the HPLMN.

3. Optionally, the VPLMN AMF may check whether the mobile base station relay is authorized to operate as a relay on behalf of the HPLMN.

4. If the AMF determines that the mobile base station relay is authorized to operate as a relay on behalf of the HPLMN, the AMF sends a Registration Accept message to the mobile base station relay.

5. The mobile base station relay establishes a PDU session with the VPLMN and uses the resulting IP connectivity to establish an F1 interface toward a CU in the HPLMN.

### 6.X.4	Impacts on services, entities and interfaces

The following impacts have been identified:

- UE:

-	No impact

- Mobile base station relay:

-	Support pre-configuration with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a relay

-	Support pre-configuration with a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for relay configuration

-	Support for PLMN selection based on the prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a relay

-	Support indicating in the Registration Request the intent to operate as a relay on behalf of the VPLMN/HPLMN

-	Support IAB inter-donor full migration procedure

-	Support for establishing an F1 interface to a CU in the HPLMN using IP connectivity provided by a PDU session established in the VPLMN

- AMF:

-	Support for determining whether a mobile base station relay attempting to register is allowed to operate as a relay and accepting/rejecting the Registration Request accordingly

END OF CHANGES