SA WG2 Meeting #150e	S2-2202822

# April 6th – 12th, 2022; Elbonia               		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	23.700-41: Solution for KI#5; support of a Network Slice with an Area of Service not matching existing TA boundaries.

Document for: 	Approval

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_ph3/ Rel-18

Abstract of the contribution:

# 1 Discussion

This pCR proposes a solution to KI#5 in TR 23.700-41, regarding the support of a Network Slice with a AoS not matching existing TA boundaries.

# 2 Proposal

It is proposed to update 23.700-41 as follows

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| Solution X: support of a Network Slice with an AoS not matching existing TA boundaries. |  |  |  |  | x |  |

NEXT CHANGE (2) ALL text is new

## 6.X	Solution #X: support of a Network Slice with an AoS not matching existing TA boundaries.

## 6.X.1	Description

As the use cases for network slicing become more and more advanced and permit addressing private networking and industrial IoT applications, the Area of Service (AoS) of network slice can become limited to small areas of a PLMN which may not map to already existing TA boundaries. In order to accommodate this, with the current set of specification the way to achieve this would be to redesign the TAs topology of the network.

Indeed, if the goal of an operator is to support legacy UEs while using such slices, the only possible solution is to redesign the TA boundaries and potentially add new and smaller TAs.

So, it can be concluded that to support legacy UEs, the only solution possible to address the problem space is to redesign the TAs to enable matching the needs to deploy limited AoS network slices not matching already deployed TAs topology.

While this can be a solution, this implies that even users that do not have interest in the specific limited AoS slices may end up having to be allocated to such smaller tracking area and/or the TAI list can become rather complicated to form or even in some cases exceed the limit of 16 TAs in the TAI-list. It may in some situations cause also additional signalling traffic due to mobility management and needs of UE configuration update even if the UEs are not interested at all by the use of these specific slices. So, for rel-18 and beyond, improvements can be considered.

If there are network slices that the operator knows the customer has full control on the UE population of, and the specification of the supported UEs capabilities is part of the SLA, we can explore alternative option that is presented here.

The solution is based on allowing the broadcast on SIB of additional TACs (we expect only a small number if not just one additional to be only needed to cover most cases, RAN2 to define how many can be configured) that supporting UEs can read. These additional/secondary TAC values are passed to the CN with the associated slices supported in NG-AP messages uses to maintain the supported slices and explicitly indicated as "Secondary TACs" in UE specific signalling when the TAC is included today to signal the UE location in the Initial UE message,  The AMF then forms the RA by considering the support of slices as today, but it can only include in the Allowed NSSAI network slices requiring TAs that are broadcasted as secondary TAs in the RA to the supporting UEs. Note well that the support of slices that are not fully supported in a TA is not signalled, as per today's specifications (so, e.g.,  TA4 only uniformly supports S-NSSAI 1 and S-NSSAI 4). See figure 6.x.1-1.

Figure 6.x.1-1-Example TA topology including Primary and Secondary TAs

Based on the uniform support of network slices per TA in figure 6.x.1-1, if, the UE requests S-NSSAI 3 and S-NSSAI 1 only, then the RA includes TAI 3 but not TAI1. It is not permitted to add TA4 as cells broadcasting TA4 do not always support S-NSSAI 3 (see above in figure 6.x.1-1 that the cells broadcasting TA4 under primary TA5 do not support S-NSSAI 3).

In other words, the uniform support of slices is assumed also at the secondary TAI level.

Since a gNB only provides the supported slices under the TAC it administers, S-NSSAI(s) uniform support by a secondary TA shall be configured in each gNB even though for the cells under its scope support may be considered uniform (e.g. in figure 6.x.1-1 the TA5 and TA1 may be under different gNBs and so TA4 may be considered uniformly supporting S-NSSAI 3 from the gNB supporting TAI1 and not supporting S-NSSAI 3 in gNB supporting TAI5). The alternative is that the AMF or NSSF determine the uniform support based on the received information from the RAN by combining the support indication received by different NG-AP sources (i.e. if gNB 1 declares S-NSSAI3 is supported by TAI4, and gNB5 does not indicate support  of S-NSSAI3 by TAI4, then TAI4 does not support S-NSSAI 3 uniformly and the NSSF/AMF can summarize this when they combine the indication by different gNBs).

### 6.X.2	Procedures

When the UE performs a MRU, it includes support secondary TAs in UE 5GMM capabilities. The Initial UE message or the UL NAS transport message where the Registration Request is carried, convey the User Location Information IE with the primary TAC and the secondary TACs of the Cell Id to determine the S-NSSAIs that are supported in the cell where the UE is.

Figure 6.x.2-1- enhanced ULI in INITIAL UE/ UL NAS TRANSPORT MESSAGEs

#### we propose to modify the "User Location Information IE" in clause  9.3.1.16 of 38.413 from

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |

to

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI LIST | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI LIST | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |

where the definition of TAI LIST can be like

TAI LIST

This IE indicates the list of TAIs broadcast by the NG RAN node and can be accessed by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TAI Item |  | 1..<maxnoofsupportedTACs> |  |  | – |  |
| >TAC | M |  | 9.2.2.5 | Broadcast TAC | – |  |
| >Broadcast PLMNs |  | 1..<maxnoofsupportedPLMNs> |  |  | – |  |
| >>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>TAI Type | M |  | ENUMERATED (primary, secondary, …) | slice categorization | - |  |

The TAI Type information shall also be conveyed alongside the TAI in the NG-AP NG SETUP REQUEST, RAN CONFIGURATION UPDATE messages. similarly, the same applies on Xn-AP Xn SETUP REQUEST and NG-RAN CONFIGURATION UDPATE message and in the F1-AP  F1 SETUP REQUEST and GNB-DU CONFIGURATION UDATE messages.

In the registration Request message itself, the gNB also includes

In the UE Registration Request step 1 of  Figure 4.2.2.2.2-1: "Registration procedure" the registration request includes the "last visited TAI" information.

[...] UE to (R)AN: AN message (AN parameters, Registration Request (Registration type, SUCI or 5G-GUTI or PEI, [last visited TAI (if available)], [...]

it is proposed that now it is augmented with the "Last visited secondary TAIs" by supporting UEs.

[...] UE to (R)AN: AN message (AN parameters, Registration Request (Registration type, SUCI or 5G-GUTI or PEI, [last visited TAI (if available)],[last visited secondary TAIs(if available and supported by the UE]] [...]

### 6.X.3	Impacts on services, entities and interfaces

The solution has the following impacts:

NG-RAN node:

-	Broadcast one or more additional TAIs via a new information element in SIB

-	Support new messages in the NG-AP and Xn-AP as well as F1AP procedures to share support of additional secondary TAI, or indicating the cell where the UE is support >1 TAC and what the TAC and which ones the primary/secondary values are.

AMF

-	Obtain information on the slice support for primary and secondary TAIs via NGAP procedures

-	Assign the registration area considering the primary and secondary TACs and whether the UE is a legacy UE or new UE.

-	Page UEs considering the primary and secondary TAC if informed via NGAP procedures as well as the corresponding registration areas of the UEs

-	Obtain UE’s primary TAC and secondary TACs via initial UE message

UE

-	When multiple TACs are available for the selected PLMN, the UE selects (all things being equal) a primary or secondary TAC inside the RA

-	perform registration procedure by taking into account the secondary TAC broadcasted.

AMF: noneEND OF CHANGES