SA WG2 Meeting #150e	S2-2202978

6 – 12 April 2022, e-Meeting	(revision of S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	Key Issue: Support of 5GS Aspects of Discontinuous Coverage

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract of the contribution: Identifies a new KI for support of 5GS Aspects of Discontinuous Coverage

# 1. Introduction

The SID agreed for Study on satellite access Phase 2 in SP-211615 includes the following objectives:

The study item aims at investigating on further 5GC/EPC enhancements to support satellite access based on the R17 5GSAT_ARCH achievements with the following 5GC/EPC areas for study: - WT#1: Discontinuous coverage WT#1.1: Architectural enhancements to support discontinuous coverage for mobility enhancement (e.g. paging enhancement) WT#1.2: Architectural enhancements considering prediction, awareness & notification of UE wake-up time, power saving optimizations.

While there is a solution for Discontinuous coverage for EPS in Release 17, there is no solution for Discontinuous coverage for 5GS in Release 17. However, it is possible that the solution for EPS in Release 17 may be unsuitable for 5GS owing to different performance objectives. For example, the solution for EPS in Release 17 attempts to reduce or minimize UE power consumption by allowing the UE to deactivate the AS layer during a coverage gap. A consequence of this is that a UE might miss obtaining coverage using a different satellite RAT, a TN RAT or after having moved into an area where there is satellite (or TN) coverage because the UE does not look for such coverage due to prioritizing the avoidance of cell searching while in a coverage gap.

However for a UE supporting satellite access over NR, minimizing power consumption may be less important, or no more important, than minimizing a period of no coverage. For such a UE, it may then be better to, for example, periodically scan for an available satellite cell or TN cell or be provided with information on the availability of such alternative cell coverage. None of this is addressed by the solution for EPS in Release 17. A KI thus seems suitable to look at the alternative performance requirements for a UE with 5GS access and to identify solutions which may support such objectives better than the type of solution for EPS in Release 17.

# 2. Text Proposal

The following text is proposed to be applied to TR 23.700-28.

*** Start of the change (all new text) ***

## 5.x	Key Issue #X: Support of 5GS Aspects of Discontinuous Coverage

### 5.x.1	General description

There is a solution for discontinuous coverage for satellite access with EPS in Release 17, but there is no solution for discontinuous coverage for satellite access with 5GS in Release 17. However, it is possible that the solution for EPS in Release 17 may be unsuitable for 5GS owing to different performance objectives. For example, the solution for EPS in Release 17 attempts to reduce or minimize UE power consumption by allowing the UE to deactivate the AS layer during a coverage gap. A consequence of this is that a UE might miss obtaining coverage using a different satellite RAT, a TN RAT or after having moved into an area where there is satellite (or TN) coverage because the UE does not look for such coverage due to prioritizing the avoidance of cell searching while in a coverage gap.

However for a UE supporting satellite access over NR, minimizing power consumption may be less important, or no more important, than minimizing a period of no coverage. For such a UE, it may then be better to use a solution that better supports minimizing a period of no coverage as well as reducing or minimizing power consumption.

The objectives of this KI are to:

- 	For a UE with NR satellite access, evaluate the limitations of the solution in Release 17 (when applied to 5GS) for minimizing a period of no coverage as well as reducing or minimizing power consumption

-	Identify solutions to overcome any limitations found