SA WG2 Meeting #150e	S2-2203562

# April 6th – 12th, 2022; Elbonia

Source: 	Oracle, Toyota, OPPO

Title: 	TR 23.700-80: KI#5 - BDT Extensions to support non-real-time Application AI/ML traffic transport

Document for: 	Approval

Agenda Item: 	9.21

Work Item / Release:	AIMLsys / Rel-18

Abstract of the contribution: Proposed Solution for KI#5 to support the transport of the non-real-time Application AI/ML traffic.

# 1	Discussion

Network capacity planning is a major challenge for any mobile operator.  In order to enable the MNOs to assist ASPs to transport their Application AI/ML traffic in a more controlled time-table to ensure the availability of network resources, this solution proposes to extend the existing Background Data Transfer (BDT) mechanism to support planned and event driven application AI/ML traffic.

As described in TS 23.503, clause 6.1.2.4, today BDT supports two different ways of applying data transfer policies. One is for a future PDU session and another one is for an existing PDU session.  The initial intent of BDT is to support non-time-critical data during off-hour.

However, given the ability of BDT of allowing to negotiate in advance the desired data transfer time-table and location, data volume and data transfer policy (e.g. aggregated bitrate for one UE or a group of UEs, charging keys, etc.), including the ability to update it based on changing network conditions (e.g. triggered by NWDAF notifications), it is reasonable to extend the existing BDT mechanisms to support the non-real-time Application AI/ML data transfer that meets the operational behaviours as described in TS 22.261, clause 6.40.2:

- the in-time data transfer,

- the event driven (e.g. responding to the threshold reporting of “Network Performance” from NWDAF for the area of interest and time window, UE’s location etc.) data transfer, and

- the dynamic policy adaptation data transfer

According to TS 22.261, clause 7.10, there are KPIs defined for the QoS performance for the data transfer to support split AI/ML inference, AI/ML model download and FL learning.  In order to leverage the existing BDT mechanisms as described above to support the non-real-time Application AI/ML traffic, the BDT Transfer Policy needs to be extended to support additional QoS parameters beyond the traffic bit rate and volume that has been defined.   The following additional QoS parameters are proposed by this solution to be added to the BDT Transfer Policy in order to support Application AI/ML traffic:

- Packet Delay Budget for UL/DL

- Packet Error Rate for UL/DL

Further details of the proposal are described in the following Solutions clause.

# 2	Proposal

The proposed solution is to address KI#5: 5GC Enhancement to enable Application AI/ML Traffic Transport.

Editor’s Note: It is TBD if this solution also applies to KI#6 since it proposes to add QoS support to BDT.

Start of Changes

# 6	Solutions

## 6.X	Solution #X: Background Data Transfer Extensions to support non-real-time Application AI/ML traffic transport

### 6.X.1	Description

This KI is to address KI#5: 5GC Enhancement to enable Application AI/ML Traffic Transport.

Network capacity planning is a major challenge for any mobile operator.  In order to enable the MNOs to assist ASPs to transport their Application AI/ML traffic in a more controlled time-table to ensure the availability of network resources, this solution proposes to extend the existing Background Data Transfer (BDT) mechanism to support planned and event driven application AI/ML traffic.

As described in TS 23.503, clause 6.1.2.4, today BDT supports two different ways of applying data transfer policies, One is for a future PDU session and another one is for an existing PDU session.  Even though the initial intent of BDT is to support non-time-critical data during off-hour, given the ability of BDT of allowing negotiation in advance of the desired data transfer time-table and location, data volume and data transfer policy (e.g. bit rate for one UE or a group of UEs, charging keys, etc.), including the ability to update it based on changing network conditions (e.g. triggered by NWDAF notifications) it is reasonable to extend the following existing BDT mechanisms to support the non-real-time Application AI/ML data transfer that meets the operational behaviours as described in TS 22.261, clause 6.40.2:

- the in-time data transfer,

- the event driven (e.g. responding to the threshold reporting of “Network Performance” from NWDAF for the area of interest and time window, UE’s location etc.) data transfer, and

- the dynamic policy adaptation data transfer

According to TS 22.261, clause 7.10, there are KPIs defined for the QoS performance for the data transfer to support split AI/ML inference, AI/ML model download and FL learning.  In order to leverage the existing BDT mechanisms as described above to support the non-real-time Application AI/ML traffic, the BDT Transfer Policy needs to be extended to support additional QoS parameters beyond the traffic bit rate and volume that have been defined.   The following additional QoS parameters are proposed by this solution to be added to the BDT Transfer Policy to support Application AI/ML traffic:

- Packet Delay Budget for UL/DL per UE

- Packet Error Rate for UL/DL per UE

- Editor’s Note: It is TBD if and how Maximum Data Burst Volume for UL/DL per UE can be derived as well from the latency and reliability requirements for UL/DL per UE provided by the AF, and be included in the transfer policy.

Note that, the QoS parameters for uplink and downlink may be asymmetrical based on the KPIs that were defined in TS 22.261, clause 7.10.

Further details of the proposal are described in the following Solutions clause.

### 6.X.2	Solutions

The intent of this solution is to extend the existing BDT data transfer mechanism to support non-real-time Application AI/ML data transfer.

The solution describes the update to the BDT transfer policy in clause 6.1.2.4 of TS 23.503. This update includes added QoS parameters support.  It also updates the existing BDT negotiation procedures for future PDU session as defined in clause 4.16.7.2 of TS 23.502 to include the additional QoS parameters support, and in addition, it specifies how AIML AF discovers its serving NEF as well as the need for the NEF to authenticate/authorize the AIML AF. Finally, this solution presents the procedures on how BDT can be applied to the existing or future PDU session.

#### 6.X.2.1	Additional QoS Parameters for BDT Transfer Policy to support Application AI/ML data transfer

The current BDT data transfer policy does not support the QoS parameters other than the maximum aggregated bitrate.  In order to meet the KPIs that have been defined in TS 22.261, clause 7.10 to support the QoS performance for the data transfer for various Application AI/ML traffic, this solution proposed to enhance the BDT transfer policy for the support of the following QoS parameters that have been indicated in the KPIs:

- Packet Delay Budget for UL/DL per UE

- Packet Error Rate for UL/DL per UE

This solution proposes to update the BDT policy descriptions in clause 6.1.2.4 of TS 23.503 to include the support of the above QoS parameters.  The example for the update to clause 6.1.2.4 of TS 23.503 is shown in “bold italic” below.

#### 6.1.2.4 Negotiation for future background data transfer The AF may contact the PCF via the NEF (and Npcf_BDTPolicyControl_Create service operation) to request a time window and related conditions for future background data transfer (BDT). NOTE 1: The NEF may contact any PCF in the operator network. The AF request shall contain an ASP identifier, the volume of data to be transferred per UE, the expected amount of UEs, the desired time window, the External Group Identifier and optionally, Network Area Information, MAC address or IP 3-tuple to identify the Application server, request for notification. The AF provides as Network Area Information either a geographical area (e.g. a civic address or shapes), or an area of interest that includes a list of TAs or list of NG-RAN nodes and/or a list of cell identifiers. When the AF provides a geographical area, then the NEF maps it based on local configuration into of a short list of TAs and/or NG-RAN nodes and/or cells identifiers that is provided to the PCF. In addition the AF may provide latency and reliability requirements for UL/DL per UE. NOTE 2: The latency and reliability requirements provided by the AF should be explicit enough such that the PCF will be able to map them to their corresponding Packet Delay Budget (PDB) for UL/DL per UE, Packet Error Rate(PER) for UL/DL per UE, for UL/DL per UE. Editor’s Note: It is TBD how the PCF is able to derive from the AF requirements a BDT policy that includes the above QoS requirements for the negotiated time window. The NEF may map the ASP identifier based on local configuration to a DNN and S-NSSAI that is in addition provided to the PCF. The MAC address or IP 3-tuple to identify the Application server may be provided by the AF or may be locally configured at the PCF and it is used for the generation of a URSP rule for the application as well as a PCC rule for the application traffic. The request for notification is an indication that the ASP accepts that the BDT policy can be re-negotiated using the BDT warning notification procedure described in clause 4.16.7.3 of TS 23.502 [3]. NOTE 3: A 3rd party application server is typically not able to provide any specific network area information and if so, the AF request is for the whole operator network. The PCF shall first retrieve all existing BDT policies stored for any ASP from the UDR. The PCF may retrieve analytics on "Network Performance" from NWDAF following the procedure and services described in TS 23.288 [24]. Afterwards, the PCF shall determine, based on the information provided by the AF, the analytics on "Network Performance" if available and other available information (e.g. network policy and existing BDT policies) one or more BDT policies. The PCF may be configured to map the ASP identifier to a target DNN and S-NSSAI if the NEF did not provide the DNN, S-NSSAI to the PCF. A BDT policy consists of a recommended time window for the background data transfer, a reference to a charging rate for this time window, optionally a maximum aggregated bitrate (indicating that the charging according to the referenced charging rate is only applicable for the aggregated traffic of all involved UEs that stays below this value) and optionally QoS parameters of Packet Delay Budget (PDB) for UL/DL per UE, Packet Error Rate(PER) for UL/DL per UE etc. to be equally used by any UE in the BDT group. Finally, the PCF shall provide the candidate list of BDT policies to the AF via NEF together with the Background Data Transfer Reference ID. If the AF received more than one BDT policy, the AF shall select one of them and inform the PCF about the selected BDT policy. NOTE 3: The maximum aggregated bitrate (optionally provided in a BDT policy) is not enforced in the network. The operator may apply offline CDRs processing (e.g. combining the accounted volume of the involved UEs for the time window) to determine whether the maximum aggregated bitrate for the set of UEs was exceeded by the ASP and charge the excess traffic differently. NOTE 4: It is assumed that the 3rd party application server is configured to understand the reference to a charging rate based on the agreement with the operator. : : :

#### 6.X.2.2	BDT Negotiation for future PDU session to support Application AI/ML data transfer

This section presents further clarifications on how to leverage the existing BDT procedures for negotiation of future background data transfer as specified in TS 23.502, clause 4.16.7.2 to support the Application AI/ML data transfer.

Prior to the establishment of PDU session to support the BDT within a specified time-window, AF initiates requests to its serving NEF to (re)negotiate BDT policy (as described in clause 6.1.2.4 of TS 23.503) that might have been pre-provisioned or earlier negotiated and stored in the UDR.

Figure 6.X.2.2-1: BDT Policy Negotiation for future PDU session to support Application AI/ML data transfer

The procedures as shown in Figure 6.X.2.1-1 above is based on the procedures specified in TS 23.502, clause 4.16.7.2 to support negotiation for future background data transfer.  The following descriptions focus on the changes to the existing procedures:

Prior to the transport of the Application AI/ML data within the specified time-window, the AF negotiates with the 5G Core for the BDT policies that apply to its given Application AI/ML data transfer corresponding to the authorized 3rd party (i.e. AF), the AF is required to discover its serving NEF, if it has not done so, using the mechanism as described in TS 23.501, clause 6.3.14.

In Step 1a, in addition to the list of parameters for Nnef_BDTNegotiation_Create Request from AF as described in clause 4.16.7.2 of TS 23.502, the AI/ML AF should include the added latency and reliability requirements as described in 6.X.2.1.

In Step 1b, the NEF may authenticate the AF and authorize the BDT request from the AF.  If the given AF is not authenticated and authorized, the NEF will reject AF’s request through the Nnef_BDTNegotiation_Create Response, and the following steps are skipped.

In Step 2, in addition to the list of parameters for Npcf_BDTPolicyControl_Create Request from AF as described in clause 4.16.7.2 of TS 23.502, the NEF should include the added latency and reliability requirements exactly as received from the AF.

In Step 4, PDB and PER etc. QoS parameters may also be provided by UDR as part of the BDT policies to the PCF. This may happen if the PCF (or another PCF) had received from the AF/NEF in a previous BDT negotiation, latency and reliability requirements, mapped it to the corresponding PDB and PER etc. QoS parameters and their corresponding 5QI, and stored it in the UDR.

In Steps 5, 6 the PCF maps the additional latency and reliability requirements to PDB and PER etc. QoS parameters and to a proper 5QI. If the 5QI is dynamically assigned by the PCF (ie it is neither standardized nor pre-configured in the AN) the additional QoS parameters will be explicitly provided alongside their corresponding 5QI (you may refer to 23.501 5.7.2.1 to see a generic description of the 5QI mechanism).

NOTE 1: In the case when any of Packet Delay Budget per UE, Packet Error Rate per UE and Maximum Data Burst Volume for per UE are asymmetric for UL and DL direction, at least two PCC rules will have to be provided by the PCF. Each PCC rule will be for each direction (ie UL or DL). Each PCC rule will have its own 5QI and the corresponding TFTs.

After the BDT policy negotiation and before the PDU session is established to transport the Application AI/ML data, if the AF decides to select an alternative BDT policy (e.g. data rate reduction, relaxing delay constraints of non-real-time traffic etc.) as the result of certain AI/ML operation decision, the AF may trigger Step 8 to update the corresponding PCF via the support of NEF for the new selected BDT policy.

NOTE: It is not in the scope of this solution to specify the event or consideration that triggers the AF decision to update the BDT policy.

In Step 12, the additional QoS parameters may be added to the UDR by the PCF as part of the updated BDT policies.

#### 6.X.2.3	BDT Activation of Negotiated Future PDU Session or Existing PDU Session to support Application AI/ML data transfer

This clause describes two ways to activate/apply the background data transfer – i.e. (a) activation of the negotiated future PDU session(s) based on the previously negotiated BDT policy as described in clause 6.X.2.2 above; and (b) activation of the existing PDU session by applying the BDT policy just before the start of the BDT transfer window.

For the activation of the negotiated future PDU sessions(s) based on the previously negotiated BDT policy to support the Application AI/ML data transfer, before the start of the BDT transfer window, the AF triggers the UDR via NEF by actuating Nnef_ApplyPolicy_Create service to notify the PCF as described in clause 4.15.6.8 of TS 23.502.  PCF will then leverage the negotiated BDT policy to generate the PCC rules to support the establishment of the future PDU session(s).  For future details on how to apply the previously negotiated BDT policy can be referred to clause 6.1.2.4  of TS 23.503 and clause 4.15.6.8 of TS 23.502.

If the PDU session has been established and the AF would like to apply the BDT policy to an existing PDU session for individual UE to support the Application AI/ML data transfer, then the AF invokes the Npcf_PolicyAuthorization_Create service directly to PCF or via NEF (via the N5 interface).  The AF provides the Background Data Transfer Reference ID together with the AF session information to the PCF that serves the PDU session.  The PCF authorizes the AF request.  If the PCF determines that the AF can’t be authorized, it rejects the AF request.  Once the PCF authorizes the AF request, the PCF retrieves the corresponding BDT policy from UDR to derives the PCC rule for the BDT according to the transfer policy.  The PCF updates the SMF with corresponding new PCC rule(s) with PCF initiated SM Policy Association Modification procedures as described in TS 23.502, clause 4.16.5.2.  Further details on how AF applies the Background Data Transfer Policy to an existing session are provided in clause 6.1.2.4 of TS 23.503.

Figure 6.X.2.3-1: Applying BDT policy to an existing PDU session to support Application AI/ML data transfer

- AF decides to leverage an existing PDU session to support its AI/ML data transfer, then the AF will, at the time the BDT is about to start, provide, for each UE, the Background Data Transfer Reference ID together with the AF session information to the PCF that serves the PDU session (via the N5 interface).  This Npcf_PolicyAuthorization_Create service as described in clause 5.2.5.3.2 of TS 23.502 is to authorise an AF request and to create policies as requested by the authorized AF for the PDU Session to which the AF session is bound. More specifically, the added latency and reliability requirements as described in 6.X.2.1 should be included in the AI/ML AF request.

NOTE: It is not the scope of this solution to specify the event(s) or consideration(s) that triggers the AF decision to apply the BDT policy to an existing PDU session.

- The PCF authorizes the AF request.

- If the PCF determines that the AF can’t be authorized, it rejects the AF request by including the rejection in Npcf_PolicyAuthorization_Create response.  All the subsequent steps below are skipped.

4.&5. Once the PCF authorizes the AF request, the PCF retrieves the corresponding BDT policy from UDR to derives the PCC rule(s) for the BDT according to the transfer policy that has been pre-provisioned.

6.	The PCF updates the SMF with corresponding new PCC rule(s) with PCF initiated SM Policy Association Modification procedures as described in TS 23.502, clause 4.16.5.2.

Further details on how PCC rules are applied to either future or an existing PDU session are provided in clause 6.1.2.4 of TS 23.503.

### 6.X.3	Impacts on services, entities and interfaces

AIML AF shall be able to provide latency and reliability requirements in BDT negotiation request

PCF shall be able to consider such information for the generation of PCC rules. Specifically, the PCF shall be able to map the above information to QoS parameters, which include Packet Delay Budget (PDB) for UL/DL per UE or/and Packet Error Rate (PER) for UL/DL per UE (on top of the original BDT related PCC rules), and store/retrieve them in/from the UDR as part of the BDT transfer policy.

NEF shall be able to consider latency and reliability requirements received from the AF in BDT negotiation request as part of authorizing the AF’s permission to submit the BDT request.

End of Changes